| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0104 |
| --- | --- |

| Title: | Partition boundary filtering in DBBP |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Mikhail Mishurovskiy, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Depth-based block partitioning (DBBP) was adopted at the last meeting. In DBBP, an arbitrarily shaped block partitioning for the collocated texture block is derived from a binary segmentation mask computed by the corresponding depth block. Two partitions are motion-compensated and then merged by averaging them based on the depth-based segmentation mask. In this contribution, to improve the performance of the current DBBP of about -0.2%, the filtering process for smoothing the regions around the arbitrary partition efficiently, instead of averaging, is proposed. The result shows that the proposed method obtains the gain of about -0.2% in dependent views and about -0.1% in overall performance. In addition, the gain is quite consistent in all the sequences.

# Proposed Method

DBBP partitions a block into object and background regions. The arbitrarily shaped block partitioning for the collocated texture block is derived from a binary segmentation mask computed by the corresponding depth block.

First, object and background is partitioned by the segmentation mask. Next, object- and background-focus motion compensations are performed for one block. Each of the motion-compensated blocks consists of the object and background regions. Finally, these two blocks are merged by simply extracting the object and background from the block, based on the mask. In the case of the object or the background pixel near the partition, an averaging value of the pixel and the collocated one in the other block is used, as shown in Fig. 1. However, the collocated pixel is not classified to either object or background. Hence, it might result in the distortion near the boundary.

Fig. 1. Averaging around the boundary pixels

In order to figure out this problem, firstly, the proposed method merges two motion-compensated blocks by simply extracting the object and background from the block, based on the mask. Then, simple filtering is performed on the boundary pixels p and q, as displayed in Fig. 2.

Fig. 2. Filtering around the boundary pixels

Filtering pixels p’ and q’ can be generated as follows:

p’ = ((p-1) + (p)<<1 + (q))>>2,  q’= ((p) + (q)<<1 + (q-1))>>2

# Proposed Text

- H.8.5.3.3.9 Decoding process for depth based block partition wise inter sample prediction

- …

The array PredSamplesDbbpL is modified as follows:

for ( y = 0; y < nCSL; y++ )
for( x = 0; x < nCSL; x++ ) {

if( segMask[ x ][ y ]  = =  curSegmentFlag  ) {
		PredSamplesDbbpL[ x ][ y ] = predSamplesL[ x ][ y ]
	}
       PredSamplesTmpL [x][y] = PredSamplesDbbpL[ x ][ y ]

}

for ( y = 0; y < nCSL; y++ )
		for( x = 0; x < nCSL; x++ ) {
			tFlag = segMask[x][( y = = 0 ) ?y : (y−1)]
			lFlag = segMask[(x = = 0)?x : ( x − 1 )][ y ]
			bFlag = segMask[ x ][( y = = nCSL − 1 ) ? y : (y + 1) ]
			rFlag = segMask[( x = = nCSL −1 ) ? x : (x + 1) ][ y ]
			cFlag = segMask[ x ][ y ]
			filterHorFlag = !( ( lFlag  &&  rFlag  &&  cFlag ) | | ( !lFlag  &&  !rFlag  && !cFlag ) )
			if( filterHorFlag ) {
				tmpPixL =( PredSamplesTmpL [(x = = 0)?x : ( x − 1 )][y]  + PredSamplesTmpL [x][y]<<1 + PredSamplesTmpL [( x = = nCSL −1 ) ? x : x + 1][y]  )  >> 2
				}  else {
				tmpPixL = PredSamplesTmpL [ x ][ y ]
				}
			filterVerFlag = !( ( tFlag  &&  bFlag  &&  cFlag ) | | ( !tFlag  &&  !bFlag  && !cFlag ) )
			if( filterVerFlag ) {
				PredSamplesDbbpL [x][y] =( PredSamplesTmpL [x][ ( y = = 0 ) ?y : (y−1)] + tmpPixL <<1 + PredSamplesTmpL [x][ ( y = = nCSL − 1 ) ? y : y + 1]  )  >> 2
				}  else {
				PredSamplesDbbpL [x][y] = PredSamplesTmpL [ x ][ y ]
				}			}
		}

The arrays PredSamplesDbbpCb and PredSamplesDbbpCr are modified as follows:

for ( y = 0; y < nCSC; y++ )
for( x = 0; x < nCSC; x++ ) {

if( segMask[ x<<1 ][ y<<1 ]  = =  curSegmentFlag  ) {
		PredSamplesDbbpCb[ x ][ y ] = predSamplesCb[ x ][ y ]
		PredSamplesDbbpCr[ x ][ y ] = predSamplesCr[ x ][ y ]
	}
 PredSamplesTmpCb [x][y] = PredSamplesDbbpCb[ x ][ y ]

PredSamplesTmpCr [x][y] = PredSamplesDbbpCr[ x ][ y ]

}

for ( y = 0; y < nCSC; y++ )
		for( x = 0; x < nCSC; x++ ) {
			tFlag = segMask[x<<1][( y = = 0 ) ?y<<1 : (y−1)<<1]
			lFlag = segMask[(x = = 0)?x<<1 : ( x − 1 )<<1][ y<<1 ]
			bFlag = segMask[ x <<1][( y = = nCSL − 1 ) ? y<<1 : (y + 1)<<1 ]
			rFlag = segMask[( x = = nCSL −1 ) ? x <<1: (x + 1)<<1 ][ y<<1 ]
			cFlag = segMask[ x<<1 ][ y<<1 ]
			filterHorFlag = !( ( lFlag  &&  rFlag  &&  cFlag ) | | ( !lFlag  &&  !rFlag  && !cFlag ) )
			if( filterHorFlag ) {
				tmpPixCb =( PredSamplesTmpCb [(x = = 0)?x : ( x − 1 )][y]  + PredSamplesTmpCb [x][y]<<1 + PredSamplesTmpCb [( x = = nCSL −1 ) ? x : x + 1][y]  )  >> 2
				tmpPixCr =( PredSamplesTmpCr [(x = = 0)?x : ( x − 1 )][y]  + PredSamplesTmpCr [x][y]<<1 + PredSamplesTmpCr [( x = = nCSL −1 ) ? x : x + 1][y]  )  >> 2
				}  else {
				tmpPixCb = PredSamplesTmpCb [ x ][ y ]
				tmpPixCr = PredSamplesTmpCr [ x ][ y ]
				}
			filterVerFlag = !( ( tFlag  &&  bFlag  &&  cFlag ) | | ( !tFlag  &&  !bFlag  && !cFlag ) )
			if( filterVerFlag ) {
				PredSamplesDbbpCb [x][y] =( PredSamplesTmpCb [x][ ( y = = 0 ) ?y : (y−1)] + tmpPixCb <<1 + PredSamplesTmpCb [x][ ( y = = nCSL − 1 ) ? y : y + 1]  )  >> 2
				PredSamplesDbbpCr [x][y] =( PredSamplesTmpCr [x][ ( y = = 0 ) ?y : (y−1)] + tmpPixCr <<1 + PredSamplesTmpCr [x][ ( y = = nCSL − 1 ) ? y : y + 1]  )  >> 2
				}  else {
				PredSamplesDbbpCb [x][y] = PredSamplesTmpCb [ x ][ y ]
				PredSamplesDbbpCr [x][y] = PredSamplesTmpCr [ x ][ y ]
				}			}
		}

# Test Results

Test results of the proposed method are summarized in Table 1. As shown in Table 2, when the filtering is performed around the boundary pixels instead of the averaging, the coding gain of about -0.1% can be achieved. Especially in Undo_Dancer, the proposed method improves the coding performance by -0.5% in the dependent views

Table 1. Performance of the proposed method

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can improve the coding performance of DBBP. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).