| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0105 |
| --- | --- |

| Title: | Performance evaluation on DLT-based residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

DLT-based residual coding (F0159) was adopted for the efficient depth intra coding at Geneva meeting. It compresses a difference between indexes of original and predicted pixels for several intra modes. Since the reason why the DLT-based residual coding is applied for only several modes is not clear enough, this contribution tests the performance when it is extended into all the intra modes and removed, respectively. The results illustrate that both cases has no coding gain.

# DLT-Based Residual Coding

The DLT-based residual coding method encodes residual between indexes of original and predicted pixels for DC, vertical, horizontal, and DMM modes, as follows:

residual = DLT[original] – DLT[predicted]

For the other modes, the conventional residual coding is performed.

residual = original – predicted

Since the magnitude of the intra residual is relatively high, the DLT-based residual coding can reduce the residual size.

However, there is no clear reason why the DLT-based residual coding is applied for only several modes, so this contribution evaluates the performance when it is extended into all the intra modes and removed, respectively.

# Proposed Text

- (Removal of the depth-based residual coding)

- I.8.4.4.1 General decoding process for intra blocks

- …

- -

- -

- The picture reconstruction process prior to in-loop filtering for a color component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

…

# Test Results

Test results of the proposed method are summarized in Tables 1 to 4. Tables 1 and 2 illustrate the performance when the DLT-based residual coding is extended to all the coding modes, in CTC and all intra condition, respectively. Tables 3 and 4 shows the performance when the DLT-based residual coding is removed in CTC and all intra condition.

Table 1. Performance when the DLT-based residual coding is extended to all the coding modes (CTC)

Table 2. Performance when the DLT-based residual coding is extended to all the coding modes (All intra)

Table 3. Performance when the DLT-based residual coding is removed (CTC)

Table 4. Performance when the DLT-based residual coding is removed (All intra)

# Conclusion

When the DLT-based residual coding method is removed, it is observed no coding loss. Hence, it is recommended to remove it to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).