| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0106 |
| --- | --- |

| Title: | Separate enabling flag for SDC and DMM and Study on DMM4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Byeongdoo Choi, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Intra SDC and DMM is enabled and disabed by vps_depth_modes_flag. When SDC is enabled and DMM is disabled, one bit for a CU-level DMM flag is always transmitted. This contribution proposes to define a separate flag for DMM and unify enabling flags of inter and intra SDC, because intra SDC is performed in the exactly same manner as inter SDC, rather than DMM. In addition, it is discussed whether DMM4 can have inter-layer dependency or not.

# Separate Enabling Flag for SDC and DMM

Inter SDC, intra SDC, and DMM have been developed to improve the depth coding efficiency. Inter SDC and intra SDC are performed with exactly same concept. However, intra SDC is enabled and disabled by vps_depth_modes_flag, which also controls DMM. So, in the worst case, when intra SDC is enabled and DMM is disabled, one bit for a CU-level DMM flag is always transmitted. Hence, The proposed method defines a separate flag for DMM (depth_modeling_mode_flag) and unify enabling flags of inter and intra SDC (segment_dc_coding_flag) in VPS.

# Study on DMM4

Two DMMs have been added to the conventional intra modes for the efficient edge coding as follows:

- DMM1: Explicit wedgelet signalling;

- DMM4: Inter-component-predicted contour partitioning;

Since DMMs have been developed with similar concept, they are little bit redundant. In addition, when a slice type is intra, DMM4 tries to refer the corresponding texture block. However, it is not allowed that an intra slice has an inter-layer dependency. Usually, the intra slice is decoded without help of other slices or layers.

# Test Results

Test results of the proposed method are summarized in Tables 1 to 3. Tables 1 illustrates that the separate enabling flag affects no coding performance at all in CTC. Tables 2 and 3 illustrates the performance of DMM4 in CTC and all intra condition.

Table 1. Performance of separate enabling flag (CTC)

Table 2. Performance of DMM4 (CTC)

Table 3. Performance of DMM4 (All intra)

# Conclusion

The proposed method defined the two enabling flags. One is for inter and intra SDC, and the other is for DMM. Also, the coding performance of DMM4 was reported.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).