| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0107r2 |
| --- | --- |

| Title: | Constraints on the range of NBDV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang, Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 13588729866 felixzyc@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JCT3V-G0082. In this contribution, it is proposed to limit NBDV into a predefined range. The proposed method can save the bandwidth required or reduce the cache miss when fetching the reference views without any coding loss.

# Problem Statement

In current 3D-HEVC, some major coding tools, such as Inter-view Motion Prediction (IVMP), Advanced Residual Prediction (ARP) and View Synthesis Prediction (VSP), need a disparity vector which specifies a corresponding location in an inter-view reference picture, as shown in Fig.1.

Fig. 1. Disparity vector specifying a corresponding location in an inter-view reference picture

Neighboring Block Disparity Vector (NBDV) [1][2][3] is proposed to estimate such a disparity vector. Currently, NBDV is derived or each CU. First temporal DCP neighboring blocks are evaluated, followed by a check of the spatial DCP neighbors. Finally, MCP coded neighbor blocks are searched. Once a disparity motion vector is found, the whole disparity vector derivation process terminates. When no disparity motion vector is found from the neighboring blocks, a zero disparity vector is used.

The range of NBDV is as the same as MV in current design, which is -8191.75 to 8192. However, the range of all the possible accurate DVs among two views is limited and is often much smaller than the image size. Therefore, the current design may request more bandwidth or on-chip memory than it really needs for the coded depth map, which will only be accessed by NBDV.

# Proposed Method

To solve the above problem, we propose to limit the range of NBDV.

In 7th JCT3V meeting, JCT3V-G0082 proposed to calculate the range of NBDV according to the possible disparity range between the current texture view and the reference view using camera parameters. It was commented that JCT3V-G0082 didn’t improve the worst case thus further study is needed.

In this proposal, the allowable range of NBDV is set to a predefined constant to prevent improve the worst case.

In concrete terms, the horizontal component of NBDV shall not exceed the range of −1023.75 to 1024, inclusive, in units of luma samples. And the vertical component of NBDV shall not exceed the range of [-CtbSizeY+0.25, CtbSizeY], inclusive, in units of luma samples, where CtbSizeY is the size of LCU.

By such a limitation, the possible accessed area in reference view is much smaller. Table 2-1 shows the size of the possible accessed area before and after the limitation for different formats (the size of LCU is 64*64). Therefore, the cache miss when fetching the reference views can be reduced. In other words, the bandwidth or on-chip memory can be saved. For example, if 100% cache hit possibility is required, the on-chip memory for depth maps can be saved by 95.3% for 4Kx2K sequences.

Table 2-1 Size of the possible accessed area before and after the limitation for different formats

|  | Before Limitation |  |  | Before Limitation |  |  | Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format nickname | Width | Height | Size | Width | Height | Size |  |
| XGA | 1024 | 768 | 786432 | 1024 | 192 | 196608 | 25.0% |
| 720p HD | 1280 | 720 | 921600 | 1280 | 192 | 245760 | 26.7% |
| 4VGA | 1280 | 960 | 1228800 | 1280 | 192 | 245760 | 20.0% |
| SXGA | 1280 | 1024 | 1310720 | 1280 | 192 | 245760 | 18.8% |
| 525 16SIF | 1408 | 960 | 1351680 | 1408 | 192 | 270336 | 20.0% |
| 16CIF | 1408 | 1152 | 1622016 | 1408 | 192 | 270336 | 16.7% |
| 4SVGA | 1600 | 1200 | 1920000 | 1600 | 192 | 307200 | 16.0% |
| 1080 HD | 1920 | 1080 | 2073600 | 1920 | 192 | 368640 | 17.8% |
| 2Kx1K | 2048 | 1024 | 2097152 | 2048 | 192 | 393216 | 18.8% |
| 2Kx1080 | 2048 | 1080 | 2211840 | 2048 | 192 | 393216 | 17.8% |
| 4XGA | 2048 | 1536 | 3145728 | 2048 | 192 | 393216 | 12.5% |
| 16VGA | 2560 | 1920 | 4915200 | 2048 | 192 | 393216 | 8.0% |
| 3616x1536 (2.35:1) | 3616 | 1536 | 5554176 | 2048 | 192 | 393216 | 7.1% |
| 3672x1536 (2.39:1) | 3680 | 1536 | 5652480 | 2048 | 192 | 393216 | 7.0% |
| 3840x2160 (4*HD) | 3840 | 2160 | 8294400 | 2048 | 192 | 393216 | 4.7% |
| 4Kx2K | 4096 | 2048 | 8388608 | 2048 | 192 | 393216 | 4.7% |
| 4096x2160 | 4096 | 2160 | 8847360 | 2048 | 192 | 393216 | 4.4% |
| 4096x2304 (16:9) | 4096 | 2304 | 9437184 | 2048 | 192 | 393216 | 4.2% |
| 7680x4320 | 7680 | 4320 | 33177600 | 2048 | 192 | 393216 | 1.2% |
| 8192x4096 | 8192 | 4096 | 33554432 | 2048 | 192 | 393216 | 1.2% |
| 8192x4320 | 8192 | 4320 | 35389440 | 2048 | 192 | 393216 | 1.1% |

# Proposed Text Change

The modification of the text is based on “JCT3V-G1001 3D-HEVC Draft Text 3” and in green highlight.

    - Derivation process for disparity vectors

……

When dvAvailFlag is equal to 0, refViewIdx is set equal to DefaultRefViewIdx, and mvDisp is set equal to ( 0, 0 ).

mvDisp[0] = Clip3(-1023.75, 1024, mvDisp[0])

mvDisp[1] = Clip3(-CtbSizeY + 0.25, CtbSizeY, mvDisp[1])

Depending on depth_refinement_flag[ nuh_layer_id ], the following applies:

……

# Experimental Results

The two proposed method is implemented into HTM 10.0r1 software, and proposed method is compared with HTM 10.0r1. The test condition is CTC.

Experimental results show that such a limitation will not bring any coding loss and the encoding and decoding time are similar to anchor.

Table 3-1: Experimental results for proposed method

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.4% | 100.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.1% | 98.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.6% | 97.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102.1% | 105.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 104.3% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102.1% | 98.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.3% | 101.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.5% | 107.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 98.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 103.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 101.5% |

# Conclusion

This contribution proposes to limit NBDV into a predefined range. Such a limitation can reduce the cache miss when fetching the reference views and save the bandwidth or on-chip memory for hardware design. We recommend adopting the proposed constraints on NBDV.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- L. Zhang, Y. Chen and M. Karczewicz, “3D-CE5.h related: Disparity vector derivation for multiview video and 3DV,” MPEG document M24937, April 2012.

- J. Sung, M. Koo and S. Yea, “3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding,” JCT3V document A0126, July 2012.

- L. Zhang, Y. Chen and M. Karczewicz, “CE5.h: Disparity vector generation results,” JCT3V document A0097, July 2012.