| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0108 |
| --- | --- |

| Title: | Clean-up on DMM and SDC DC value derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Xu Chen, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution unifies DC prediction process for DMM1, DMM4, SDC DMM1 and SDC DMM4. The unification can simplify the implementation process and the standard specification description for DC prediction part. The experimental results of the proposed method are exactly identical as htm10.r1 anchor.

# Introduction

DMM1, DMM4, SDC DMM1 and SDC DMM4 intra modes use DC prediction process to derive the constant value for the decoding of the two partitions' reconstruction value. The DC derivation methods for those four intra modes are described as follows:

DMM1 / DMM4: The samples located at the neighbouring top row and left column is used to derive the DC prediction value. Two samples of the first sample, last sample or middle sample of the top reference sample row or the left reference sample column may be chosen to generate a DC predator [1]. The prediction process is illustrated in Figure 1.

(a) Case 1 				(b) Case 2

(c) Case 3 				(d) Case 4

Figure 1: Illustration of DMM1 / DMM4 DC prediction

SDC DMM1: Firstly, the prediction sample block for DMM1 is generated. Then, the samples located at the four corners of the prediction sample block are used to derive the DC values for SDC DMM1 which is illustrated in Figure 2.

Figure 2: Illustration of SDC DMM1 DC prediction

SDC DMM4: Firstly, the prediction sample block for DMM1 is generated. Then, the all of sample values for the prediction sample block are checked until finding out two different values that are used as DC values. The pseudo-code is shown as below:

As we can see from the above description, the DC prediction for DMM1, DMM4, SDC DMM1 and SDC DMM4 modes have similar concept but different derivation processes, which complicate the implementation design. This contribution proposes to unify the DC prediction process for those modes.

# Proposed method

The prediction block of SDC DMM1 and SDC DMM4 is composed of DC values for DMM1 and DMM4. The construction process of prediction block for SDC DMM1 and SDC DMM4 is shown at Figure 3 and Figure 4 respectively.  Considering that two DMM1 partitions always cover two different corners respectively and the whole SDC DMM1 prediction block only contains two different sample values as shown in Figure 1, the sample values for the four corners of the SDC DMM1 prediction block are identical to the two DC value of DMM1. Therefore, the derivation process for the checking of the four corner samples for SDC DMM1 mode is unnecessary.

Regarding SDC DMM4 mode, the prediction block also only contains two different sample values as shown in Figure 4. Therefore the two DC values for this mode generated from comprehensive scanning of the prediction block’s samples are also identical DMM4 DC values.

According to the above analysis, the DC values derivation process for SDC DMM1 and SDC DMM4 in current standard specification and software implementation can be bypassed. The DC values for DMM1 and DMM4 can be reused as SDC DMM1 and SDC DMM4’s DC. Therefore, the DC derivation process for DMM1, DMM4, SDC DMM1 and SDC DMM4 can be unified as a single process which can simplify the implementation and clean-up the specification. This clean-up process has the identical results as the old DC derivation processes.

Figure 3: Illustration of the construction of prediction block of SDC DMM1 mode

Figure 4: Illustration of the construction of prediction block of SDC DMM4 mode

# Experimental results

The proposed method is implemented on HTM10.0r1. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The table 1 and table 2 show the test results generated whose anchor is HTM10.0r1. From the test results, it can found that the proposed method can get the identical results as the anchor.

Table 1: Results for all - intra (anchor: htm10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.7% | 100.8% | 99.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 94.3% | 99.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.8% | 104.9% | 99.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.4% | 100.2% | 101.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.5% | 110.2% | 98.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 102.6% | 101.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.2% | 101.6% | 97.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 102.7% | 101.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.8% | 100.0% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 103.5% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 102.2% | 99.9% |

Table 2: Results for CTC (anchor: htm10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.6% | 98.0% | 98.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.6% | 105.3% | 98.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.9% | 98.3% | 101.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.8% | 94.5% | 100.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 81.0% | 95.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.8% | 94.8% | 98.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.8% | 81.6% | 96.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.9% | 109.2% | 99.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.4% | 100.6% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.5% | 92.2% | 98.2% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.4% | 95.3% | 98.7% |

# Conclusion

This contribution proposes a unification method for the derivation of DC prediction value for DMM1, DMM4, SDC DMM1 and SDC DMM4. The proposed method can get the identical results as the anchor, and can also simplify the implementation for DC calculation and clean-up the standard specification description. Therefore, it's recommended to incorporate the proposed method into the next version of HTM and working draft.

# Changes to the description of WD

Please refer the change to the attachment “JCT3V-H0108-WD.docx”

# References

[1] L. Zhang, G. Tech, K. Wegner, S.Yea, 3D-HEVC Test Model 7, JCT3V-G1005, SanJose, US, January, 2014

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).