| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0109 |
| --- | --- |

| Title: | 3D-CE2 related: Simplification of ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

3D-HEVC supports advanced residual prediction (ARP) for both of temporal residual and inter-view residual. This contribution proposes a simplification to remove 0.5 weighting for ARP process. The experimental results reportedly shows minor gain of 0.02% in synthesized.

# Introduction

The advanced residual prediction (ARP) for temporal and inter-view, as proposed in Error: Reference source not found, is illustrated in Figure 1.

(a) Temporal ARP			(b) Inter-view ARP

Figure 1. Prediction structure of ARP in 3D-HEVC.

When the ARP mode is invoked, the predicted residual is added on top of the prediction signal of the non-base view. Since quantization difference between base and non-base views may lead to less prediction accuracy, three weighting factors are applied to the residue generated from the reference view: 0, 0.5 and 1. It needs triple ARP process for encoding side to select best weighting factor that determined by RDO results.

# Proposed method

## Remove 0.5 weighting factor process for ARP

When the ARP process is invoked, the conventional weighting factor selection process is performed. The following steps are performed (the parts modified are highlighted in (removed) and yellow (changed):

Merge Process:

- Merge candidates list construction, Motion compensation and RDO for 0 weighting factor of ARP.

- Merge candidates list construction, Motion compensation and RDO for 1 weighting factor of ARP.

- 1bit for ARPW signalling.

Inter Process:

- Motion estimation, Motion compensation and RDO for 0 weighting factor of ARP.

- Motion estimation, Motion compensation and RDO for 1 weighting factor of ARP.

- 1bit for ARPW signalling.

# Experimental results

Table 1-2 shows the simulation results of the proposed method. Common test conditions specified in JCT3V-G1100[2] is used for the evaluation. The platform 3D-HTM 10.0r1Error: Reference source not found is utilized and the simulation results show that the probability of ARPW=0.5 selected is low and the proposed method has minor coding gain performances in aspect of synthesized BD performances. Also, the encoding time is less than the anchor.

Table 1.  Rate of 0.5 weighting factor selected in ARP

|  | (ARPW=0.5 selected)/ARP Cases |
| --- | --- |
| Rate | 0.053 |

Table 2.  Coding results for 0.5 weighting factor removal for ARP

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.2% | -0.05% | -0.03% | -0.08% | 98.7% | 92.2% | 101.3% |
| Kendo | 0.0% | 0.4% | 0.2% | 0.17% | 0.17% | 0.11% | 99.1% | 115.4% | 98.7% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.06% | 0.04% | -0.05% | 99.1% | 86.0% | 97.0% |
| GT_Fly | 0.0% | 0.0% | -0.1% | -0.01% | -0.01% | 0.00% | 98.6% | 95.6% | 99.9% |
| Poznan_Hall2 | 0.0% | 0.3% | -0.3% | 0.02% | 0.01% | -0.07% | 98.5% | 108.4% | 99.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.09% | 0.10% | 0.02% | 99.2% | 100.4% | 98.4% |
| Undo_Dancer | 0.0% | -0.2% | -0.2% | -0.04% | -0.04% | -0.07% | 98.3% | 92.7% | 106.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.02% | 98.1% | 92.4% | 95.3% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.06% | 0.06% | -0.01% | 99.0% | 97.9% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.02% | 0.01% | -0.03% | 98.5% | 97.9% | 99.9% |
| average | 0.0% | 0.0% | -0.1% | 0.03% | 0.03% | -0.02% | 98.7% | 97.9% | 99.6% |

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San Jose, USA, 9 Jan –17 Jan. 2014.

- Karsten Müller, Anthony Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan, 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/