| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0110 |
| --- | --- |

| Title: | 3D-CE3 related: Redundancy removal for DMM1 signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen1, Xiaozhen Zheng1, Ce Zhu2, Yongbing Lin1, Jianhua Zheng1 1 HiSilicon technologies 2School of Electronic Engineering University of Electronic Science and Technology of China, Chengdu, P. R. China,611731 | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com eczhu@uestc.edu.cn |
| Source: | HiSilicon Technologies, UESTC |  |  |

_____________________________

# Abstract

This contribution proposes a method to code the table index by the difference value which can shorten coding list index and fewer bit needed to be coded. The experimental results reportedly shows gain of 0.07% in synthesized.

# Introduction

In the current 3D-HEVC [1], the table index in DMM1 is binarizied as a fixed-length code. As coding bits for table index can be used for signaling more number than table index, redundancy happens. For example, 7 bits for 4×4 DMM1 table index coding will make 41 redundancy as table index for 4×4 DMM1 equal to 87(27-87=41).

Table 1. DMM1 table index coding bits needed

| Block size | Index number | Coding bits needed | Redundancy |
| --- | --- | --- | --- |
| 4×4 | 87 | 7 | 41 |
| 8×8 | 767 | 10 | 257 |
| 16×16 | 1351 | 11 | 697 |
| 32×32 | 1504 | 11 | 544 |

# Proposed method

- Set the constant value:23, 255, 327, 480 for the block size of 4×4 to 32×32 respectively, which can make the difference value between table index and constant value not more than 2coding bits-1. It means 1 bit can be saved by coding the difference value.

Table 2. Difference value coding bits needed

| Block size | Constant value | Difference value between Constant value and table Index | Coding bits needed | Redundancy |
| --- | --- | --- | --- | --- |
| 4×4 | 23 | <=64 | 6 | 0 |
| 8×8 | 255 | <=512 | 9 | 0 |
| 16×16 | 327 | <=1024 | 10 | 0 |
| 32×32 | 480 | <=1024 | 10 | 0 |

- Coding the difference value.

- If the difference value(DV) not greater than the constant value(CV), 1 more signed bit needed for signalling the table index equal to CV+DV or CV-DV.

- If the difference value(DV) greater than the constant value(CV), the table index equal to CV+DV.

Table 3. coding bits needed by proposed method

| Block size | Index number | Coding bits needed | Saving |
| --- | --- | --- | --- |
| 4×4 | >46 | 6 | 45.9% |
|  | others | 7 |  |
| 8×8 | >500 | 9 | 34.7% |
|  | others | 10 |  |
| 16×16 | >654 | 10 | 51.5% |
|  | others | 11 |  |
| 32×32 | >960 | 10 | 36.1% |
|  | others | 11 |  |

# Experimental results

Table 1-2 shows the simulation results of the proposed method. Common test conditions specified in JCT3V-G1100[2] is used for the evaluation. The platform 3D-HTM 10.0r1Error: Reference source not found is utilized and the simulation results show that the proposed process method has 0.07 coding gain performances in aspect of synthesized BD performances.

Table 4.  Result for 3-view scenario under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.06% | 0.02% | -0.01% | 99.2% | 80.1% | 95.4% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.01% | -0.05% | -0.08% | 98.2% | 101.8% | 97.0% |
| Newspaper_CC | 0.0% | -0.1% | -0.2% | -0.05% | -0.05% | -0.15% | 98.1% | 79.6% | 94.8% |
| GT_Fly | 0.0% | -0.1% | -0.2% | -0.03% | -0.03% | 0.01% | 97.9% | 105.9% | 98.9% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.4% | -0.05% | -0.04% | -0.19% | 98.0% | 96.4% | 91.4% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | -0.01% | -0.01% | -0.03% | 97.6% | 108.2% | 95.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | -0.06% | 97.9% | 102.3% | 104.0% |
| Shark | 0.0% | 0.0% | 0.0% | -0.01% | -0.03% | -0.05% | 97.6% | 93.9% | 92.9% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.00% | -0.03% | -0.08% | 98.5% | 87.2% | 95.7% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | -0.02% | -0.02% | -0.07% | 97.8% | 101.3% | 96.4% |
| average | 0.0% | 0.0% | -0.1% | -0.01% | -0.02% | -0.07% | 98.1% | 96.0% | 96.2% |

Table 5.  Result for 3-view scenario under all-intra configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.07% | 100.2% | 102.2% | 102.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.09% | 99.9% | 101.1% | 100.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.12% | 99.7% | 96.6% | 95.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 100.1% | 96.5% | 95.9% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.07% | 100.2% | 96.3% | 96.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.03% | 100.2% | 97.3% | 96.7% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.03% | 100.0% | 95.7% | 96.1% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 100.2% | 98.7% | 96.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.09% | 99.9% | 100.0% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 100.1% | 96.9% | 96.3% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.07% | 100.1% | 98.0% | 97.5% |

Additional results

Code the bins of difference value (and signed bit if needed) in DMM1 with the bypass mode.

Table 6.  Result for 3-view scenario under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | -0.1% | 0.01% | -0.02% | 0.00% | 80.7% | 64.7% | 80.1% |
| Kendo | 0.0% | 0.1% | -0.1% | -0.01% | -0.06% | -0.05% | 81.4% | 77.3% | 79.8% |
| Newspaper_CC | 0.0% | -0.1% | -0.1% | -0.03% | -0.06% | -0.10% | 81.0% | 64.6% | 78.1% |
| GT_Fly | 0.0% | 0.0% | -0.2% | -0.02% | -0.01% | 0.10% | 79.8% | 68.2% | 81.6% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.5% | -0.08% | -0.09% | -0.28% | 78.7% | 74.0% | 78.3% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | -0.01% | 0.00% | -0.02% | 79.7% | 77.2% | 82.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.02% | 79.5% | 73.5% | 85.6% |
| Shark | 0.0% | -0.2% | -0.1% | -0.03% | -0.06% | -0.09% | 78.6% | 65.8% | 75.1% |
| 1024x768 | 0.0% | 0.0% | -0.1% | -0.01% | -0.05% | -0.05% | 81.0% | 68.9% | 79.3% |
| 1920x1088 | 0.0% | -0.1% | -0.2% | -0.03% | -0.03% | -0.06% | 79.2% | 71.8% | 80.7% |
| average | 0.0% | 0.0% | -0.1% | -0.02% | -0.04% | -0.06% | 79.9% | 70.7% | 80.2% |

(time is not reliable as run by different server)

Table 7.  Result for 3-view scenario under all-intra configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.08% | 100.7% | 103.3% | 103.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.08% | 102.6% | 104.8% | 105.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.11% | 101.3% | 96.2% | 95.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 100.8% | 97.0% | 97.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.07% | 101.7% | 101.1% | 100.4% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.04% | 101.3% | 99.8% | 98.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.04% | 101.0% | 99.7% | 100.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 99.3% | 98.8% | 97.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.09% | 101.5% | 101.5% | 101.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.05% | 100.8% | 99.3% | 98.9% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.07% | 101.1% | 100.1% | 99.8% |

(time is not reliable as run by different server)

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

UESTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San Jose, USA, 9 Jan –17 Jan. 2014.

- Karsten Müller, Anthony Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan, 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/