| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0111 |
| --- | --- |

| Title: | CE1 related : The unification of sub-PU process for MPI blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to align the sub-PU motion-hole filling process for the MPI (Motion Parameter Inheritance) block - the first merge candidate in the depth merge list- with that for the IvMC (Inter-view Motion Compensation) process. In the current design, motion-holes (i.e. sub-PU blocks without motion vectors) appearing during the Sub-PU level IvMC are filled with CMP (Center Motion Parameters) of the corresponding blocks. In contrast, for the MPI blocks, which derive motion parameters from the collocated texture blocks at sub-PU level[1], motion-holes are filled with the latest motion parameter, not CMP. Therefore, it is proposed that CMP be used for motion-hole filling of the MPI blocks so that the Sub-PU level hole filling processes are unified in the IvMC and MPI cases. The experimental result shows there is no coding loss for the synthesized view while removing the need of saving the latest motion parameter.

# Introduction

At the last meeting, the Sub-PU hole filling process is adopted. According to JCT3V-G0147[2], the motion holes can be filled with the default motion vector, which is the motion vector in the center-positioned sub-PU of the corresponding PU. However, in the sub-PU MPI process, it still fills the motion holes with the latest motion vector. To unify the sub-PU motion hole filling process, it is proposed that the default motion vector of the MPI is defined in the same way of the Sub-PU IvMC process.

# Proposed method

The proposed method is to fill the motion holes in the sub-PU MPI blocks with the CMP. As the current sub-PU IvMC process is performed, firstly it checks the center sub-PU position. If there is a motion parameter, it is defined as a default motion vector. Then, the motion parameters of the collocated texture blocks are copied to the current block at the sub-PU level. If there is a sub-PU which does not have any motion parameter, this is filled with the default motion vector, as shown in Figure 1. If there is no CMP available, the Sub-PU MPI process is skipped.

Figure 1 The current motion hole filling process (a) and the proposed method (b)

# Experimental results

The experimental results is performed under the common test conditions[3]. The experimental result for the proposed method is shown in Error: Reference source not found. For the result of the synthesized view, the proposed method has no coding loss.

Table 1 The experimental result of the proposed method (Anchor : HTM-10.0r1)

# Conclusion

In this proposal, the hole filling of sub-PU for the MPI blocks is proposed. The proposed method unifies the hole filling process with the one of the current Sub-PU IvMC process without any loss. Therefore, it is recommended that this proposal is adopted to the next version.

# Reference

- Y. Chen, H. Liu, L. Zhang, “CE2:Sub-PU based MPI,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0119, San Jose, US, Jan. 2014.

- Y. S. Heo, M. S. Lee, W. W. Gwun, Y. J. Lee, G. H. Park, “3D-CE1.h related: Simplification of sub-PU level temporal interview motion prediction,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0147, San Jose, US, Jan. 2014.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, San Jose, US, Jan. 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).