| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0112 |
| --- | --- |

| Title: | CE2 related: Removal of ARP for AMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Jungdong Seo, and Sehoon Yea | Email: | junghak.nam@lge.com jungdong.seo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, advanced residual prediction (ARP) is used to SKIP, MERGE, and AMVP modes for dependent texture view coding with the 2Nx2N PU size. However, it is observed that the ARP for AMVP mode gives no impact on the overall coding performance while increasing the encoding complexity. Therefore, this contribution proposes removal of ARP for AMVP mode in 3D-HEVC. The proposed method incurs no coding loss in terms of the synthesized PSNR.

# Proposed method

This contribution proposes removal of ARP process for AMVP mode in 3D-HEVC. That means that ARP process is applied to SKIP mode and MERGE mode. Therefore, encoding complexity can be reduced by removing RD optimization process with ARP for AMVP mode.

# Proposed text

The changes for proposed text are highlighted in yellow.

          - Coding unit extension semantics

The variable rpEnableFlag is derived as specified in the following:

  - rpEnableFlag = iv_res_pred_flag[ nuh_layer_id ]  &&  RpRefPicAvailFlag &&
	( CuPredMode[ x0 ][ y0 ]  !=  MODE_INTRA )  &&  ( PartMode  = =  PART_2Nx2N ) && 
	 merge_flag[ x0 ][ y0 ]	(Error: Reference source not found-1)

iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0.

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

    - icEnableFlag = ( merge_idx[ x0 ][ y0 ] != 0 )  | |  !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-2)

- Otherwise (merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0  &&  refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1  &&  refViewIdxL0  !=  ViewIdx)	(Error: Reference source not found-3)

# Experimental results

The proposed method is integrated on 3D-HTM 10.0r1. The proposed method incurs no coding loss in terms of the synthesized PSNR, as shown in Table 1.

Table 1 Removal of ARP for AMVP mode under CTC

# Conclusion

This contribution proposed removal of ARP for AMVP mode in 3D-HEVC. It is recommended to adopt the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).