| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0113 |
| --- | --- |

| Title: | CE3 related: Alignment of WD and HTM for syntax element depth_dc_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Jungdong Seo, and Sehoon Yea | Email: | junghak.nam@lge.com jungdong.seo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

There is a mismatch in the number of context-models for syntax element depth_dc_flag between the HTM software and WD. While only one context is used in WD for depth_dc_flag, two contexts are used in HTM. This contribution proposes two solutions for alignment of the context for depth_dc_flag in WD and HTM. Solution 1) is that HTM is aligned to WD. Experimental results report minor coding loss on synthesized views in AI configuration. Solution 2) is that WD is aligned to HTM. It obviously needs no experiments and only requires the draft text change.

# Introduction

In the current 3D-HEVC, syntax element depth_dc_flag specifies that DC offsets for intra block coded by segment-wise DC coding (SDC) or depth modeling modes (DMM) not coded by SDC are present. However, there is a mismatch in the number of context models between the HTM software and WD. While one context is used in WD, two contexts are used in HTM. The context model for depth_dc_flag in the current WD is as follows:

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| cu_extension( ) intra_mode_ext( ) | depth_intra_mode_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Table Error: Reference source not found-2 | 0 | 1 | 2 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..2 | 3..5 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | dbbp_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dim_not_present_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

Table Error: Reference source not found-2 – Values of initValue for depth_dc_flag ctxIdx

| Initialization variable | ctxIdx of depth_dc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 0 | 0 | 64 |

Table Error: Reference source not found-3 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Error: Reference source not found | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0, 1,2 Error: Reference source not found | na | na | na | na | na |

However, in the HTM software, a separate context model is being used for the DC offsets of SDC and DMM, respectively, which is an obvious misalignment of the reference SW and the WD.

# Proposed method

This contribution proposes two solutions for alignment of context model for depth_dc_flag in WD and HTM.

Solution 1): HTM is aligned to WD. In that case, the only one context is used in the HTM software for depth_dc_flag without WD change.

Solution 2): WD is aligned to HTM. In that case, the two contexts are used in the WD for depth_dc_flag without HTM change.

# Proposed text

The changes for the proposed text of Solution 2) are highlighted in yellow.

        - Initialization process for context variables

The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses 7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses Error: Reference source not found to Error: Reference source not found.

- Table Error: Reference source not found-4 is appended to the end of Table 9-4.

- Error: Reference source not found to Error: Reference source not found are appended to the end of the subclause.

Table Error: Reference source not found-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| cu_extension( ) intra_mode_ext( ) | depth_intra_mode_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Table Error: Reference source not found-2 | 0,1 | 12,3 | 24,5 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..2 | 3..5 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | dbbp_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dim_not_present_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

.....

Table Error: Reference source not found-5 – Values of initValue for depth_dc_flag ctxIdx

| Initialization variable | ctxIdx of depth_dc_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 0 | 0154 | 640 | 154 | 64 | 154 |

.....

        - Derivation process for ctxTable, ctxIdx and bypassFlag

          - General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Table Error: Reference source not found-6 is appended to the end of Table 9-37.

Table Error: Reference source not found-6 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0, 1 (subclause I.9.3.4.2.8) | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Error: Reference source not found | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0, 1,2 Error: Reference source not found | na | na | na | na | na |

          - Derivation process of ctxInc using left and above syntax elements

.....

          - Derivation process of ctxInc for the syntax element depth_dc_flag

Input to this process is the variable DmmFlag[ x0 ][ y0 ]

Output of this process is ctxInc.

The variable NumSegments is derived as follows:

NumSegments = (DmmFlag[ x0 ][ y0 ]) ? 2 : 1

The variable ctxInc is derived as follows:

ctxInc = (NumSegments == 1) ? 0 : 1

# Experimental results

Solution 1) is integrated on 3D-HTM 10.0r1. The method achieves no coding loss in terms of the synthesized PSNR in the CTC configuration, as shown in Table 1. However, The method leads to a minor coding loss in terms of the synthesized PSNR in the AI configuration, as shown in Table 2.

Table 1 Experimental results of Solution 1) under CTC

Table 2 Experimental results of Solution 1) under AI

# Conclusion

This contribution proposes alignment of context model for depth_dc_flag syntax. It is recommended to adopt the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).