| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0114 |
| --- | --- |

| Title: | Simplification of DC calculation for SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Jungdong Seo, and Sehoon Yea | Email: | junghak.nam@lge.com jungdong.seo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the DC value of SDC for HEVC intra prediction modes is calculated as the average of four-corner samples in a prediction block. This contribution proposes simplification of DC calculation for SDC. The proposed method is calculated as the average using samples of different positions according to the intra prediction mode. Experimental results show the proposed method leads to no coding loss in terms of the synthesized PSNR.

# Introduction

In the last meeting, the Segment-wise DC Coding (SDC) approach was generalized all depth intra prediction modes including HEVC intra prediction modes and depth modeling modes. SDC process utilizes the mean of the predicting depth values both encoder and decoder sides. The mean value for HEVC intra prediction modes is calculated as the average of the left-top, right-top, left-bottom, and right-bottom samples in a predicted block as shown in Figure 1.

Figure 1 Selection of samples for calculating the predicted depth value in SDC

According to intra prediction mode, some samples of the left-top, right-top, left-bottom, and right-bottom samples are identical. For example, in the case of horizontal prediction mode, the left-top sample is equal to right-top sample. Similarly, the left-top sample is equal to left-bottom sample in the case of vertical prediction mode.

# Proposed method

This contribution proposes simplification of DC calculation for SDC. The proposed method is calculated as the average using samples of different position according to intra prediction mode. As shown in Figure 2(a), left side samples which mean left-top and left-bottom samples are used for one of prediction modes from INTRA_ANGULAR2 to INTRA_ANGURA11 having a horizontal direction. The mean of these modes is computed by

dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1 ] + 1 )  >>  1

As shown in Figure 2(b), upper side samples which mean left-top and right-top samples are used for one of prediction modes from INTRA_ANGULAR26 to INTRA_ANGURA34 having a vertical direction. The mean of these modes is computed by

dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ nTbS − 1 ][ 0 ] + 1 )  >>  1

(a)                              (b)

Figure 2 Proposed selection of samples for calculating the predicted depth value in SDC

The mean for remaining HEVC intra prediction modes is still calculated four corner samples same as anchor.

# Proposed text

The changes for the proposed text are highlighted in yellow.

        - Segmental depth intra coding process

Inputs to this process are:

- a luma location ( xTb, yTb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nTbS specifying the transform block size,

- the intra prediction mode predModeIntra.

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = 0.. nTbS − 1.

- The array of predicted samples predSamples[ x ][ y ], with x, y =0..nTbS − 1, is derived as follows:

- The variable nPartTbS is set equal to Min( nTbS, 1  <<  Log2MaxTrafoSize ).

- The variable numPartsInRow is set equal to ( nTbS / nPartTbS ).

- For i in the range of 0 to ( numPartsInRow * numPartsInRow − 1), inclusive, the following applies:

  - Let xOffset and yOffset be equal to the values xA and yA, respectively, for which MinTbAddrZs[ xA ][ yA ] is equal to i.

  - The general intra sample prediction process as specified in subclause 8.4.4.2.1 is invoked with the transform block location ( xTb + xOffset * nPartTbS , yTb + yOffset * nPartTbS ), the intra prediction mode predModeIntra, the transform block size nPartTbS, and the variable cIdx as inputs, and the output is an (nPartTbS)x(nPartTbS) array predSamplesPart.

    - [ Ed. (GT): It needs to better specified, which are the neighbouring samples, when above process is invoked. HEVC version 1 text is in general not so specific about this. Here things get worse, since for some neighbouring blocks only the prediction signal (without SDC residual) is available at this point, whereas for other blocks the reconstructed values are available. The intention here seems to use the temporally set resSamples as derived below as neighbouring samples. However, this still needs to be better specified (by clarifying also 8.4.4.2.1 in the base spec).]

  - For x, y = 0..nPartTbS − 1, inclusive, predSamples[ x + xOffset * nPartTbS ][ y + yOffset * nPartTbS ] is set equal to predSamplesPart[ x ][ y ].

  - For temporary use in the following intra sample prediction process, the following assignment is made:

    - For x, y = 0..nPartTbS − 1, inclusive, resSamples[ x + xOffset * nPartTbS ][ y + yOffset * nPartTbS ] is set equal to predSamplesPart[ x ][ y ].

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows:

- If predModeIntra is equal to INTRA_DMM_WFULL, the following applies:

    - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]

- Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the derivation process for a texture predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sample location ( xTb, yTb ), and the variable nTbS as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

- Otherwise (predModeIntra is not equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX), the following applies:

  - For x, y = 0..nTbS − 1 wedgePattern[ x ][ y ] is set equal to 0.

Depending on DltFlag[ nuh_layer_id ] the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

- If DltFlag[ nuh_layer_id ] is equal to 0, the following applies:

  - For x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - SL[ xTb0 + x ][ yTb0 + y ] =  predSamples[ x ][ y ] + DcOffset[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-1)

- Otherwise (DltFlag[ nuh_layer_id ] is equal to 1), the following applies:

  - The variables dcPred[ 0 ] and dcPred[ 1 ] are derived as specified in the following:

    - If predModeIntra is not equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX, the following applies:

        - dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1 ] + predSamples[ nTbS − 1 ][ 0 ]
		+ predSamples[ nTbS − 1 ][ nTbS − 1 ] + 2 )  >>  2	(Error: Reference source not found-2)

    - Otherwise if predModeIntra is equal to INTRA_DMM_WFULL, the following applies:

        - dcPred[ wedgePattern[ 0 ][ 0 ] ] = predSamples[ 0 ][ 0 ]	(Error: Reference source not found-80)

        - dcPred[ wedgePattern[ nTbS − 1 ][ 0 ] ] = predSamples[ nTbS − 1 ][ 0 ]	(Error: Reference source not found-81)

        - dcPred[ wedgePattern[ 0 ][ nTbS − 1 ] ] = predSamples[ 0 ][ nTbS − 1 ]	(Error: Reference source not found-82)

        - dcPred[ wedgePattern[ nTbS − 1 ][ nTbS − 1 ] ] = predSamples[ nTbS − 1 ][ nTbS − 1 ]	(Error: Reference source not found-83)

    - Otherwise if (predModeIntra is equal to INTRA_DMM_CPREDTEX), the following applies:

dcPred[ wedgePattern[ 0 ][ 0 ] ] = predSamples[ 0 ][ 0 ]
				foundFlag = 0
				for ( x = 0; x < nTbS; x++ )
					for ( y = 0; y < nTbS; y++ )	(Error: Reference source not found-84)

if ( found Flag  = =  0  &&  wedgePattern[ x ][ y ]  !=  wedgePattern[ 0 ][ 0 ]) {
							dcPred[ wedgePattern[ x ][ y ] ] = predSamples[ x ][ y ]
							found Flag= 1
						}

    - Otherwise if predModeIntra is greater than 1 and less than 11, the following applies:

dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1  ] + 1 )  >>  1	(Error: Reference source not found-85)

    - Otherwise if predModeIntra is greater than 25 and less than 35, the following applies:

dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ nTbS − 1 ][ 0 ] + 1 )  >>  1	(Error: Reference source not found-86)

    - Otherwise, the following applies:

        - dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1 ] + predSamples[ nTbS − 1 ][ 0 ] + predSamples[ nTbS − 1 ][ nTbS − 1 ] + 2 )  >>  2	(Error: Reference source not found-87)

  - For x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-88)

      - dltIdxResi = DcOffset[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-89)

      - SL[ xTb0 + x ][ yTb0 + y ] = predSamples[ x ][ y ] + 
	Idx2DepthValue[ dltIdxPred + dltIdxResi ] − dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-90)

# Experimental results

The proposed method is integrated on 3D-HTM 10.0r1. The proposed method incurs no coding loss in terms of the synthesized PSNR, as shown in Table 1.

Table 1 Simplification of DC calculation for SDC under CTC

Table 2 Simplification of DC calculation for SDC under AI

# Conclusion

This contribution proposed simplification of DC calculation for SDC in 3D-HEVC. It is recommended to adopt the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).