| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0115 |
| --- | --- |

| Title: | Simplification of virtual depth map derivation for DBBP |  |  |
| --- | --- | --- | --- |
| mofeStatus: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Jungdong Seo, and Sehoon Yea | Email: | junghak.nam@lge.com jungdong.seo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the last meeting, the depth-based block partitioning (DBBP) method was adopted in 3D-HEVC. In the DBBP method, an arbitrarily shaped block partitioning is derived based on a binary segmentation mask computed from the virtual depth map which is derived by depth oriented neighboring block based disparity vector (DoNBDV). Therefore, it is required to fetch two depth blocks to get the virtual depth map. To reduce the number of depth fetching, this contribution proposes to use NBDV instead of DoNBDV to derive the virtual depth map. The proposed method incurs no coding loss in terms of the synthesized PSNR.

# Introduction

In the DBBP mode, a disparity vector derived from the DoNBDV process is applied to identify a corresponding depth block in a reference view. It is required to fetch two depth blocks to get the virtual depth map. The first depth fetching is to obtain a refined disparity vector derived from DoNBDV process. The second depth fetching is to obtain the virtual depth block to compute binary segmentation mask on DBBP.

# Proposed method

To remove overheads of depth fetching, this contribution proposes use of a disparity vector derived from NBDV process to get the virtual depth map. In other words, the proposed method remove use of a disparity vector derived from DoNBDV process in DBBP mode even though a derivation process for a disparity vector could be refined by the depth view components.

# Proposed text

The changes of the proposed text are highlighted in yellow.

      - Derivation process for a depth predicted contour pattern

Inputs to this process are:

- a variable sampInt specifying the sampling interval,

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nTbS specifying the transform block size.

Output of this process is:

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations ( xP, yP ) equal to ( xTb, yTb ), the disparity vector mvDisp equal to MvRefinedDisp[ xTb ][ yTb ] MvDisp[ xTb ][ yTb ], the view identifier refViewIdx equal to RefViewIdx[ xTb ][ yTb ], the variables nPbW and nPbH, equal to nTbS, and the variable partIdc equal to 1 as inputs, and the output is the array refSamples of size (nTbS)x(nTbS) and the flag horSplitFlag.

The derivation process for an inter-layer predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sampling interval sampInt, the block size nTbS and the reference sample array refSamples[ x ][ y ] as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ].

# Experimental results

The proposed method is integrated on 3D-HTM 10.0r1. The proposed method incurs no coding loss in terms of the synthesized PSNR, as shown in Table 1.

Table 1 Simplification of virtual depth map derivation for DBBP under CTC

# Conclusion

This contribution proposes use of NBDV instead of DoNBDV to derive the virtual depth map. Experimental results are shown that the proposed method have no impact on the coding performance under CTC configuration. It is recommended to adopt the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).