| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0116 |
| --- | --- |

| Title: | Removal of delta DC values for DMM4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Jungdong Seo, and Sehoon Yea | Email: | junghak.nam@lge.com jungdong.seo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, each partition of depth modeling modes (DMM) can be coded together with an optional delta DC value. However, it is observed that compensation of delta DC values for DMM4 mode gives no impact on the overall coding performance while increasing the encoding complexity. Therefore, this contribution proposes removal of delta DC values for the DMM4 mode in 3D-HEVC. The proposed method incurs no coding loss in terms of the synthesized PSNR both under the CTC and AI configurations.

# Introduction

In the current 3D-HEVC, each partition of DMM1 or DMM4 modes can be coded together with an optional delta DC value. One syntax element which is depth_dc_flag is firstly coded to indicate whether all the delta DC values of each partition are equal to 0. If there is at least one non-zero delta DC values for one partition, the magnitude of the delta DC value and the sign flag are transmitted. In the encoder, RD optimization is needed to find optimal delta DC value for several DC value candidates.

# Proposed method

This contribution proposes removal of delta DC values for DMM4 mode in 3D-HEVC. That means that compensation of delta DC values for DMM is only applied to DMM1 mode. Therefore, encoding complexity can be reduced by removing RD optimization process for determining optimal DC values of DMM4.

# Proposed text

The changes for the proposed text are highlighted in yellow.

          - Intra mode extension semantics

The variable Log2MaxDmmCbSize is set equal to 5.

dim_not_present_flag[ x0 ][ y0 ] equal to 1 specifies that the depth_intra_mode_flag syntax element is not present and that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. dim_not_present_flag[ x0 ][ y0 ] equal to 0 specifies that that the depth_intra_mode_flag syntax element might be present. When not present, the value of dim_not_present_flag[ x0 ][ y0 ] is inferred to be equal to 1.

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] = !dim_not_present_flag[ x0 ][ y0 ]  	(Error: Reference source not found-1)

depth_intra_mode_flag[ x0 ][ y0 ] is used to specify the depth intra mode of the current prediction unit.

The variable DepthIntraMode[ x0 ][ y0 ] is derived as specified in the following:

  - DepthIntraMode[ x0 ][ y0 ] = dim_not_present_flag[ x0 ][ y0 ] ? −1 : depth_intra_mode_flag[ x0 ][ y0 ]	(Error: Reference source not found-2)

Table Error: Reference source not found-1 specifies the value for the depth intra mode and the associated names.

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name

| DepthIntraMode | Associated name |
| --- | --- |
| −1 | INTRA_DEP_NONE |
| 0 | INTRA_DEP_DMM_WFULL |
| 1 | INTRA_DEP_DMM_CPREDTEX |

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] = (!dim_not_present_flag[ x0 ][ y0 ] && !depth_intra_mode_flag[ x0 ][ y0 ])  	(Error: Reference source not found-3)

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

# Experimental results

The proposed method is integrated on 3D-HTM 10.0r1. The proposed method incurs no coding loss in terms of the synthesized PSNR both CTC and AI configurations.

Table 1 Removal of DMM4 DC mode under CTC

Table 2 Removal of DMM4 DC mode under AI

# Conclusion

This contribution proposed removal of delta DC values for DMM4 mode. It is recommended to adopt the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).