| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0117 |
| --- | --- |

| Title: | 3D-CE1 related: Simplification on merge list for depth video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the last meeting, the disparity derived from depth (DDD) technique was adopted for depth video coding. The method predicts depth values from the disparity vector of the collocated texture block. It reportedly showed 0.2% BD rate gain in the synthesized view. However, the technique introduces an additional disparity-to-depth conversion process in the decoding process and renders the merge-list pruning process more complicated. This contribution proposes to remove the DDD candidate from the depth merge-list. The experimental results reportedly show 0.1% BD-rate loss in synthesized views.

# Introduction

Disparity derivation from depth (DDD) technique is adopted in last meeting for depth video coding [1]. It predicts depth values from disparity vector of the collocated texture block. The process is inserted in second position of merge candidate list of depth video after the motion parameter inheritance (MPI) method. The experimental results of the method reportedly showed 0.2% DB rate gain in synthesized view. However, it has some limitations.

The DDD method has different characteristics with other merge candidates. They contain motion parameters such as reference index and motion vectors, and the parameters have physical meaning as vector data in decoding process. On the other hand, motion parameter of the DDD method cannot have physical meaning as vector, and it has to be converted to predicted depth values for decoding.

Because of the difference, it is hard to perform the pruning process in merge candidate list. The pruning is processed by comparison between motion parameters of candidates, but their motion parameters have different physical meaning. To perform the pruning on the DDD method, the comparison has to be processed in depth value domain.

It also has limitation in aspect of coding performance. In last meeting, the MPI method was improved with Sub-PU concept. Since the DDD technique is located after the MPI in merge list, its performance is influenced by the enhancement of the MPI. Therefore, it is proposed to remove the DDD method.

# Experimental results

The proposed methods are integrated into HTM-10.0 [2], and the tests are conducted under the common test condition (CTC). Table 1 shows the coding results of the removal of DDD method.

# Conclusion

In this contribution, the simplification of merge list is proposed for depth video. Since the DDD technique has some limitations, it is proposed to remove the method in merge list. The proposed method reportedly shows 0.12% BD-rate loss in the synthesized view.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Zhang, J. An, J. Lin, Y. Chang, S. Lei, “3D-CE2: Results on additional merging candidates for depth coding,” JCT3V-G0063, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.

[2] HTM-10.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0

[3] K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,”JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.