| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0118 |
| --- | --- |

| Title: | 3D-CE2 related: Separation of syntax elements for ARP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

The syntax element for the ARP (Advanced Residual Prediction) mode is iv_res_pred_weight_idx in 3D-HEVC. It can have integer values ranging from 0 to 2 and represents both on/off-status of the ARP and the weighting factor when it is on. It undergoes binarization into 0 (ARP off), 10 (ARP on), or 11 (ARP on with a weighting factor of 0.5. This contribution proposes to replace iv_res_pred_weight_idx with two separate one-bit flags for a clearer description of the syntax. The first flag (ARP_mode_flag) indicates whether ARP is on, and the second one (ARP_weight_flag) indicates if the weighting factor is 1 or 0.5. The numbers and types of context models for the two flags are identical with those used for binarized bins of the current syntax element (i.e. iv_res_pred_weight_idx). Experimental results confirm the proposed method does not affect coding-efficiency

# Introduction

In the current 3D-HEVC, advanced residual prediction (ARP) mode is used to reduce the residual data in motion compensation process [1]. To signal the mode, one syntax element is employed, iv_res_pred_weight_idx. It presents both on/off and weighting factor of the mode. However, they are difference information from each others, and treated as separated syntax elements in reference software. Therefore, it is proposed to separate the syntax elements for ARP mode as on/off flag and weighting factor flag.

# Proposed method

This contribution proposes to separate the syntax element for ARP mode as two flags. ARP flag indicates on/off of the mode, and ARP weight flag presents whether weight factor of ARP mode is 1 or 0.5. The CABAC context models are also separated.

# Experimental results

The proposed methods are integrated into HTM-10.0r1 [2], and the tests are conducted under the common test condition (CTC) [3]. Table 1 shows the coding results of the separated syntax elements for ARP.

Table 1. Experimental results of the syntax separation for ARP

# Conclusion

In this contribution, separation of syntax element for ARP mode is proposed. The proposed method can make 3D-HEVC text be aligned to reference software. The experimental results reportedly show that the results of the proposed are the same as the results of the reference software.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] G. Tech , K. Wegner , Y. Chen, S. Yea “3D-HEVC Draft Text 3,“ JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.

[2] HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

[3] K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,”JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.