| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0119 |
| --- | --- |

| Title: | 3D-CE3 related: Simplification on CABAC contexts for the syntax related to depth intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC depth intra coding, three context models are employed for encoding of dim_not_present_flag to indicate whether the current block is encoded as conventional (i.e. HEVC) intra mode or non-conventional intra mode. One of these three context-models is chosen depending on dim_not_present_flag’s of the left and the above neighbors. This contribution proposes to remove such dependency and use one context-model for dim_not_present_flag. Experimental results reportedly show that the proposed method does not cause any BD-rate loss under the CTC and AI condition.

# Introduction

In the 3D-HEVC, three CABAC context models are used for dim_not_present_flag to indicate whether the current block is encoded as HEVC intra mode or depth intra mode [1] as shown in Table 1. Among them, only one context model is selected based on the status of the neighboring blocks. The context index of the context is derived as follows:

ctxIdx = dim_not_present_flag(L) + dim_not_present_flag(U)

,where L and U means left and above blocks of the current block. Because the context information of upper block is used for encoding of the current block, line buffers are needed to process the information of upper CTU row.

Table 1. Values of initValue for dim_not_present_flag ctxIdx

| Initialization variable | ctxIdx of dim_not_present_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 155 | 156 | 141 | 185 | 214 | 155 | 170 | 157 |

# Proposed methods

This contribution proposes to simplify the context models for the dim_not_present_flag. The proposed methods use only one context as a candidate without referring to any information of neighboring blocks. For the context model, initial values are selected (154, 141, 155) for each picture type based on JCT3V-G0122 [1], and CNU values are also considered.

# Experimental results

The proposed methods are integrated into HTM-10.0r1 [2], and the tests are conducted under the common test condition(CTC) and all intra condition (AI) [3]. Table 2 and 3 show the coding results of one context model with initial values based on JCT3V-G0122 in CTC and AI, respectively. Table 4 and 5 present the results of one context model with CNU initial values in CTC and AI, respectively.

Table 2. One context model with initial values for dim_not_present_flag in CTC

Table 3. One context model with initial values for dim_not_present_flag in AI

Table 4. One context model with CNU values for dim_not_present_flag in CTC

Table 5. One context model with CNU values for dim_not_present_flag in AI

# Conclusion

In this contribution, simplified context model derivation methods are proposed for encoding of dim_not_present_flag. The proposed methods do not cause any BDBR increase.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] H. Liu, Y. Chen “CE5: Generic SDC for all Intra modes in 3D-HEVC ,“ JCT3V-G0122, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.

[2] HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

[3] K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,”JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.