| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0120 |
| --- | --- |

| Title: | 3D-CE3 related: Simplification on CABAC contexts for advanced residual prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In ARP (Advanced Residual Prediction), three CABAC context models are used to encode iv_res_pred_weight_idx. Among them are two context models used for the bit indicating on/off of ARP, and one context model is employed for the bit indicating the weight value when ARP is on. This contribution proposes to use only one context model for the ARP on/off bit as in the ic_flag case thus removing the need of saving ARP flags of the left-neighbor CUs. Experimental results reportedly show 0.03% BD rate loss in synthesized view under the common test condition.

# Introduction

In the current 3D-HEVC, two CABAC context models are used to indicate on/off of advanced residual prediction (ARP) [1]. Among them, only one context model is selected based on the status of the neighboring block. The context index of the context is derived as follows:

ctxIdx = iv_res_pred_weight_idx (L)

where L means left block of the current block, and iv_res_pred_weight_idx( ) returns 1 when ARP mode is applied on the block, otherwise it returns 0. However, almost syntax elements of intra mode and coding unit extension use one context model for each picture type as shown in Table 1. Since the ARP mode information of the current block is not used for next blocks, the context simplification can help to reduce the complexity of the process. Therefore, it is proposed to simplify the CABAC context for the ARP mode indication.

Table 1. Association of ctxIdx and syntax elements of extension syntax

| Syntax element | initType |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| depth_intra_mode_flag | 0 | 1 | 2 |
| wedge_full_tab_idx | 0 | 1 | 2 |
| depth_dc_flag | 0 | 1 | 2 |
| depth_dc_abs | 0 | 1 | 2 |
| iv_res_pred_weight_idx |  | 0..2 | 3..5 |
| ic_flag |  | 0 | 1 |
| sdc_flag | 0 | 1 | 2 |
| dim_not_present_flag | 0..2 | 3..5 | 6..8 |

# Proposed methods

The proposed method simplifies the context models for the iv_res_pred_weight_idx. It uses only one CABAC context model without referring to any information of neighboring blocks. Initial values of the context model are selected 162 for each picture type based on JCT3V-G0061.

# Experimental results

The proposed methods are integrated into HTM-10.0r1 [2], and the tests are conducted under the common test condition (CTC) [3]. Table 2 show the coding results of one context model in CTC.

Table 2. Experimental results of the proposed method vs. HTM-10.0r1 in CTC

# Conclusion

In this contribution, simplified CABAC context model derivation methods are proposed for encoding of ARP mode. The proposed method reportedly shows 0.03% BDBR loss in synthesized view of CTC.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] G. Tech , K. Wegner , Y. Chen, S. Yea “3D-HEVC Draft Text 3,“ JCT3V-G1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.

[2] HTM-10.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1

[3] K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,”JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San José, CA.