| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0123 |
| --- | --- |

| Title: | CE1: Results on simplification for VSP merging candidate construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Lu Yu | Tel: Email: | +86-571-87953107 felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

In the last meeting, a simplified method for VSP merging candidate construction was proposed (JCT3V-G0104). In the proposed method, the VSP flag storage and checking process are removed by disabling VSP inheritance and only one default VSP candidate is inserted to a fixed position in the merge candidate list. The simulation results show that the proposed method has no impact on the coding performance (0.00%).

# Introduction

In the current design of 3D-HEVC, there might exist multiple VSP candidates with totally same prediction blocks for the following reasons:

- VSP candidates are inherited from neighboring blocks of current PU and multiple VSP-coded neighboring blocks results in multiple VSP candidates in merge candidate list (MCL);

- All the VSP modes use the NBDV as the DV for virtual depth block fetching and conduct backward motion information warping using the disparities converted from the virtual depth block.

So with the same DV for virtual depth block fetching, all the VSP-coded blocks would have all the same synthesis results.

The reason that we have multiple VSP candidates was that DVs for virtual depth block fetching were different in the previous design of 3D-HEVC. But with the unified DV, i.e., NBDV, there’s no need to keep multiple VSP candidates in MCL.

Besides, currently we are trying to reduce the dependency between the HEVC and 3D-HEVC MCL construction process, the VSP inheriting mechanism requires checking of the VSP flags of the spatial neighboring blocks and leads to additional memory access and VSP flag storage.

So we propose to reduce the number of VSP candidates in MCL and remove the VSP flags checking process as well as the VSP flag storage.

# Proposed Method

## Removal of VSP flag storage

In the proposed method, the VSP candidates are no longer inherited from the spatial neighboring blocks. For those candidates corresponded to a VSP-coded neighboring block, they are treated as normal DCP candidates (as shown in Fig. 1). The DV stored for such spatial neighboring block is used for inter-view prediction of its corresponding DCP candidate in MCL.

Fig. 1. VSP candidates are inherited as DCP candidates

As shown in Fig. 1, candidate A1, B1, and A0 in MCL is inherited as DCP candidate. DVA0, DVA1, and DVB1 are used for inter-view prediction of candidate A0, A1, and B1, respectively.

With the VSP inheritance disabled, the VSP flag storage for spatial neighboring blocks is no longer needed.

## VSP candidate in fixed position

In the current design, the default VSP candidate is inserted right after the DV candidate in MCL, which means the location of VSP candidate varies based on the construction results of its previous candidates in MCL.

Here we propose to put the default VSP candidate (also the only VSP candidate) in a fixed position, i.e. fourth position in MCL, as shown in Fig. 2.

Fig. 2. Fixed VSP position

With the fixed position of VSP candidate, a decoder could skip the MCL construction process by directly setting the coding mode of current PU as VSP mode when the merge index is equal to 3 (corresponded to the fourth position in MCL) and the VSP candidate is always put in a relatively higher place than the default VSP candidate in current design.

# Simulation Results

The proposed method was implemented on the latest reference software HTM10.0r1 and only CTC case is tested. The simulation results are shown in Table 1, and it shows that no coding loss is observed on average (0.00%).

Table 1. Performance of proposed method vs HTM10.0r1

# Conclusion

The proposed method simplifies the VSP candidate construction process of merge candidate list, without impact on the coding performance on average. It is recommended to adopt the proposed method.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).