| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0127 |
| --- | --- |

| Title: | AHG8: Complexity Assessment on Depth Intra Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gwo Giun Lee Chun-Fu Chen Tomohiro Ikai Zheng-Han Yu No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University and Sharp Corporation |  |  |

_____________________________

# Abstract

This proposal provides a case study of complexity assessment on depth intra modes in 3D-HEVC and complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed. The analyzed results in this proposal might be considered as anchor complexity to assist JCT3V in evaluating the proposals coming from several proponents in a fair manner. In this proposal, the issue in the size of wedgelet pattern is also issued. The analysis result suggests if the maximum size of DMM increases to 6464, then storage size will be dramatically increased with respect to the maximum size is 3232. This proposal also presents the experimental results without depth modeling modes (DMM) coding tools. When DMM 1 and DMM 4 are both disable, the results reveal that the coding loss is 0.0%, -0.23%, and 5.73% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively in all intra coding condition; on the other hand, in CTC coding condition, the results reveal that the coding loss is 0.11%, -0.15%, and 2.39% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively.

# Introduction and Problem Statement

The complexity issue has been studied for several decades; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity will be the platform-dependent and will be varied on different platform since the analyzed results cannot be transparent to another platform. The proposals in previous JCT3V meetings [1]-[10] provide a guideline to evaluate the intrinsic complexity of an algorithm. Thus, this proposal follows the analysis method to extract the intrinsic algorithmic complexity that is platform-independent and transparent to platforms. The coding tools, depth intra mode, including depth modeling mode 1 (DMM 1), DMM 4m and angular mode, would be considered as an exercise to assess its complexity in this proposal. On the other hand, the issue of wedgelet pattern would also be discussed in this proposal.

# Method of Complexity Assessment

To assess the complexity, the flowchart for completing one data granularity should be established first to determine data causality. One important issue should be concerned during the flowchart construction; the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provides a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules at various data granularities. The data granularity means that amount of data should be performed or produced within one process.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be mathematical expressions due to the fact that mathematical expressions are more intrinsic to algorithms and mathematical expressions would not be affected by platform. When encountering some if-else conditions, the worst case of conditions would be considered. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g., the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data transfer rate

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred during the process of one data granularity. The intermediate data would not be taken into consideration since these data would be disappeared after finishing the process. In addition, transfer rate of input data and output data shall be discussed individually since the characteristic of sending data and fetching data are different.

In this proposal, the flowchart is constructed according to the specification [9] and the test model [10] and the major process is defined according to that is there any data to be processed or not. The data granularities studied in this proposal is one 3232 block with all supported PU sizes.

# Complexity Assessment

## DMM 1

### Algorithmic description

According to the specification text and the test model, the flowchart of DMM 1 is shown in Fig. 1. The major processing of DMM 1 is the processes containing derivation and assignment of depth partition value and segmental depth intra coding. According to flowchart, the complexity of DMM 1 is assessed at various data granularities and the details are described in the following sections.

Fig. 1 Flowchart of DMM 1

### Wedgelet pattern

The maximal block size of DMM in current 3D-HEVC is 32×32. Although, the wedgelet pattern is pre-defined in spec text; the designer can stored the table in advance. In general, if the designer stores the pattern table in advance, the accumulated bits from 4×4 block size to 32×32 block size are 1,935,072 bits; on the other hand, if the DMM supports the block size of 64×64, the accumulated bits will be dramatically increased to 26,834,656 bits. (The increasing ratio is about 14 times when the maximal block size of DMM is 32×32.). Consequently, this is a reason that the DMM would not support 64×64.

Compared with 32×32 block, DMM pattern size reaches 236 times, suggesting it might be difficult to store the whole tables in fast access storage, such as cache or SRAM. Compared with one full-HD picture with YUV 420 color format (1920×1088×1.5 bytes), DMM pattern size is below 10% (7.72%), suggesting storing whole DMM table in large-scale storage, such as DRAM, is possible. It is remarked that DMM table fetching from large storage to fast access storage is required in some case. Thus the data size and frequency should be considered in terms of data transfer rate (See 3.1.5).

Table I Number of bits for wedgelet pattern at different block size

| PU size | Number of possible wedgelet pattern | Number of bits for one wedgelet pattern | Total bits | Accumulated bits | % w.r.t 32×32×8 bits | % w.r.t. one 19201088 picture |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 16 | 1,376 | 1,376 | 17% | 0.01% |
| 8×8 | 766 | 64 | 46,024 | 50,400 | 615% | 0.20% |
| 16×16 | 1,350 | 256 | 345,600 | 396,000 | 4,834% | 1.58% |
| 32×32 (worst case) | 1,503 | 1,024 | 1,539,072 | 1,935,072 | 236,21% | 7.72% |
| 64×64♪ | 6,079 | 4,096 | 24,899,584 | 26,834,656 | 327,571% | 107.05% |
| The result of 6464 block is derived the equation in spec text by setting the variable, block size, as 64 |  |  |  |  |  |  |

### Number of operations

- Derive and assign depth partition value: 
DLT ON: 3+N2 additions/subtractions, 3+3N2 LUT access
DLT OFF: 3+N2 additions/subtractions, 3+N2 LUT access

vertEdgeFlag = (partitionPattern[0][0] != partitionPattern[N-1][0])? 1 : 0;
horEdgeFlag = (partitionPattern[0][0] != partitionPattern[0][N-1])? 1 : 0

If (vertEdgeFlag == horEdgeFlag)

dcValLT = (neiSamples[-1][0] + neiSamples[0][-1]) >> 1;

If (horEdgeFlag == 0), and neiSamples[-1][2N-1] and neiSamples[2N-1][-1] are both available,

dcValBR = abs(neiSamples[-1][2N-1] - neiSamples[-1][0]) > 
abs(neiSamples[2N-1][-1] - neiSamples[0][-1]) ? 
neiSamples[-1][2N-1]: neiSamples[2N-1][-1]

Otherwise (horEdgeFlag != 0)

dcValBR = horEdgeFlag ? (neiSamples[-1][N-1]+neiSamples[N-1][-1]) >>1 : 2BitDepth-1

Otherwise (vertEdgeFlag!= horEdgeFlag)

dcValBR = horEdgeFlag ? neiSamples[(N-1)>>1][-1]: neiSamples[-1][(N-1)>>1]
dcValLT = horEdgeFlag ? neiSamples[-1][N-1]: neiSamples[N-1][-1]

If(DltFlag == 1)

For 0 <= x, y < N-1:
predDCVal = (partitionPattern[x][y] == partitionPattern[0][0]) ? dcValLT : dcValBR
dcOffset =dcOffsetAvailFlag ? DcOffset[xTb][yTb][partitionPattern[x][y]] : 0
predSamples[x][y] = Idx2DepthValue[DepthValue2Idx[predDCVal]+dcOffset]

Otherwise (DltFlag == 0)

For 0 <= x, y < N-1:
predDCVal = (partitionPattern[x][y] == partitionPattern[0][0]) ? dcValLT : dcValBR
dcOffset =dcOffsetAvailFlag ? DcOffset[xTb][yTb][partitionPattern[x][y]] : 0
predSamples[x][y] = predDCVal + dcOffset

where neiSamples are left and above reference samples and N is PU size.

- Segmental depth intra coding is ON: 
DLT ON: N2 addictions, 4+3N2 LUT access
DLT OFF: N2 additions, N2 LUT access

If (DltFlag == 1)

dcPred[wedgePattern[0][0]]      = predSamples[0][0]
dcPred[wedgePattern[N-1][0]]    = predSamples[N-1][0]
dcPred[wedgePattern[0][N-1]]    = predSamples[0][N-1]
dcPred[wedgePattern[N-1][N-1]]  = predSamples[N-1][N-1]

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[wedgePattern[x][y]]]
dltIdxResi = DcOffset[xTb][yTb][wedgePattern[x][y]]
Recblock[x][y] = Idx2DepthValue[dltIdxPred + dltIdxResi]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][wedgePattern[x][y]]

- Segmental depth intra coding is OFF:
DLT ON: N2 additions, 2N2 LUT access
DLT OFF: Inverse transform

If (DltFlag == 1)

For 0<= x, y < N:
idx = DepthValue2Idx[predSamples[x][y]] + resSamples[x][y]
Recblock[xTb+x][yTb+y] = Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] − 1, idx ) ]

Otherwise (DltFlag == 0)

Inverse transform is applied.

where xTb, yTb are the top-left sample position of current block and Recblock is the reconstructed block.

The number of operations of DMM 1 with different configurations for one PU is recorded in the algebraic way in Table II and the attached spreadsheet illustrated the numeric results in detail.

Table II Number of operations of one PU

| Process |  | Add/Sub |  | LUT access |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DLT |  |  |  |
|  |  | ON | OFF | ON | OFF |
| Derive and assign depth partition value |  | 3+N2 | 3+N2 | 3+3N2 | 3+N2 |
| Segmental depth intra coding | ON | N2 | N2 | 4+3N2 | N2 |
|  | OFF | N2 | N/A | 2N2 | N/A |

### Storage requirement

Input buffer of DMM 1 should store wedgelet pattern, wedgelet pattern index, above and left reference samples, dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of DMM 1, some intermediate data should be stored, such as predicted DC (dcValXX and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table III and the numeric result for different PU size is illustrated in Table IV. (The result in table is the case of that SDC and DLF both are ON.)

Table III Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Wedgelet pattern | N×N×1 |
| Wedgelet pattern index | 11 |
| Above and left reference samples | 8×8 |
| Predicted DC | 2×8 |
| dcOffset | 2×2×8 |
| Reconstructed block | N×N×8 |

Table IV Numeric expression of storage requirement at different PU size

| PU size | Total bits |
| --- | --- |
| 4×4 | 256 |
| 8×8 | 688 |
| 16×16 | 2,416 |
| 32×32 | 9,328 |
| 64×64 | 37,072 |

### Data transfer rate

Input data and output transferred between DMM 1 and other modules would be taken into consideration. Hence, wedgelet pattern, wedgelet pattern index, above and left reference samples, dcOffset, and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table V and the numeric result for different PU size is illustrated in Table VI. (The result in table is the case of that SDC and DLF both are ON.)

Table V Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Wedgelet pattern (Input) | N×N×1 |
| Wedgelet pattern index (Input) | 11 |
| Above and left reference samples (Input) | 8×8 |
| dcOffset (Input) | 2×2×8 |
| Reconstructed block (Output) | N×N×8 |

Table VI Numeric result of data transfer rate at different PU size

| PU size | Bits/PU |
| --- | --- |
| 4×4 | 240 |
| 8×8 | 672 |
| 16×16 | 2,400 |
| 32×32 | 9,312 |
| 64×64 | 37,056 |

The number of bits for one wedgelet pattern is from 16 bits to 1,024 bits depending PU size and then one CTU (64×64) can have 256 to 4 table indices depending on PU size. Therefore possible total fetch account for 4,096 bits for one CTU. Compared with 32×32 block, the fetching size corresponds to four 32×32 block. In the case of four 32×32 bi-pred PU in CTU, the fetching size is 4×39×39×1.5×2×8 bits (=146,016). So the amount of data transfer on wedgelet pattern in DMM 1 (32×32×4) can be equivalent to 2.8% of the reference pixels in motion compensation. Consequently, it is relatively small as compared to motion compensation process.

## DMM 4

### Algorithmic description

According to the specification text and the test model, the flowchart of DMM 4 is shown in Fig. 2. The major processing of DMM 4 is the processes including contour pattern generation, derivation and assignment of depth partition value and segmental depth intra coding. According to flowchart, the complexity of DMM 4 is assessed at various data granularities and the details are described in the following sections.

Fig. 2 Flowchart of DMM 4

### Number of operations

Due to the fact that the part of deriving and assigning depth partition value of DMM 4 is identical to DMM 1, this section would only display the process which derives contour pattern via reconstructed Luma picture and block reconstruction. However, the numeric result includes all procedures of DMM 4.

- Contour pattern generation: N2 comparisons, N2 additions/subtractions

wedgePatterm[x][y] = ( refSamples[x][y] > ThresVal ), for 0<= x, y < N

where refSamples is the co-located reconstructed Luma picture, and N is block size.

- Segmental depth intra coding is ON: 
DLT ON: N2 addictions, 4N2 LUT access
DLT OFF: N2 additions, N2 LUT access

If (DltFlag == 1)

dcPred[wedgePattern[0][0]] = predSamples[0][0]
foundFlag = 0

for ( x = 0; x < N; x++ )
for ( y = 0; y < N; y++ )	
  if ( foundFlag = = 0 && wedgePattern[x][y] != wedgePattern[0][0]) {
      dcPred[wedgePattern[x][y]] = predSamples[x][y]
      foundFlag= 1
  }

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[wedgePattern[x][y]]]
dltIdxResi = DcOffset[xTb][yTb][wedgePattern[x][y]]
Recblock[x][y] = Idx2DepthValue[dltIdxPred + dltIdxResi]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][wedgePattern[x][y]]

- Segmental depth intra coding is OFF:
DLT ON: N2 additions, 2N2 LUT access
DLT OFF: Inverse transform

If (DltFlag == 1)

For 0<= x, y < N:
idx = DepthValue2Idx[ predSamples[ x ][ y ] ] + resSamples[ x ][ y ]
Recblock[ xTb + x ][ yTb + y ] = Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] − 1, idx ) ]

Otherwise (DltFlag == 0)

Inverse transform is applied.

where xTb, yTb are the top-left sample position of current block and Recblock is the reconstructed block.

The number of operations of DMM 4 with different configurations for one PU is recorded in the algebraic way in Table VIIError: Reference source not found and the attached spreadsheet records the numeric results of all combination in detail.

Table VII Number of operations of DMM 4 in terms of PU size = N

| Process |  | Add/Sub |  | LUT access |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DLT |  |  |  |
|  |  | ON | OFF | ON | OFF |
| Contour pattern generation |  | N2+N2 |  | 0 |  |
| Derive and assign depth partition value |  | 3+N2 | 3+N2 | 3+3N2 | 3+N2 |
| Segmental depth intra coding | ON | N2 | N2 | 4N2 | N2 |
|  | OFF | N2 | N/A | 2N2 | N/A |

### Storage requirement

Input buffer of DMM 4 should store contour pattern, above and left reference samples, co-located Luma samples, dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of DMM 4, some intermediate data should be stored, such as predicted DC (dcValLT, dcValBR and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table VIII and the numeric result for different PU size is illustrated in Table IX. (The result in table is the case of that SDC and DLF both are ON.)

Table VIII Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Reconstructed Luma samples | N×N×8 |
| Contour pattern | N×N×1 |
| Above and left reference samples | 8×8 |
| Predicted DC | 2×8 |
| dcOffset | 2×2×8 |
| Reconstructed block | N×N×8 |

Table IX Numeric expression of storage requirement at different PU size

| PU size | Total bits |
| --- | --- |
| 4×4 | 384 |
| 8×8 | 1,200 |
| 16×16 | 4,464 |
| 32×32 | 17,520 |
| 64×64 | 69,840 |

### Data transfer rate

Input data and output transferred between DMM 4 and other modules would be taken into consideration. Hence, above and left reference samples, co-located Luma samples, dcOffset, and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table X and the numeric result for different PU size is illustrated in Table XI. (The result in table is the case of that SDC and DLF both are ON.)

Table X Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Reconstructed Luma samples (Input) | N×N×8 |
| Above and left reference samples (Input) | 8×8 |
| dcOffset (Input) | 2×2×8 |
| Reconstructed block (Output) | N×N×8 |

Table XI Numeric result of data transfer rate per PU at different PU size

| PU size | Bits/PU |
| --- | --- |
| 4×4 | 352 |
| 8×8 | 1,120 |
| 16×16 | 4,192 |
| 32×32 | 16,480 |
| 64×64 | 65,728 |

## Conventional intra prediction (Angular mode)

### Number of operations:

- Angular intra prediction: 4N2 (4N2+2N) addition/subtractions, 4N2 multiplications

For the case of predModeIntra <= 18, the other case is similar.

If (intraPredAngle < 0)
  If (( N * intraPredAngle )  >>  5 < −1)
	ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ], for x = −1..( N * intraPredAngle )  >>  5
Otherwise, 
  ref[ x ] = p[ −1 + x ][ −1 ], with x = N + 1..2 * N

For 0 <= x, y < N-1:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5
iFact = ( ( y + 1 ) * intraPredAngle ) & 31

If (iFact != 0)
  predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5
Otherwise (iFact == 0)
  predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]

If (predModeIntra == 26 (Vertical mode) && cIdx == 0 && N < 32)
For 0 <= x < N-1 and y= 0:
  predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Segmental depth intra coding is ON: 
DLT ON: 3+3N2 addictions, 2N2 LUT access
DLT OFF: N2 a dditions

If (DltFlag == 1)

dcPred[0] = ( predSamples[0][0] + predSamples[0][N − 1] + 
predSamples[N − 1][ 0 ] + predSamples[N − 1][N − 1] + 2 )  >>  2

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[0]]
dltIdxResi = DcOffset[xTb][yTb][0]
Recblock[x][y] = predSamples[ x ][ y ] + Idx2DepthValue[dltIdxPred + dltIdxResi] − dcPred[0]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][0]

where xTb, yTb are the top-left sample position of current block and Recblock is the reconstructed block.

The number of operations of angular mode with different configurations for one PU is recorded in the algebraic way in Table XII and the attached spreadsheet illustrated the numeric results in detail.

Table XII Number of operations of one PU

| Process |  | Mul. |  | Add/Sub |  | LUT access |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DLT |  |  |  |  |  |
|  |  | ON | OFF | ON | OFF | ON | OFF |
| Angular intra prediction |  | 4N2 |  | 4N2 (6N2) |  | 0 |  |
| Segmental depth intra coding | ON | 0 |  | 3+N2 | N2 | 2N2 | 0 |
|  | OFF | N/A |  | N/A |  | N/A |  |

### Storage requirement

Input buffer of angular mode should stores above and left reference samples and dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of angular mode, some intermediate data should be stored, such as predicted DC (predDCVal and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table VIII and the numeric result for different PU size is illustrated in Table IX. (The result in table is the case of that SDC and DLF both are ON.)

Table XIII Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Above and left reference samples | 2N+2N (2N+N for 64×64 block) |
| Predicted DC | 1×8 |
| dcOffset | 1×8 |
| Reconstructed block | N×N×8 |

Table XIV Numeric expression of storage requirement at different PU size

| PU size | Total bits |
| --- | --- |
| 4×4 | 272 |
| 8×8 | 784 |
| 16×16 | 2,576 |
| 32×32 | 9,232 |
| 64×64 | 34,344 |

### Data transfer rate

Input data and output transferred between angular mode and other modules would be taken into consideration. Hence, above and left reference samples, dcOffset and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table X and the numeric result for different PU size is illustrated in Table XI. (The result in table is the case of that SDC and DLF both are ON.)

Table XV Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Above and left reference samples (Input) | 2N+2N (2N+N for 64×64 block) |
| dcOffset (Input) | 1×8 |
| Reconstructed block (Output) | N×N×8 |

Table XVI Numeric result of data transfer rate per PU at different PU size

| PU size | Bits/PU |
| --- | --- |
| 4×4 | 264 |
| 8×8 | 776 |
| 16×16 | 2,568 |
| 32×32 | 9,224 |
| 64×64 | 34,848 |

## Overall Summary

According to the numeric analysis in the attached spreadsheet, the worst case is usually occurred at the case of that SDC and DLT are both ON. Hence, the following tables record the complexity of DMM 1, DMM 4, and angular mode on 6464 CTU level at various PU size. Table XVII, Table XVIII, and Table XIX show the numeric representations of required resource for one 6464 CTU with various PU size. Therefore, Table XX provides an overall report on depth intra modes’ complexity at 6464 CTU level. (Red letter indicates the worst case.)

Table XVII Number of operations of depth intra modes for one 6464 CTU

|  | DMM 1 |  | DMM 4 |  | Angular mode |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | Add/Sub | LUT access | Add/Sub/Comp | LUT access | Mul | Add/Sub | LUT access |
| 4×4 | 8,960 | 26,368 | 17,152 | 29,440 | 16,384 | 31,488 | 8,192 |
| 8×8 | 8,384 | 25,024 | 16,576 | 28,864 | 16,384 | 29,888 | 8,192 |
| 16×16 | 8,240 | 24,688 | 16,432 | 28,720 | 16,384 | 29,232 | 8,192 |
| 32×32 | 8,204 | 24,604 | 16,396 | 28,684 | 16,384 | 28,684 | 8,192 |
| 64×64 | 8,204 | 24,604 | 16,396 | 28,684 | 16,384 | 28,675 | 8,192 |

Table XVIII Storage requirement of depth intra modes for one 6464 CTU

| PU size | DMM 1 | DMM 4 | Angular mode |
| --- | --- | --- | --- |
| 4×4 | 256 | 384 | 272 |
| 8×8 | 688 | 1,200 | 784 |
| 16×16 | 2,416 | 4,464 | 2,576 |
| 32×32 | 9,328 | 17,520 | 9,232 |
| 64×64 | 37,072 | 69,840 | 34,344 |

Table XIX Data transfer rate of depth intra modes for one 6464 CTU. (bits/6464 CTU)

| PU size | DMM 1 | DMM 4 | Angular mode |
| --- | --- | --- | --- |
| 4×4 | 61,440 | 90,112 | 32,768 |
| 8×8 | 43,008 | 71,680 | 16,384 |
| 16×16 | 38,400 | 67,072 | 8,192 |
| 32×32 | 37,248 | 65,920 | 4,096 |
| 64×64 | 37,056 | 65,728 | 2,048 |

Table XX Worst case complexity of depth intra modes at 6464 CTU

| Complexity Metrics | DMM 1 | DMM 4 | Angular mode |
| --- | --- | --- | --- |
| Number of operations (Mul) | 0 | 0 | 16,384 |
| Number of operations (Add/Sub/Comp) | 8,960 | 17,152 | 31,488 |
| Number of operations (LUT access) | 26,368 | 29,440 | 8,192 |
| Storage requirement (Bits) | 37,072 | 69,840 | 34,344 |
| Data transfer rate (Bits per 6464 block) | 61,440 | 90,112 | 32,768 |

## Experimental results

The anchor software is HTM-10.0r1. This proposal also evaluated coding performance of without DMM 1 and 4 in HTM-10.0r1 and the test coding conditions include all intra and CTC. Table XXI and Table XXII show the results when DMM 1 and DMM 4 are both disable of all intra and CTC coding condition, respectively. Table XXIII and Table XXIV show the results when DMM 1 is disable of all intra and CTC coding condition, respectively. Table XXV and Table XXVI show the results when DMM 4 is disable of all intra and CTC coding condition, respectively.

Table XXI All intra configuration: coding results when DMM 1 and DMM 4 are disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.21% | 4.27% | 58.2% | 102.2% | 100.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.23% | 4.52% | 58.9% | 102.3% | 100.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.67% | 7.77% | 56.4% | 102.2% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.30% | 4.68% | 56.7% | 86.6% | 81.7% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.27% | 6.12% | 55.4% | 100.5% | 100.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.37% | 2.24% | 56.2% | 74.5% | 72.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 8.39% | 56.3% | 97.8% | 94.4% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.19% | 7.85% | 59.9% | 89.8% | 78.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.37% | 5.52% | 57.8% | 102.3% | 100.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.14% | 5.86% | 56.9% | 89.8% | 85.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.23% | 5.73% | 57.3% | 94.5% | 91.3% |

Table XXII CTC configuration: coding results when DMM 1 and DMM 4 are disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.1% | 0.07% | -0.18% | 1.55% | 58.2% | 102.2% | 100.7% |
| Kendo | 0.0% | 0.1% | -0.2% | -0.01% | -0.09% | 1.35% | 58.9% | 102.3% | 100.8% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.00% | -0.72% | 4.32% | 56.4% | 102.2% | 100.8% |
| GT_Fly | 0.0% | 1.0% | 0.6% | 0.16% | -0.34% | 0.81% | 56.7% | 86.6% | 81.7% |
| Poznan_Hall2 | 0.0% | 0.4% | 0.0% | 0.10% | -0.26% | 2.07% | 55.4% | 100.5% | 100.2% |
| Poznan_Street | 0.0% | 0.1% | 0.3% | 0.05% | -0.33% | 1.26% | 56.2% | 74.5% | 72.8% |
| Undo_Dancer | 0.0% | 1.0% | 0.9% | 0.26% | 0.41% | 3.95% | 56.3% | 97.8% | 94.4% |
| Shark | 0.0% | 1.4% | 1.3% | 0.26% | 0.29% | 3.84% | 59.9% | 89.8% | 78.8% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.02% | -0.33% | 2.40% | 57.8% | 102.3% | 100.7% |
| 1920x1088 | 0.0% | 0.8% | 0.6% | 0.17% | -0.04% | 2.39% | 56.9% | 89.8% | 85.6% |
| average | 0.0% | 0.5% | 0.4% | 0.11% | -0.15% | 2.39% | 57.3% | 94.5% | 91.3% |

Table XXIII All intra configuration: coding results when DMM 1 is disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.13% | 3.52% | 67.5% | 101.9% | 100.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.18% | 3.69% | 68.2% | 102.1% | 100.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.44% | 5.91% | 67.2% | 101.6% | 100.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.29% | 2.27% | 65.9% | 86.5% | 81.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.17% | 4.76% | 64.1% | 100.6% | 100.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.24% | 1.62% | 65.3% | 74.1% | 73.3% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 3.61% | 65.9% | 97.4% | 94.4% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.21% | 3.75% | 69.0% | 91.1% | 78.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.25% | 4.37% | 67.6% | 101.9% | 100.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.18% | 3.20% | 66.0% | 90.0% | 85.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.21% | 3.64% | 66.6% | 94.4% | 91.2% |

Table XXIV CTC configuration: coding results when DMM 1 is disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.4% | 0.1% | 0.08% | -0.10% | 1.39% | 96.6% | 99.9% | 103.9% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.00% | -0.11% | 1.13% | 97.2% | 118.6% | 114.4% |
| Newspaper_CC | 0.0% | -0.1% | -0.1% | -0.02% | -0.55% | 3.28% | 95.3% | 109.5% | 110.8% |
| GT_Fly | 0.0% | 0.5% | 0.3% | 0.09% | -0.11% | 0.81% | 95.3% | 100.2% | 100.1% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.3% | -0.05% | -0.30% | 1.52% | 94.7% | 100.4% | 99.9% |
| Poznan_Street | 0.0% | 0.2% | 0.2% | 0.06% | -0.24% | 0.99% | 95.2% | 100.1% | 100.0% |
| Undo_Dancer | 0.0% | 0.7% | 0.6% | 0.17% | 0.25% | 1.76% | 96.0% | 100.3% | 99.9% |
| Shark | 0.0% | 0.8% | 0.9% | 0.17% | 0.07% | 1.84% | 95.8% | 100.6% | 99.7% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.02% | -0.25% | 1.94% | 96.4% | 109.3% | 109.7% |
| 1920x1088 | 0.0% | 0.4% | 0.4% | 0.09% | -0.07% | 1.39% | 95.4% | 100.3% | 99.9% |
| average | 0.0% | 0.3% | 0.2% | 0.06% | -0.14% | 1.59% | 95.8% | 103.7% | 103.6% |

Table XXV All intra configuration: coding results when DMM 4 is disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.08% | 90.9% | 100.0% | 100.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.07% | 90.9% | 100.1% | 100.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.12% | 89.6% | 100.1% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.22% | 0.49% | 90.1% | 84.0% | 79.2% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.06% | 91.2% | 100.1% | 99.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.04% | 88.4% | 70.8% | 70.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.08% | 0.18% | 90.9% | 100.6% | 98.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.43% | 0.54% | 92.3% | 94.6% | 81.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.09% | 90.5% | 100.1% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.14% | 0.26% | 90.6% | 90.0% | 86.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.09% | 0.20% | 90.5% | 93.8% | 91.4% |

Table XXVI CTC configuration: coding results when DMM 4 is disable. (Anchor: HTM-10.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.0% | 0.05% | 0.01% | -0.01% | 98.5% | 98.6% | 100.1% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.00% | 0.00% | 0.03% | 99.4% | 113.8% | 113.0% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | 0.00% | -0.02% | 0.04% | 98.1% | 111.3% | 110.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | -0.02% | -0.06% | -0.28% | 98.5% | 98.5% | 100.2% |
| Poznan_Hall2 | 0.0% | 0.1% | -0.4% | -0.04% | -0.05% | -0.15% | 98.4% | 99.3% | 100.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.02% | 98.1% | 98.5% | 100.1% |
| Undo_Dancer | 0.0% | 0.1% | 0.1% | 0.02% | 0.06% | 0.06% | 98.4% | 98.2% | 99.8% |
| Shark | 0.0% | -0.1% | -0.1% | -0.01% | 0.13% | 0.25% | 98.6% | 98.4% | 99.7% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.01% | -0.01% | 0.02% | 98.7% | 107.9% | 108.0% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | -0.01% | 0.01% | -0.02% | 98.4% | 98.6% | 100.0% |
| average | 0.0% | 0.0% | -0.1% | 0.00% | 0.01% | 0.00% | 98.5% | 102.1% | 103.0% |

# Conclusion

From Table I to Table XX, the complexity of depth intra mode is assessed to assist the JCT3V team in evaluating the proposals related to depth intra modes. The results can be considered as anchor complexity. Thus, with the help of the intrinsic complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only the evaluated complexity of coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison. On the other hand, this proposal also suggest that the suitable maximum size of DMM 1 would be 3232 due to the fact that the storage size of wedgelet pattern for 6464 block is significantly increased.

# Reference

- JCT3V-G0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” San Jose, USA, 11 - 17 Jan. 2014.

- JCT3V-G0114, “AHG 10: Complexity Assessment on Illumination Compensation (IC),” San Jose, USA, 11 - 17 Jan. 2014.

- JCT3V-F0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-F0093, “3D-CE3.h: Results on simple merge candidate list construction for 3DV,” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-E0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-E0132, “AHG 10: Complexity Assessment on Simplified Depth Coding (SDC),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-D0010, “JCT-3V AHG Report: Complexity assessment (AHG10),“ Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,“ Incheon, KR, 20-26 April 2013.

- JCT3V-F1001, “3D-HEVC Draft Text 2,” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-F1005, “Test Model Description of 3D-HEVC and MV-HEVC,” Geneva, CH, 25 Oct. - 1 Nov. 2013.