| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0130 |
| --- | --- |

| Title: | 3D-CE2: Further improvements on advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Hongbin Liu Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the advanced residual prediction (ARP) is employed to code the residue of the current prediction unit in non-base texture views more efficiently. For each prediction unit (PU), when ARP is applied, the same motion information is shared within current PU. To further improve the coding efficiency of ARP, the current PU-level ARP is extended to block-level by splitting one prediction unit into several blocks to perform ARP separately. Therefore, each block may have different motion information. It is observed that there is a coding gain of 0.15% in terms of BD rate for coded texture views and the coding gain could be up to 1.1% for dependent views.

# Introduction

In 3D-HEVC, advanced residual prediction (ARP), as proposed in Error: Reference source not found, is enabled for non-base texture views. Error: Reference source not found illustrates the prediction structure of ARP in multiview video coding and 3D video coding for one reference picture list.

(a) Temporal ARP			(b) Inter-view ARP

Figure 1. Prediction structure of ARP in 3D-HEVC.

In ARP, three additional blocks in addition to current prediction unit (PU) are identified. They are defined as: temporal reference block in the current view (denoted by CurrRef), inter-view reference block (denoted by Base) within the picture of the same access unit as the current picture in base view and the reference block of Base (denoted by BaseRef) in the base view and an access unit different from the access unit containing the current picture. More specifically, the vectors utilized in identifying these blocks are listed in Table 1:

Table 1: Motion vectors utilized in ARP

|  | Temporal ARP | Inter-view ARP |
| --- | --- | --- |
| Base | DV from NBDV process (denoted by derivedMv) | Coded disparity motion vector of current PU (denoted by cMv) |
| CurrRef | Coded temporal motion vector of current PU (denoted by cMv) | Temporal motion vector of the block covering center position of Base (denoted by derivedMv) |
| BaseRef | cMv + derivedMv | cMv + derivedMv |

Note in Table 1, the temporal ARP indicates the case that current reference picture is a temporal reference picture while the inter-view ARP indicates the case that current reference picture is an inter-view reference picture. Error: Reference source not found depicts the prediction structure for both cases.

After the additional three blocks are identified, the residual is generated by current PU and its reference block (either CurrRef in temporal prediction or Base in inter-view prediction) and the rest two blocks are utilized to generate the residual predictor. In addition, a weighting factor w (being 0, 1 or 0.5) is further multiplied to the residual predictor and the weighted residual predictor is added to current temporal reference block to generate the final prediction of current block for one reference picture list.

In ARP process, only one vector, derivedMv (green solid arrow line in Error: Reference source not found) is obtained either from the coded disparity motion vectors of spatial/temporal neighboring blocks, as depicted in Error: Reference source not found (a), (i.e., NBDV result of current PU) or from the temporal motion vectors of the block covering the center position in the inter-view reference block, as depicted in Error: Reference source not found (b). These derived motion vectors may be not accurate enough, especially for larger PUs. Therefore, the current design of ARP may be further improved.

# Proposed Scheme

To solve the above problem, in this proposal, a block-level ARP method is proposed which has been proposed in JCT3V-G0121 [2].

When block-level ARP is enabled, each PU is firstly split into several blocks (8 x 8 block size is used in the simulations), and each block share the same motion information as current PU. However, the derivedMv (i.e., disparity vector in temporal ARP or temporal motion vector in inter-view ARP) could be updated for each 8x8 block.

For Temporal ARP, when current PU uses a sub-PU inter-view merge candidate, the default derivedMv (denoted by DV[ i ] for the i-th 8x8 block in Figure 2 (a)) is firstly set to be the DV from NBDV process, as in current ARP. When the block covering the center position of the i-th 8x8 block within CurrRef contains a disparity motion vector, DV[ i ] is updated to be that disparity motion vector. It is noted that for other merging candidates and AMVP mode, the block-level ARP is disabled and current ARP may be applied.

For Inter-view ARP, the default derivedMv (denoted by mvLX[ i ] for the i-th 8x8 block in Figure 2 (b)) is set to the temporal motion vector associated with a block covering the center position of Base, as in current ARP. When the block covering the center position of the i-th 8x8 block within Base contains a temporal motion vector, mvLX[ i ] is updated to be that temporal motion vector.

As illustrated above, for both cases, only the block level (8x8) motion information of the reference block located by current PU’s motion vector, either dotted line block CurrRef in Figure 2 (a) or Base in Figure 2 (b), is accessed to generate the final residual predictor.

As shown in Figure 2, in block-level temporal ARP, the blocks in the base view to be accessed may not be located within one WxW region, wherein WxW is the size of the current (2Nx2N) PU. However, in block-level inter-view ARP, the blocks in the base view to be accessed are located within one WxW region. Therefore block-level inter-view ARP is of lower complexity than block-level temporal ARP.

On the other hand, the proposed block-level inter-view ARP has the same complexity as the sub-PU inter-view motion prediction with temporal ARP enabled.

(a)	Temporal ARP				(b) Inter-view ARP

Figure 2. Prediction structure of block-level ARP.

# Experimental Results

This section provides simulation results of the proposed ARP improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 10.0r1 [3] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [4].

Two sets of simulation results are provided wherein the whole proposal (Test #1) and the block-level inter-view ARP (Test #2) are evaluated separately. The coding performance of the proposed block-level ARP is given in Table 2. As shown in Table 2, the overall average bitrate reduction is 0.15%, 0.15% and 0.14% for the decoded texture views, coded and synthesized views. When block-level ARP is only applied to Inter-view ARP, the coding gain compared to the HTM anchor is 0.1%, as shown in Table 3.

Table 2: Coding gain with respect to anchor for 3-view case (Test #1): block-level ARP for both Temporal and Inter-view ARP

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.64% | -0.67% | -0.22% | -0.21% | -0.24% | 102% | 103% | 105% |
| Kendo | -0.43% | -0.47% | -0.19% | -0.21% | -0.17% | 95% | 97% | 95% |
| Newspapercc | -0.27% | -0.39% | -0.10% | -0.12% | -0.12% | 105% | 100% | 104% |
| GhostTownFly | -0.16% | -0.27% | -0.05% | -0.04% | 0.02% | 116% | 115% | 116% |
| PoznanHall2 | -0.79% | -1.09% | -0.32% | -0.33% | -0.35% | 106% | 107% | 107% |
| PoznanStreet | -0.39% | -0.27% | -0.08% | -0.08% | -0.07% | 107% | 112% | 110% |
| UndoDancer | -0.44% | -0.60% | -0.12% | -0.12% | -0.12% | 104% | 104% | 104% |
| Shark | -0.53% | -0.48% | -0.09% | -0.08% | -0.10% | 107% | 105% | 105% |
| 1024x768 | -0.45% | -0.51% | -0.17% | -0.18% | -0.18% | 101% | 100% | 101% |
| 1920x1088 | -0.46% | -0.54% | -0.13% | -0.13% | -0.12% | 108% | 109% | 108% |
| average | -0.45% | -0.53% | -0.15% | -0.15% | -0.14% | 105% | 105% | 106% |

Table 3: Coding gain with respect to anchor for 3-view case (Test #2): block-level ARP for Inter-view ARP

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.26% | -0.43% | -0.11% | -0.10% | -0.13% | 107% | 101% | 109% |
| Kendo | -0.23% | -0.17% | -0.11% | -0.13% | -0.13% | 104% | 95% | 102% |
| Newspapercc | -0.14% | -0.16% | -0.04% | -0.06% | -0.03% | 104% | 95% | 101% |
| GhostTownFly | -0.04% | -0.13% | -0.03% | -0.02% | -0.02% | 98% | 94% | 91% |
| PoznanHall2 | -0.48% | -0.78% | -0.22% | -0.22% | -0.24% | 98% | 96% | 100% |
| PoznanStreet | -0.32% | -0.05% | -0.05% | -0.05% | -0.06% | 103% | 100% | 101% |
| UndoDancer | -0.38% | -0.59% | -0.10% | -0.10% | -0.10% | 104% | 96% | 100% |
| Shark | -0.31% | -0.24% | -0.06% | -0.04% | -0.06% | 93% | 85% | 87% |
| 1024x768 | -0.21% | -0.25% | -0.09% | -0.10% | -0.10% | 105% | 97% | 104% |
| 1920x1088 | -0.30% | -0.36% | -0.09% | -0.09% | -0.10% | 99% | 94% | 96% |
| average | -0.27% | -0.32% | -0.09% | -0.09% | -0.10% | 101% | 95% | 99% |

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San José, US, 11–17 Jan. 2014.

- L. Zhang, Y. Chen, M. Karczewicz, Q. Yu, S. Ma, “CE4: Further improvements on advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0121, 7th Meeting: San José,  US, 11–17 Jan. 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José,  US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).