| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0131 |
| --- | --- |

| Title: | 3D-CE3: Delta DC coding for SDC and DMM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qin Yu1 Li Zhang2 Hongbin Liu2 Ying Chen2 Siwei Ma1 1School of EECS, Peking University, Institute of Digital Media, Beijing, China 25775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Peking University and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, when Segment-wise DC Coding (SDC) or Depth Modeling Mode (DMM) is enabled, residual is coded with syntax elements depth_dc_abs and depth_dc_sign_flag. depth_dc_abs may need to be coded with a high number of context coded bins (in theory up to 13). Although HEVC version 1 allows a larger number of context coded bins for coefficients, it is preferred to have less context coded bins without coding efficiency drop. To improve the throughput of entropy coding, the number of context coded bins should be reduced. The proposal provides results of the coding of depth_dc_abs with 9 context coded bins. Furthermore, in this proposal, we reduce the number of context coded bins to 3. Experimental results show that 0.1% BD rate difference is observed for the proposed method.

# Introduction

To preserve the depth’s characteristics, new modes including Segment-wise DC Coding (SDC) and Depth Modeling Mode (DMM) are introduced into intra mode coding. Meanwhile, SDC is also extended to inter mode coding (inter SDC).

For these modes, one or two DC residual values (delta DC) need to be coded and transmitted to the decoder. The syntax elements used for representing DC residual are shown in Table 1. The absolute value of the DC residual is signaled as depth_dc_abs which is in the range [0, 255].

Table 1: Syntax elements of DC residual for SDC/DMM

| if( depth_dc_flag[ x0 + k ][ y0 + j ] ) |  |
| --- | --- |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ] > 0 ) |  |
| depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| } |  |

## Number of bins coded with context in 3D-HEVC

Table 2 shows the binarization of syntax element depth_dc_abs. Binarized depth_dc_abs is a concatenation of a prefix bin string (Truncated Rice, TR with cRiceParam = 0 and cMax = 13) and (when present) a suffix bin string (0-th order Exp-Golomb code, i.e., EG0 code). The prefix bin string is coded with context model and the suffix bin string is bypass coded. From Table 2, we can see that total number of bins to be coded can be up to 28 (e.g., when value is equal to 255) for the worst case, and there are at most 13 bins (e.g., when value is equal to or larger than 12) that are coded with context model and the remaining ones are bypass coded.

Therefore, the delta DC coding will account for a high percentage of the overall context coded bins in this case.

Table 2: binarization of depth_dc_abs

| Value | Prefix bin string (Truncated Unary code) |  |  |  |  |  |  |  |  |  |  |  |  | suffix bin string (EG0 code) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
| 13 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 14 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 100 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 101 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 111111101110011 |
| BinIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | … 27 |

## Number of bins coded with context in HEVC version 1

In subclause 9.3.4.2.1 of HEVC version 1 specification [1], Table 9-37 shows the syntax coding of HEVC CU unit. From that table, we can see that last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are the syntax elements which have the longest context coded bins. The number of context coded bins is 9.

However, for depth_dc_abs and inter_sdc_resi_abs_minus1, the maximum number of context coded bins is 13. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. They are the bottle neck when doing parallel CABAC. To improve the throughput of the CABAC, the number of context coded bins should be reduced to a level similar to HEVC. Moreover, hardware is designed for the worst case, it is also important to reduce the worst case number of the context coded bins.

## Number of bins coded in HEVC version 1

By analyzing HEVC version 1 specification [1], we noticed that the syntax element abs_mvd_minus2 may have as many as 30 bins to be coded, although all of them are bypass coded.

- Proposal

Two methods are provided in this contribution. The first one is exactly the same as method #1 described in [2] wherein depth_dc_abs is binarized with prefix (using TR with cRiceParam = 0, cMax = 13) and suffix (if present, using EG0). All bins of the prefix are coded with the same context model. For the worst case, total 24 bins need to be decoded with 9 of them with context coding.

To further improve the throughput of CABAC, another simplified method is proposed wherein depth_dc_abs is binarized into a concatenation of a prefix bin string (truncated unary code) and (when present) a suffix bin string (0-th order Exp-Golomb code). As shown in Table 3, for value x (of depth_dc_abs) equal to or larger than 3, the prefix bin string is consisted of three ‘1’, and the suffix bin string is the binarized (x - 3) using 0-th order Exp-Golomb code; while for value x smaller than 3, the prefix bin string is the binarized x using unary code and the suffix bin string is not present. The prefix bins (up to 3) are coded with context model, while the remaining bins are bypass coded. For the worst case, total 18 bins need to be decoded with 3 of them with context coding.

Table 3: Binarization of depth_dc_abs

| Value | prefix bin string ( truncated unary code) |  |  | suffix bin string (EG0 code) |
| --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |
| 1 | 1 | 0 |  |  |
| 2 | 1 | 1 | 0 |  |
| 3 | 1 | 1 | 1 | 0 |
| 4 | 1 | 1 | 1 | 100 |
| 5 | 1 | 1 | 1 | 101 |
| … |  |  |  | … |
| 255 | 1 | 1 | 1 | 111111101111101 |
| BinIdx | 0 | 1 | 2 | … 17 |

# Experimental results

The proposed methods have been implemented into 3D-HTM v10.0r1 [3]. Simulations were conducted under common test conditions (CTC) [4].

Table 4 and Table 5 show the performance the second proposed method (i.e., TR with cMax equal to 3 for prefix) under common test conditions and all intra case, respectively. As shown in Table 4, the proposed method could achieve minor coding gain, 0.1% in aspect of synthesis view/ total bitrate for CTC. While for the all intra case, the proposed method shows no coding loss.

Table 4: Coding performance of method #2 with respect to 3D-HTM anchor (CTC)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 102% | 105% | 104% |
| Kendo | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 95% | 95% | 95% |
| Newspaper_CC | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 103% | 99% | 103% |
| GT_Fly | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 107% | 114% | 108% |
| Poznan_Hall2 | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | 91% | 97% | 99% |
| Poznan_Street | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 104% | 105% |
| Undo_Dancer | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 98% | 106% | 101% |
| Shark | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | 95% | 96% | 93% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100% | 100% | 101% |
| 1920x1088 | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 99% | 103% | 101% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99% | 102% | 101% |

Table 5: Coding performance of method #2 with respect to 3D-HTM anchor (all intra)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 99% | 100% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 94% | 94% | 95% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102% | 100% | 102% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105% | 109% | 108% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97% | 97% | 95% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 99% | 99% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 109% | 108% | 109% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 114% | 112% | 115% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99% | 98% | 99% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 106% | 105% | 105% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 102% | 103% |

Table 6 and Table 7 show the performance of the first proposed method in [2]. As shown in Table 5, the proposed method could achieve similar performance as the 3D-HTM anchor.

Table 6: Coding performance of method #1 [2] with respect to 3D-HTM anchor (CTC)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.3% | -0.1% | 0.1% | 0.0% | 0.0% | 111% | 104% | 113% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97% | 90% | 97% |
| Newspaper_CC | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 112% | 101% | 108% |
| GT_Fly | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 112% | 109% | 113% |
| Poznan_Hall2 | 0.1% | -0.4% | 0.0% | 0.0% | -0.1% | 99% | 99% | 100% |
| Poznan_Street | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 106% | 101% | 109% |
| Undo_Dancer | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 108% | 104% | 106% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 104% | 98% | 101% |
| 1024x768 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 107% | 99% | 106% |
| 1920x1088 | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 106% | 102% | 106% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 106% | 101% | 106% |

Table 7: Coding performance of method #1 [2] with respect to 3D-HTM anchor (all intra)

(to be provided)

# References

- B. Bross, W. Han, J. Ohm, G. Sullivan, Y. Wang and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-L1003, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

- Q. Yu, H. Liu, Y. Chen, L. Zhang and S. Ma, “CE5 related: Delta DC coding for SDC and DMM modes,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0124, 7th Meeting: San José, US, 11- 17 Jan. 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11- 17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of Peking University having any current or pending patent rights relating to the technology described in this contribution.