| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0134 |
| --- | --- |

| Title: | Simplifications for disparity derived depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qin Yu1 Ying Chen2 Hongbin Liu2 Siwei Ma1 1School of EECS, Peking University, Institute of Digital Media, Beijing, China 25775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Peking University and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes simplifications to Disparity Derived Depth (DDD) coding. First, look-up table used for mapping disparity vector to depth is simplified. Second, disparity vector of a texture block that is already accessed by other merge candidate is used to derive depth value for DDD. Third, DDD applies only for partition mode 2Nx2N. It is reported that proposed method has no coding performance loss.

# Problem

In last meeting, Disparity Derived Depth (DDD) coding [1] was adopted into 3D-HEVC. To generate a DDD candidate, all pixels within the depth PU are predicted by one single depth value. DDD candidate is inserted after the sub-PU MPI candidate.

The single depth value (denoted by dDep) is derived from the disparity motion vector, if available, (denoted by texDv) of the co-located texture block of the current depth PU. Specifically, denote the size of current PU by nPSW x nPSH and denote coordination of its top left sample relative to the top left sample of the depth picture by (xP, yP), the block in the corresponding texture picture that covers (xP + nPSW/2, yP + nPSH/2) is used as the reference texture block for current depth PU.

Denote j as the view index of the reference view that texDv pointing to and denote texDvx as the horizontal component of texDv, then dDep is converted from texDvx as follows.

  - dDep = Clip3( 0, ( 1  <<  bitDepthY ) − 1, ( ( DispToDepthInvScale[ j ] * texDvx + 		(1)
	DispToDepthInvOffset[ j ] ) >> DispToDepthInvShift[ j ] ) )

Here, cp_scale and cp_offset are camera parameters coded at video parameter set or slice segment header, and BitDepthY is the bit depth of depth pixel, and the variables DispToDepthInvScale[ j ], DispToDepthInvOffset[ j ] and DispToDepthInvShift[ j ] are derived for each slice as follows.

absCpScale = Abs( cp_scale[ j ] )
	signCpScale = Sign( cp_scale[ j ] )
	convShift = Ceil( Log2( cp_scale[ j ] ) ) + 9
	convPrec = ( 1 << convShift )
	for( d = 256, dMinError = 256, absMinError  = Abs( convPrec − absCpScale *256 ) ; d < 512; d++ ) {
		absCurrErr = Abs( convPrec − absCpScale * d )
		if( absCurrErr < absMinError ) {	(Error: Reference source not found-1)
			absMinError = absCurrErr
			dMinError = d
		}
	}
	signMinError = Sign( convPrec − absCpScale * dMinError )
	DispToDepthInvShift[ j ] = convShift
	DispToDepthInvScale[ j ] = signCpScale * ( dMinError  <<  ( BitDepthY + cp_precision − 1 ) )
	DispToDepthInvOffset[ j ] = −1 * signCpScale * dMinError * ( cp_off[ j ]  <<  BitDepthY ) + 
								(1  <<  ( convShift − 1 ) ) + signMinError * ( 1  <<  ( convShift − 4 ) )

There are several issues on DDD in terms of decoder complexity.

- The DDD mode is considered as complex, because it requires multiplications for disparity vector to depth value conversion in both slice and PU level.

At slice level, multiplications are required for deriving variables DispToDepthInvScale[ j ], DispToDepthInvOffset[ j ] and DispToDepthInvShift[ j ] as described above. At PU level, multiplication is required in conversion from disparity vector to depth value as illustrated in Eq. (1).

Multiplications at PU level can be avoided by building a look-up table (denoted by DispToDepthB) to convert disparity vector to depth value at slice level. Before coding a slice, for each valid disparity vector (may range from -215 to 215 – 1), it is converted to depth value as follows and the depth value is stored in the look-up table.

for( d = -215; d != 215; ++d ) {
			DispToDepthB[ j ][ d ]  = Clip3( 0, ( 1  <<  bitDepthY ) − 1, ( ( DispToDepthInvScale[ j ] * d + 	(2)
									DispToDepthInvOffset[ j ] ) >> DispToDepthInvShift[ j ] ) )
		}

Then, for DDD coded PU, the disparity vector can be converted to depth value by a table look-up operation.

- DDD candidate may require to access motion information of an additional texture block that is not accessed for the generation of the sub-PU MPI candidate for the same current PU.

In 3D-HEVC, motion information of texture blocks covering the center pixel of each sub-PU is accessed in sub-PU based MPI candidate [2]. While in DDD, motion information of the texture block covering center pixel of the PU is accessed.

An example is illustrated in Figure 1 to show the texture blocks accessed by sub-PU MPI and DDD. Motion information of the 8x8 texture block that covers red pixel is accessed by DDD, and motion information of the 8x8 texture block that covers black pixel is accessed by sub-PU MPI. When sub-PU size is equal to 8x8 as shown Figure 1 (a), motion information of the 8x8 texture block accessed by DDD is also accessed by sub-PU MPI. However, when sub-PU size is equal to 16x16 (or larger size) as shown in Figure 1 (b), motion information of the 8x8 texture block accessed by DDD is not accessed by sub-PU MPI, therefore an additional motion information access is required by DDD.

Figure 1. Texture blocks accessed by DDD and sub-PU MPI

- DDD mode applies for all partition modes but it can only improve coding performance of partition mode 2Nx2N.

DDD candidate is always inserted to the merge list of depth PU. This increases complexity of depth merge list construction, especially for small PU size such as 8x4 or 4x8 PU.

# Proposal

To address above two issues, two aspects are proposed in this contribution.

## Removal of multiplication in DDD

### Proposed method

To remove the multiplication in DDD, a look-up table is built to map disparity vector with integer accuracy to depth value (denoted by DispToDepth). DispToDepth is constructed as an inverse table of DepthToDisp which maps depth value to disparity vector in 3D-HEVC. Meanwhile, disparity vectors are constrained to be in the range [–picW, picW], where picW is the picture width.

- Denote DispMin and DispMax as the minimum and maximum disparity vector in table DepthToDisp, and denote their corresponding depth value by DepthDispMin and DepthDispMax respectively. Then, for a disparity smaller than DispMin, its corresponding depth value in table DispToDepth is set equal to DepthDispMin; and for a disparity greater than DispMax, its corresponding depth value in table DispToDepth is set equal to DepthDispMax.

- Denote two consecutive elements contained in table DepthToDisp by Disp0 and Disp1 (Disp0 and Disp1 are converted to integer accuracy before used), and denote their corresponding depth value by Depth0 and Depth1 respectively. To calculate depth values for disparity vectors between Disp0 and Disp1, following rule applies:

  - If Disp0 is less than Disp1, then for each element in the range [Disp0, Disp1-1], its corresponding depth value in table DispToDepth is set equal to Depth0. An example is shown in Figure 2.

Figure 2. Depth value derivation process.

  - If Disp0 is greater than Disp1, then for each element in the range [Disp1, Disp0-1], its corresponding depth value in table DispToDepth is set equal to Depth1.

The proposed working draft text in 3D-HEVC for constructing DepthToDisp is as follows:

- incFlag = ( DepthToDisparityB[ j ][ maxDepthVal ] > DepthToDisparityB[ j ][ 0 ] ) ? 1 : 0
minDisp[ j ] = incFlag ? DepthToDisparityB[ j ][ 0 ] : DepthToDisparityB[ j ][ maxDepthVal ]
maxDisp[ j ] = !incFlag ? DepthToDisparityB[ j ][ 0 ] : DepthToDisparityB[ j ][ maxDepthVal ]
minDisp[ j ] = ( minDisp[ j ] + 2 )  >>  2
maxDisp[ j ] = ( maxDisp[ j ] + 2 )  >>  2
minDisp[ j ] = Clip3( -pic_width_in_luma_samples, pic_width_in_luma_samples, minDisp[ j ] )
maxDisp[ j ] = Clip3( -pic_width_in_luma_samples, pic_width_in_luma_samples, maxDisp[ j ] )
numDispMins1 = maxDisp[ j ] – minDisp[ j ]
DispToDepthB[ j ][ 0 ] = incFlag ? 0 : maxDepthVal
DispToDepthB[ j ][ numDispMins1 ] = !incFlag ? 0 : maxDepthVal
dispCur = DepthToDisparityB[ j ][ 0 ]
dispCurInt = ( DepthToDisparityB[ j ][ 0 ] + 2 )  >>  2
step = incFlag ? 1 : -1
for( d = 1; d != maxDepthVal; ++d ) {
		if( ( incFlag  &&  ( dispCur < DepthToDisparityB[ j ][ d ] ) )  | |
			( !incFlag && ( dispCur > DepthToDisparityB[ j ][ d ] ) ) ) {
			dInt = ( ( DepthToDisparityB[ j ][ d ] + 2 )  >>  2 )
			numInsert = dInt – dispCurInt
			if( numInsert != 0 )
				numInsert = numInsert – step
			idxCur = dispCurInt – minDisp[ j ]
			while( numInsert  !=  0 ) {
				DispToDepthB[ j ][ idxCur+ numInsert ] = DispToDepthB[ j ][ idxCur ]
				numInsert = numInsert – step
			}
			dispCur = dInt
			DispToDepthB[ j ][ dispCur – minDisp[ j ] ] = d
		}
}

When converting disparity vector to depth value, the disparity vector is first converted to integer accuracy and clipped to [–picW, picW], and then is used as index to grab depth value from table DispToDepthB, as in the follows.

texDvx = Clip3( -picW, picW, ( ( texDvx + 2 )  >>  2 ) ) 			(3)

dDep = DispToDepthB[j][ texDvx]						(4)

Alternatively, the look-up table DispToDepthB can be built for disparity vector with quarter pixel accuracy, and the disparity vector range is not constrained, i.e., it can be any value in the range [–215, 215 - 1]. In this case, the proposed working draft is as follows:

- incFlag = ( DepthToDisparityB[ j ][ maxDepthVal ] > DepthToDisparityB[ j ][ 0 ] ) ? 1 : 0
minDisp[ j ] = incFlag ? DepthToDisparityB[ j ][ 0 ] : DepthToDisparityB[ j ][ maxDepthVal ]
maxDisp[ j ] = !incFlag ? DepthToDisparityB[ j ][ 0 ] : DepthToDisparityB[ j ][ maxDepthVal ]
numDispMins1 = maxDisp[ j ] – minDisp[ j ]
DispToDepthB[ j ][ 0 ] = incFlag ? 0 : maxDepthVal
DispToDepthB[ j ][ numDispMins1 ] = !incFlag ? 0 : maxDepthVal
dispCur = DepthToDisparityB[ j ][ 0 ]
step = incFlag ? 1 : -1
for( d = 1; d != maxDepthVal; ++d ) {
		if( ( incFlag  &&  ( dispCur < DepthToDisparityB[ j ][ d ] ) )  | |
			( !incFlag && ( dispCur > DepthToDisparityB[ j ][ d ] ) ) ) {
			numInsert = DepthToDisparityB[ j ][ d ] – dispCur – step
			idxCur = dispCur – minDisp[ j ]
			while( numInsert  !=  0 ) {
				DispToDepthB[ j ][ idxCur+ numInsert ] = DispToDepthB[ j ][ idxCur ]
				numInsert = numInsert – step
			}
			dispCur = DepthToDisparityB[ j ][ d ]
			DispToDepthB[ j ][ dispCur – minDisp[ j ] ] = d
		}
}

When converting disparity vector to depth value, the disparity vector is used as index to grab depth value from DispToDepthB directly.

### Complexity Analysis

Based on above working draft text, worst case of complexity for DDD and proposed method is compared in Table 1, in which multiplication, addition, comparison, memory set (set continues memory to one same value) and table look-up are denoted by Mul, Add, Comp, Mem. set and LU respectively. As can be seen, for the anchor (DDD in current 3D-HEVC), it requires multiplication, addition, shift and comparison operations at slice level. In addition, it requires multiplication operations at the PU level.

Slice level operations of DDD are counted for the derivation process of DispToDepthInvScale[ j ], DispToDepthInvOffset[ j ] and DispToDepthInvShift[ j ], as described in Section 1. PU level operations are counted for the conversion from disparity vector to depth value in Eq. (1). An example is given for picture resolution 1920x1088. In worst case scenario, all PUs are coded with size 4x8 or 8x4 and all PUs are code with DDD mode. In this case, there are 65280 PUs and, the total PU level multiplication, addition, shift and comparison operation are 65280, 65280, 65280 and 130560 respectively.

As described in Section 1, PU level multiplication in DDD can be shifted to slice level by building a look-up table to map each disparity vector to depth value. In this case, besides the operations in the derivation process of DispToDepthInvScale[ j ], DispToDepthInvOffset[ j ] and DispToDepthInvShift[ j ], operations in Eq. (2) should be counted for all allowable disparity vectors. For fair comparison, the look-up table is also built for disparity vector with integer accuracy and the disparity vector is constrained to be in the range [–picW, picW]. Note that, calculation in the loop itself, i.e., loop for travelling from -picW to picW, shall also be counted. For a DDD coded PU, to convert a disparity vector to depth value, calculation in Eq. (3) and Eq. (4) applies as proposed method does.

For proposed method, the slice level operation is counted for the above derivation process of look-up table DispToDepthB. Note that operations in the above “while…do” loop can be done by a memory set. For proposed method with integer disparity vector (denoted as Prop #1), the PU level operation is counted for the conversion from disparity vector to depth value in Eq. (3) and Eq. (4). For proposed method with quarter pixel accuracy (denoted as Prop #1.2), only one table look-up is required.

For slice level operations, when compared Prop #1.1 with DDD, multiplication and shift are completely removed, and additional 764 addition operations are required. When compared Prop #1 with DDD with look-up table (denoted as Anchor*), all operations except memory set are dramatically reduced. 256 additional memory set is required by proposed method.

For PU level operations, when compared Prop #1.1 with Anchor, the multiplication is removed and one table look-up is required. When compared Prop #1.1 with Anchor*, same operations are required.

Anchor needs three bytes to store DispToDepthInvScale[ j ], DispToDepthInvOffset[ j ] and DispToDepthInvShift[ j ] for each reference view, and Anchor* and Prop #1 needs 2 * picW + 1 bytes to store DispToDepthB for each reference view.

When compared with Prop #1, Prop #2 requires fewer operations at both slice level and PU level, but Prop #2 require 216 bytes to store DispToDepthB for each reference view.

Table 1. Complexity comparison of DDD and proposed method

|  | Slice level operation |  |  |  |  | PU level operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mul | Add | Shift | Comp | Mem. set | Mul | Add | Shift | Comp | LU |
| Anchor | 262 | 520 | 5 | 512 | 0 | 1 | 1 | 1 | 2 | 0 |
| Anchor* | 262 + (picW*2 + 1) | 520 + (picW*2 + 1 )*2 | 5 + ( picW*2 + 1 ) | 512 + (picW*2 + 1 )*3 | 0 | 0 | 1 | 1 | 2 | 1 |
| Prop #1.1 | 0 | 1284 | 259 | 522 | 256 | 0 | 1 | 1 | 2 | 1 |
| Prop #1.2 | 0 | 1025 | 0 | 262 | 256 | 0 | 0 | 0 | 0 | 1 |

## Removal of additional motion information access

To remove the additional motion information access in DDD, disparity motion vector of a texture block that is already accessed by sub-PU based MPI candidate is used to derive depth value for DDD. Specifically, disparity vector associated with the corresponding texture block of the center sub-PU (the sub-PU located at bottom right of the center) is used for DDD.

This simplification is called Prop #2.

## Apply DDD only to partition mode 2Nx2N

To reduce the merge list construction complexity, DDD is applied only to partition mode 2Nx2N. In this way, DDD candidate is not inserted into merge list for small PU size such as 4x8 and 8x4 PU, therefore, average merge list construction complexity is reduced.

This simplification is called Prop #3.

# Experimental results

The proposed method is integrated into current 3D-HTM 10.0r1 software [3] and simulations are performed based on the common test conditions (CTC) [4]. Anchor: 3D-HTM 10.0r1.

Following cases are tested:

- Test #1: Prop #1.2.

- Test #2: Prop #3.

- Test #3: Prop #1.2 + Prop #3

- Test #4: Prop #2 with sub-PU size (in sub-PU MPI) equal to 16x16

Coding performance of the first three tests is provided in Table 2-4. As reported in Table 2-6, there is no coding performance loss in the first three tests.

As described in Section 2.2, when sub-PU size used in sub-PU MPI is equal to 8x8 (which is CTC), motion information of the texture block accessed in DDD is also accessed in sub-PU MPI, and proposed method does not change the coding performance. However, when sub-PU size is equal to 16x16, motion information of the texture block accessed in DDD is not accessed in sub-PU MPI, and proposed method has impact on the coding performance. The result of Section 2.2 with sub-PU size equal to 16x16 is shown in Table 5, and there is only 0.01% coding performance loss on synthesized views compared with a different anchor, which changes the CTC configuration by setting the sub-PU size of the sub-PU MPI to 16x16.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Prop #1.1 and prop #1.2 provide comparable results, more details will be included.

Table 2. Coding results for Prop #1.2 (simplified disparity to depth conversion)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | -0.01% |  |  | 0.00% | 116.9% | 108.7% | 117.7% |
| Kendo | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.03% | 91.0% | 85.4% | 88.6% |
| Newspapercc | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.04% | 104.4% | 93.0% | 101.4% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 96.6% | 98.6% | 97.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | -0.01% |  |  | -0.06% | 103.2% | 99.8% | 106.6% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 94.1% | 92.5% | 95.9% |
| UndoDancer | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 103.2% | 101.7% | 104.5% |
| Shark | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 110.3% | 104.6% | 108.0% |
| 1024x768 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 104.1% | 95.7% | 102.6% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.02% | 101.5% | 99.4% | 102.5% |
| average | 0.0% | 0.0% | 0.00% |  |  | 0.00% | -0.01% | 102.5% | 98.0% | 102.5% |

Table 3. Coding results for Prop #1.2 and Prop #3

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | -0.02% |  |  | 0.02% | 108.6% | 100.7% | 108.5% |
| Kendo | 0.0% | 0.0% | 0.00% | 0.01% |  |  | 0.05% | 109.3% | 98.1% | 109.0% |
| Newspapercc | 0.0% | 0.0% | 0.00% | -0.01% |  |  | -0.06% | 120.3% | 103.6% | 115.3% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 114.6% | 111.6% | 116.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | -0.01% |  |  | -0.05% | 98.5% | 92.8% | 99.2% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 103.5% | 100.1% | 107.1% |
| UndoDancer | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.05% | 105.4% | 100.1% | 103.9% |
| Shark | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 93.4% | 89.4% | 91.8% |
| 1024x768 | 0.0% | 0.0% | 0.00% | -0.01% |  |  | 0.00% | 112.7% | 100.8% | 110.9% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 103.1% | 98.8% | 103.6% |
| average | 0.0% | 0.0% | 0.00% |  |  | 0.00% | 0.00% | 106.7% | 99.5% | 106.3% |

Table 4. Coding results of Test#2 (Alignment of DDD memory access with sub-PU MPI)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | -0.02% |  |  | 0.00% | 112.6% | 106.2% | 111.8% |
| Kendo | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.06% | 100.4% | 90.3% | 97.2% |
| Newspapercc | 0.0% | 0.0% | 0.00% | -0.01% |  |  | -0.04% | 104.0% | 89.9% | 99.1% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.02% | 110.8% | 108.9% | 109.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | -0.01% |  |  | -0.07% | 103.1% | 95.8% | 103.2% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 103.7% | 102.1% | 104.6% |
| UndoDancer | 0.0% | 0.0% | 0.00% | -0.01% |  |  | 0.05% | 115.5% | 111.5% | 114.8% |
| Shark | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 105.1% | 99.1% | 104.5% |
| 1024x768 | 0.0% | 0.0% | 0.00% | -0.01% |  |  | 0.01% | 105.7% | 95.4% | 102.7% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.01% | 107.6% | 103.5% | 107.3% |
| average | 0.0% | 0.0% | 0.00% |  |  | -0.01% | 0.00% | 106.9% | 100.5% | 105.6% |

Table 5. Coding results for Test #2 (non-CTC, alignment of DDD memory access with sub-PU MPI))

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | 0.01% |  |  | 0.02% | 106.5% | 106.1% | 107.1% |
| Kendo | 0.0% | 0.0% | 0.00% | 0.01% |  |  | -0.01% | 98.1% | 93.3% | 96.2% |
| Newspapercc | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.01% | 113.7% | 111.7% | 109.1% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.01% | 93.0% | 91.9% | 93.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | 0.01% |  |  | 0.04% | 97.3% | 97.3% | 96.0% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 0.00% |  |  | -0.02% | 112.1% | 107.2% | 108.3% |
| UndoDancer | 0.0% | 0.0% | 0.00% | -0.01% |  |  | 0.00% | 102.4% | 105.7% | 104.6% |
| Shark | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.00% | 99.7% | 104.8% | 103.0% |
| 1024x768 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.01% | 106.1% | 103.7% | 104.1% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 0.00% |  |  | 0.01% | 100.9% | 101.4% | 101.0% |
| average | 0.0% | 0.0% | 0.00% |  |  | 0.00% | 0.01% | 102.8% | 102.3% | 102.2% |

# References

- K. Zhang, J. An, J. Lin, Y. Chang, S. Lei, “CE2: results on additional merging candidates for depth,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0063, 7th Meeting: San José, US, 11–17 Jan. 2014.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San José, US, 11–17 Jan. 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of Peking University having any current or pending patent rights relating to the technology described in this contribution.