| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0135 |
| --- | --- |

| Title: | CE3 related: Simplification of dim_not_present_flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, dim_not_present_flag is signaled to indicate whether one of HEVC intra prediction modes or one of depth modeling modes is used for an Intra coded prediction unit. Context adaptive binary arithmetic coding is used to code dim_not_present_flag, in which three contexts are initialized and updated. This contribution proposes to reduce the contexts used for dim_not_present_flag from three to one. Experimental results show that proposed method has no coding performance loss.

# Introduction

In 3D-HEVC [1], dim_not_present_flag is signaled for each Intra coded Prediction Unit (PU) to indicate whether the PU uses Depth Modeling Modes (DMM) or not.

Three contexts are used in entropy coding of dim_not_present_flag. To determine which context shall be used for current PU, dim_not_present_flag of the above (denote by dimFlagAbove) and left (denoted by dimFlagLeft) neighboring blocks are checked.

- If both dimFlagAbove and dimFlagLeft are equal to 0, the first context is used.

- If one of dimFlagAbove and dimFlagLeft is equal to 0, the second context is used.

- If both dimFlagAbove and dimFlagLeft are equal to 1, the third context is used.

# Proposal

This contribution proposes to use only one context for coding dim_not_present_flag regardless of dim_not_present_flag of the above or left neighboring blocks are.

The decoding process is changed as follows. Newly added parts are highlighted in green and deleted parts are marked with strikethrough.

- I.9.3.2.2 Initialization process for context variables

The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses 7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses Error: Reference source not found to Error: Reference source not found.

- Table Error: Reference source not found-1 is appended to the end of Table 9-4.

- Error: Reference source not found to Table Error: Reference source not found-2 are appended to the end of the subclause.

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| cu_extension( ) intra_mode_ext( ) | depth_intra_mode_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..2 | 3..5 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | dbbp_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dim_not_present_flag | Table Error: Reference source not found-2 | 0..2 | 13..5 | 26..8 |

…

Table Error: Reference source not found-2 – Values of initValue for dim_not_present_flag ctxIdx

| Initialization variable | ctxIdx of dim_not_present_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 13 | 4 | 5 | 26 | 7 | 8 |
| initValue | 154 | 155 | 156 | 141 | 185 | 214 | 155 | 170 | 157 |

- I.9.3.4.2.1 General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Table Error: Reference source not found-3 is appended to the end of Table 9-37.

Table Error: Reference source not found-3 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Table Error: Reference source not found-4 | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0, 1,2 Table Error: Reference source not found-4 | na | na | na | na | na |

- I.9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

The specifications in subclause 9.3.4.2.2 apply with the following modifications.

–	Table Error: Reference source not found-4 is appended to the end of Table 9-38.

Table Error: Reference source not found-4 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| iv_res_pred_weight_idx | iv_res_pred_weight_idx [ xL ][ yL ] |  | ( condL && availableL ) |
| dim_not_present_flag | dim_not_present_flag[ xL ][ yL ] | dim_not_present_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

# Experimental results

The proposed method is integrated into current 3D-HTM 10.0r1 software [2] and simulations are performed based on the common test conditions (CTC) [3] for both Random Access case and All Intra case. Anchor: 3D-HTM 10.0r1.

The coding performance is provided in Table 1-2. As reported in Table 1-2, there is no coding performance loss in both test cases.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 1: Coding results of proposed method for 3-view case (Random Access)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.2% | 0.0% | 0.03% | -0.01% |  |  | 0.04% | 117.6% | 110.2% | 118.1% |
| Kendo | 0.0% | 0.0% | -0.02% | -0.01% |  |  | -0.02% | 92.0% | 85.0% | 90.0% |
| Newspapercc | 0.0% | -0.1% | -0.01% | -0.04% |  |  | -0.02% | 105.5% | 94.4% | 102.6% |
| GhostTownFly | 0.0% | -0.2% | -0.02% | 0.00% |  |  | 0.00% | 113.9% | 111.9% | 115.4% |
| PoznanHall2 | 0.1% | -0.1% | 0.01% | 0.02% |  |  | -0.08% | 107.8% | 105.0% | 110.4% |
| PoznanStreet | -0.1% | 0.0% | -0.02% | -0.01% |  |  | -0.01% | 103.9% | 97.9% | 104.5% |
| UndoDancer | 0.1% | -0.1% | -0.01% | 0.00% |  |  | -0.04% | 114.8% | 112.9% | 116.7% |
| Shark | -0.1% | -0.1% | -0.02% | -0.04% |  |  | -0.04% | 100.1% | 96.4% | 97.1% |
| 1024x768 | 0.1% | 0.0% | 0.00% | -0.02% |  |  | 0.00% | 105.0% | 96.5% | 103.6% |
| 1920x1088 | 0.0% | -0.1% | -0.01% | 0.00% |  |  | -0.03% | 108.1% | 104.8% | 108.8% |
| average | 0.0% | -0.1% | -0.01% |  |  | -0.01% | -0.02% | 106.9% | 101.7% | 106.9% |

Table 2: Coding results of proposed method for 3-view case (All Intra)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.01% | 105.6% | 101.4% | 103.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% |  |  | -0.01% | 81.3% | 87.0% | 83.6% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.00% | 95.9% | 97.7% | 95.7% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.00% | 111.6% | 108.7% | 111.3% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.01% | 90.1% | 88.7% | 87.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.00% | 95.2% | 95.3% | 96.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.01% | 105.6% | 104.7% | 105.1% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.01% | 100.0% | 100.3% | 101.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.00% | 94.3% | 95.4% | 94.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% |  |  | 0.01% | 100.5% | 99.6% | 100.3% |
| average | 0.0% | 0.0% | 0.0% |  |  | 0.0% | 0.00% | 98.2% | 98.0% | 98.1% |

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San José, US, 11–17 Jan. 2014.

- 3DV-HTM version 10.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-10.0r1/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).