| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0136 |
| --- | --- |

| Title: | Constraints for depth modeling modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, for a Prediction Unit (PU) coded with Depth Modeling Mode (DMM), split depth of the associated transform tree (if available) may be greater than 0, meaning that a DMM coded PU may be located in multiple transform units. However, the decoding process for this case is not defined appropriately and the decoder cannot handle it. Such flexibility in the current working draft of 3D-HEVC is considered as a bug for our current design. This contribution proposes to constrain the split depth of the associated transform tree of DMM coded PU to be equal to zero. Proposed method has no impact on coding efficiency.

# Problem

In 3D-HEVC [1], for a PU coded with DMM mode, similar decoding process as in HEVC intra prediction modes applies when split depth of the associated transform tree is greater than 0. Specifically, as described in the decoding process for intra blocks in 3D-HEVC (sub-clause I.8.4.4.1 in [1]), each Transform Unit (TU) is predicted and reconstructed in Z order (or decoding order), and reconstructed TU may be used for intra prediction of its following TUs. According to the current 3D-HEVC WD text, similar as HEVC specification, the Intra prediction for each TU within a PU is done, assuming all TUs within one PU share the same intra prediction information signaled at PU level. In 3D-HEVC, DMM modes are handled in a way similar to conventional Intra prediction modes in terms of interaction with Tus. That means, each TU can take the DMM mode signalled in the PU level and apply e.g., a Wedgelet partition.

For example, when split depth of the associated transform tree of the PU is equal to one and the PU contains four TUs, the PU is reconstructed by predicting and reconstructing these four TUs one by one in Z order. The first TU is predicted using neighboring samples of the PU, and then reconstructed by adding its residual sample (signaled in the transform tree) to the prediction sample. For each of the other TUs, it is predicted using neighboring samples of the PU and/or neighboring samples in its preceding TUs, and reconstructed by adding residue sample to the prediction sample. All the four TUs are predicted using the DMM pattern of the PU (may be signaled at PU level).

However, in this case, because each TU within the PU, instead of the entire PU, is predicted using the DMM pattern, each TU is partitioned into two regions. Therefore the PU contains more than two regions, as the example shown in Figure 1 for DMM mode 1. This is not aligned with the design intent of the DMM modes, which enables only up to two partitions for a prediction unit.

Figure 1. A PU coded with DMM mode 1 contains multiple regions.

# Proposal

This contribution proposes two constrains on Intra coded PU to tackle the above problems, as highlighted in yellow in Figure 2:

- When a PU is coded with DMM modes (DMM mode 1 or DMM mode 4), split depth of the associated transform tree (if available) is constrained to be equal to zero, i.e., the TU size is constrained to be equal to the PU size.

- When PU size is greater than the largest transform blocks size (therefore TU size cannot be forced equal to the PU size) and residue of the PU is not coded by SDC, therefore a transform tree may be signaled, the PU is constrained to be coded with HEVC intra prediction modes. This constraint is an auxiliary constraint to make sure the first constraint to be true.

Figure 2. Constraint on DMM modes

These two constraints are already fulfilled in bitstream generated by HTM encoder under CTC. Therefore, they do not change the coding performance.

The semantics constraints are provided as follows. New added parts are highlighted in green and deleted parts are marked with strikethrough. Note that the constraints reflect the highlighted branches as shown in Figure 2.

The proposed constraints can be considered as bug fixes and have no coding performance impact of the current HTM.

- 7.4.9.8	Transform tree semantics

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with half horizontal and half vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When dim_not_present_flag[ x0 ][ y0 ] is equal to 0, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is constrained as follows:

- When PartMode is equal to PART_2Nx2N , the value of split_transform_flag[ x0 ][ y0 ][ 0 ] shall be equal to 0.

- When PartMode is equal to PART_NxN, the value of split_transform_flag[ x0 ][ y0 ][ 1 ], when present, shall be equal to 0.

…

- I.7.4.9.5.1	Intra mode extension semantics

The variable Log2MaxDmmCbSize is set equal to 5.

dim_not_present_flag[ x0 ][ y0 ] equal to 1 specifies that the depth_intra_mode_flag syntax element is not present and that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. dim_not_present_flag[ x0 ][ y0 ] equal to 0 specifies that that the depth_intra_mode_flag syntax element might be present. When not present, the value of dim_not_present_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- When log2CbSize is greater than Log2MaxTrafoSize and sdc_flag[ x0 ][ y0 ] is equal to 0, the value of dim_not_present_flag[ x0 ][ y0 ] shall be equal to 1.

…

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).