| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0137 |
| --- | --- |

| Title: | Control of the availability of advanced inter-view coding predictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the last meeting, JCT3V-G0067 was adopted to disable certain merge candidates introduced in 3D-HEVC when there is no inter-view picture in current reference lists which results in the unavailability of reference view of derived DV from NBDV process. During the merge list construction process for each prediction unit, additional conditions are added to check the availability of inter-view reference pictures, although such information is the same for all prediction units (PUs) within one slice. In this contribution, the PU-level checking is replaced by slice-level checking to remove the redundancy. In addition, some further clean-ups of the working draft for other inter-view coding tools are also provided.

# Introduction

In JCT3V-G0067 [1], it is proposed to disable the temporal inter-view MV candidate (including sub-PU temporal inter-view MV candidate and shifted temporal inter-view MV candidate) and VSP candidate when there is no inter-view picture in current reference lists which results in the unavailability of reference view of derived DV from NBDV process.

The corresponding working draft text changes are highlighted in yellow:

- I.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when MergeFlag[ xPb ][ yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- …

Outputs of this process are:

…

The function differentMotion( N, M ) is specified as follows:

- If one of the following conditions is true, differentMotion( N, M ) is equal to 1:

  - predFlagLXN  !=  predFlagLXM (with X being replaced by 0 and 1),

  - mvLXN  !=  mvLXM (with X being replaced by 0 and 1),

  - refIdxLXN  !=  refIdxLXM (with X being replaced by 0 and 1),

- Otherwise, differentMotion( N, M ) is equal to 0.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for the base merge candidate list as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - For N being replaced by A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagN is set equal to 0.

  - Depending on iv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively).

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xPb, yPb ) and the variables nPbW and nPbH as inputs, and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on mpi_flag[ nuh_layer_id ], the following applies:

    - If mpi_flag[ nuh_layer_id ] is equal to 0, the variables availableFlagT and availableFlagD are set equal to 0.

    - Otherwise (mpi_flag[ nuh_layer_id ] is equal to 1), the derivation process for the texture and the disparity derived merging candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the outputs are the flags availableFlagT and  availableFlagD, the prediction utilization flags predFlagLXT, and predFlagLXD, the reference indices refIdxLXT and refIdxLXD, the motion vectors mvLXT and mvLXD (with X being 0 or 1, respectively), and the variable dispDerivedDepthVal.

Here, the variable DispAvailabilityIdc is defined as follows in the CU-level NBDV process (I.8.5.5 in JCT3V-G1001):

  - DispAvailabilityIdc[ x ][ y ] = dvAvailFlag ? DISP_AVAILABLE : 
	( DefaultRefViewIdxAvailableFlag ? DISP_DEFAULT : DISP_NONE )	(Error: Reference source not found-1)

wherein dvAvailFlag indicates whether there is one disparity motion vector from the spatial or temporal neighboring blocks used in the NBDV process and DefaultRefViewIdxAvailableFlag is used to indicate whether current slice has at least one inter-view reference picture.

When and only when current slice doesn’t include any inter-view reference pictures, the value of DispAvailabilityIdc[ x ][ y ] is set to DISP_NONE.

# Problem

The solution in JCT3V-G0067 has the following problems:

- The availability of inter-view reference pictures in the merge list construction process is checked for each PU, although such information is the same for all PUs within one slice.

- When there are no inter-view reference pictures in current slice, similar problems may happen for other advanced coding tools. In this case, these advanced coding tools shall be disabled. However, such modifications are not considered in JCT3V-G0067. The advanced coding tools include:

  - Advanced residual prediction

  - Illumination compensation

  - Disparity vector derivation process.

# Proposed Scheme

A variable is determined only in the slice level and used to control whether a certain decoding process needs to be invoked or not. With this, the decoding processes do not need to have the inter-view reference picture availability check in the block level. The specification text can be made cleaner with this proposal.

To be more specific, the following variables are derived:

- A variable ivMvPredPosFlag is set equal to ( iv_mv_pred_flag[ layerId ] && inter_layer_pred_enabled_flag ). For each PU, the check of the value of ivMvPredPosFlag is applied to determine whether the derivation process for the inter-view merge candidates and shifted disparity merging candidate are invoked. When ivMvPredPosFlag is equal to 0, the derivation process for the inter-view merge candidates and shifted disparity merging candidate is not invoked.

- A variable vspPosFlag is set equal to ( view_synthesis_pred_flag[ layerId ] && inter_layer_pred_enabled_flag ). For each PU, the check of the value of vspPosFlagis applied to determine whether the derivation process for the view synthesis prediction merge candidate is invoked. When ivMvPredPosFlag is equal to 0, the derivation process for the view synthesis prediction merging candidate is not invoked.

- A variable resPredPosFlag is set equal to ( iv_res_pred_flag[ layerId ] && inter_layer_pred_enabled_flag ). When resPredPosFlag is equal to 0, the signaling of iv_res_pred_weight_idx shall be skipped and advanced residual prediction shall be disabled for current slice.

- In one example, a variable icPosFlag is set equal to ( slice_ic_enable_flag && inter_layer_pred_enabled_flag ). Furthermore, when icPosFlag is equal to 0, signaling of ic_flag shall be skipped and illumination compensation shall be disabled for current slice.

- The disparity vector derivation process for coding units of a slice is determined to be skipped or not based on the above variables.

Furthermore, it is proposed to add some constraints that when inter-view reference pictures are unavailable for certain dependent view, the advanced coding tools (inter-view motion prediction, illumination compensation, advanced residual prediction and view synthesis prediction) shall be disabled.

# Proposed Working Draft Changes

The newly added parts compared to 3D-HEVC working draft 3 [2] are highlighted in green and the removed parts are marked with strikethrough.

- I.8.3.10		Initialization of variables

A variable ivMvPredPosFlag is set equal to (iv_mv_pred_flag[ nuh_layer_id ] && inter_layer_pred_enabled_flag).

A variable vspPosFlagis set equal to (view_synthesis_pred_flag[ nuh_layer_id ] && inter_layer_pred_enabled_flag).

A variable resPredPosFlag  is set equal to (iv_res_pred_flag[ nuh_layer_id ] && inter_layer_pred_enabled_flag).

A variable icPosFlag is set equal to (slice_ic_enable_flag && inter_layer_pred_enabled_flag).

- I.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xPb ][ yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- …

Outputs of this process are:

- …

...

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for the base merge candidate list as specified in subclause Error: Reference source not found is invoked  with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - For N being replaced by A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagN is set equal to 0.

  - Depending on ivMvPredPosFlagiv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If ivMvPredPosFlagiv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (ivMvPredPosFlagiv_mv_pred_flag[ nuh_layer_id ] is equal to 1 and inter_layer_pred_enabled_flag), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, as the inputs and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively).

  - Depending on vspPosFlagview_synthesis_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If vspPosFlagview_synthesis_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, the flag availableFlagVSP is set equal to 0.

    - Otherwise (vspPosFlagview_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on mpi_flag[ nuh_layer_id ], the following applies:

    - …

  - The merging candidate list, extMergeCandList, is constructed as follows:

- …

  - When ivMvPredPosFlag is equal to 1, Tthe variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in extMergeCandList, extMergeCandList, and i as the inputs and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

- I.7.4.9.5.2	Coding unit extension semantics

The variable rpEnableFlag is derived as specified in the following:

  - rpEnableFlag = iv_res_pred_flag[ nuh_layer_id ] resPredPosFlag   &&  RpRefPicAvailFlag 
	( CuPredMode[ x0 ][ y0 ]  !=  MODE_INTRA )  &&  ( PartMode  = =  PART_2Nx2N )	(Error: Reference source not found-2)

iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0.

- The variable icEnableFlag is set equal to 0 and when icPosFlag slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

    - icEnableFlag = ( merge_idx[ x0 ][ y0 ] != 0 )  | |  !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-3)

- Otherwise (merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0  &&  refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1  &&  refViewIdxL0  !=  ViewIdx)	(Error: Reference source not found-4)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

When DispAvailabilityIdc[ x0 ][ y0 ] is not equal to DISP_AVAILABLE, iv_res_pred_weight_idx shall be equal to 0.

- I.8.5.1	General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ]ivMvPredPosFlag is equal to 1, or resPredPosFlag iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or vspPosFlag view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, following applies:

    - If DepthFlag is equal to 0 the derivation process for disparity vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

    - Otherwise (DepthFlag is equal to 1), the derivation process for disparity vectors from neighouring depth samples as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

  - …

- H.7.4.3.1.2	Video parameter set extension 2 semantics

- iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of the layer with nuh_layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction is not used for the layer with nuh_layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with nuh_layer_id equal to layerId. When not present, the value of iv_mv_pred_flag[ layerId ] is inferred to be equal to 0. When NumDirectRefLayers[ layerId ] is equal to 0, the value of iv_mv_pred_flag[ layerId ] shall be equal to 0.

- log2_sub_pb_size_minus2[ layerId ] specifies the value of the variable SubPbSize[ layerId ] that is used in the decoding of prediction units using the inter-view merge candidate. The value of log2_sub_pb_size_minus2 shall be in the range of 0 to 4, inclusive.

- The variable SubPbSize[ layerId ] is derived as specified in the following:

- SubPbSize[ layerId ] = VpsDepthFlag( layerId ) ? 64 : 1  <<  ( log2_sub_pb_size_minus2[ layerId ]+ 2 )	(Error: Reference source not found-5)

- iv_res_pred_flag[ layerId ] indicates whether inter-view residual prediction is used in the decoding process of the layer with nuh_layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 0 specifies that inter-view residual prediction is not used for the layer with nuh_layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 1 specifies that inter-view residual prediction may be used for the layer with nuh_layer_id equal to layerId. When not present, the value of iv_res_pred_flag[ layerId ] is to be equal to 0. When NumDirectRefLayers[ layerId ] is equal to 0, the value of iv_res_pred_flag[ layerId ] shall be equal to 0.

- view_synthesis_pred_flag[ layerId ] equal to 0 specifies that view synthesis prediction merge candidates are not used for the layer with nuh_layer_id equal to layerId. view_synthesis_pred_flag[ layerId ] equal to 1 specifies that view synthesis prediction merge candidates might be used for the layer with nuh_layer_id equal to layerId. When not present, the value of view_synthesis_pred_flag[ layerId ] is inferred to be equal to 0. NumDirectRefLayers[ layerId ] is equal to 0, the value of view_synthesis_pred_flag[ layerId ] shall be equal to 0.

# References

- J. An, Y.-W. Chen, K. Zhang, J.-L. Lin and S. Lei, “Availability checking of temporal inter-view MV candidate and VSP candidate,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0067, 7th Meeting: San José, US, 11–17 Jan. 2014.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1001, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).