| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0195 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0168 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On representation format signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori, Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 shinobu.hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution discusses representation format (rep_format) signaling in VPS extension.

First, the default mapping between the rep_format and layer is discussed. Currently, this happens when rep_format_idx_present_flag is equal to 0. In this case, the number of rep_formats is constrained to be equal to the number of layers (MaxLayersMinus1). In the contribution, more flexible method is studied and proposed to release this constraint.

Second, a case in which vps_num_rep_formats_minus1 is equal to 0 is discussed. The contribution proposes a clean-up signaling vps_num_rep_formats_minus1. The contribution indicates that the update in SPS extension by update_rep_format_flag is redundant in this case.

The contribution provides comparison of signalled parameter in the bitstream with examples which are based on the use-cases from CTC in MV-HEVC and SHVC.

# Introduction

In the latest SHVC and MV-HEVC draft [1][2], image resolution, bitdepth and chroma format information are signalled in VPS extension, as quoted in Table 1. One of the big benefits of this structure is to enable decoders to easily overlook and interpret the whole bitstream structure rather than parsing SPSs for all layers.

Table 1 rep_format syntax in VPS extension (from JCTVC-P1008/JCT3V-G1004)

| 1 | rep_format_idx_present_flag | u(1) |
| --- | --- | --- |
| 2 | if( rep_format_idx_present_flag ) |  |
| 3 | vps_num_rep_formats_minus1 | ue(v) |
| 4 | for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| 5 | rep_format( ) |  |
| 6 | if( rep_format_idx_present_flag ) |  |
| 7 | for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| 8 | if( vps_num_rep_formats_minus1 > 0 ) |  |
| 9 | vps_rep_format_idx[ i ] | u(v) |

According to [1][2], mapping between representation formats (rep_formats) and layers is interpreted by decoders based on the value of rep_format_idx_present_flag (See Figure 1).

It is pointed that the default mapping in the VPS is provided just for the information for decoders since an update of the mapping is enabled in SPS with update_rep_format_flag. Thus, it can be asserted that the important information as representation format in VPS is the number of rep_formats and those contents.

From this assumption, the contribution proposes a clean-up on the rep_format signalling.

Figure 1 Representation format signaling and mapping in the current draft

# Proposal

## Changes in VPS extension

Proposed syntax and semantics in VPS extension are shown in Table 2. No syntax element is removed but the signalling condition is modified. In the semantics, the changes are yellow highlighted.

Table 2 proposed syntax in VPS extension

| 1 | vps_num_rep_formats_minus1 | ue(v) |
| --- | --- | --- |
| 2 | for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| 3 | rep_format( ) |  |
| 4 | if( vps_num_rep_formats_minus1 > 0 ) |  |
| 5 | rep_format_idx_present_flag | u(1) |
| 6 | if( rep_format_idx_present_flag ) |  |
| 7 | for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| 8 | vps_rep_format_idx[ i ] | u(v) |

vps_num_rep_formats_minus1 plus 1 specifies the number of the following rep_format( ) syntax structures in the VPS. When not present, the value of vps_num_rep_formats_minus1 is inferred to be equal to MaxLayersMinus1. The value of vps_num_rep_formats_minus1 shall be in the range of 0 to 255, inclusive.

rep_format_idx_present_flag equal to 1 specifies that the syntax elements vps_rep_format_idx[ i ] is present. rep_format_idx_present_flag equal to 0 specifies that the syntax element vps_rep_format_idx[ i ] is not present. When not present, the value of rep_format_idx_present_flag is inferred to be equal to 0.

vps_rep_format_idx[ i ] specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of vps_rep_format_idx[ i ] is inferred to be equal to ( rep_format_idx_present_flag ? 0 : i Min ( i, vps_num_rep_formats_minus1) ). The value of vps_rep_format_idx[ i ] shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive. The number of bits used for the representation of vps_rep_format_idx[ i ] is Ceil( Log2( vps_num_rep_formats_minus1 + 1 ) ).

## Changes in SPS extension

It is proposed to add a constraint to the semantics of update_rep_format_flag in SPS extension, in order to avoid confusion for decoders.

update_rep_format_flag equal to 1 specifies that sps_rep_format_idx is present and that the sps_rep_format_idx-th rep_format( ) syntax structures in the active VPS applies to the layers that refer to this SPS. update_rep_format_flag equal to 0 specifies that sps_rep_format_idx is not present. When not present, the value of update_rep_format_flag is inferred to be equal to 0. It is a requirement of bitstream conformance that the value of update_rep_format_flag shall be 0 if the value of vps_num_rep_formats_minus1 in the active VPS is equal to 0.

# Examples

## View or SNR scalability, 3layers (MV-HEVC CTC)

Figure 2 shows an example for three layers with view/SNR scalability. It is same as MV-HEVC CTC. A picture size is W and H for width and height, respectively. Here, chroma_format and bitdepth part are not touched for simplicity.

Figure 2 View/SNR scalability, 3 layers (MV-HEVC CTC)

As shown in Table 3, currently there are two ways to represent the case. When the value of rep_format_idx_present_flag is equal to 1, the signaling is simpler than the other case (rep_format_idx_present_flag is equal to 0).

However, in the current, it is questioned if rep_format_idx_present_flag is necessary since the number of rep_format is just one.

In terms of the design consistency (not for the 1-bit reduction), the simplification is proposed. In the proposal, vps_num_rep_formats_minus1 is signaled equal to 0 at first. Thus, it is not necessary to signal rep_format_idx_present_flag. Since the number of rep_formats is just one, the default mapping process occurs as the layers refer the 0th rep_format.

Table 3 Comparison of signaled/inferred value for the case in Section 3.1

| Syntax element | Current |  | Proposed |
| --- | --- | --- | --- |
| rep_format_idx_present_flag | 1 | 0 | (inferred as 0) |
| vps_num_rep_formats_minus1 | 0 | (inferred as 2) | 0 |
| 0th rep_format() | W*H | W*H | W*H |
| 1st rep_format() | - | W*H | - |
| 2nd rep_format() | - | W*H | - |
| vps_rep_format_idx[ 0 ] | (inferred as 0) | (inferred as 0) | (inferred as 0) |
| vps_rep_format_idx[ 1 ] | (inferred as 0) | (inferred as 1) | (inferred as 0) |
| vps_rep_format_idx[ 2 ] | (inferred as 0) | (inferred as 2) | (inferred as 0) |

## Spatial scalability, 2layers (SHVC CTC)

Figure 3 shows an example for two layers with spatial scalability. It is same as one of SHVC CTC. A picture size is W * H for the base layer and 2W*2H for the enhancement layer.

Figure 3 Spatial scalability, 2 layers (SHVC CTC)

As shown in Table 4, currently there are two ways to represent the case. In contrast to the example in section 3.1, the signaled value is simpler when the value of rep_format_idx_present_flag is equal to 0.

There are also two ways in the proposed. Since Layer1 may use the 0th rep_format (W*H), the capability to select the rep_format by vps_rep_format_idx[i] is kept in the proposal.

In the example, there is no big difference between the current and the proposed.

Table 4 Comparison of signaled/inferred value for the case in Section 3.2

| Syntax element | Current |  | Proposed |  |
| --- | --- | --- | --- | --- |
| rep_format_idx_present_flag | 1 | 0 | 1 | 0 |
| vps_num_rep_formats_minus1 | 1 | (inferred as 1) | 1 | 1 |
| 0th rep_format() | W*H | W*H | W*H | W*H |
| 1st rep_format() | 2W*2H | 2W*2H | 2W*2H | 2W*2H |
| 2nd rep_format() | - | - | - | - |
| vps_rep_format_idx[ 0 ] | (inferred as 0) | (inferred as 0) | (inferred as 0) | (inferred as 0) |
| vps_rep_format_idx[ 1 ] | 1 | (inferred as 1) | 1 | (inferred as 1) |
| vps_rep_format_idx[ 2 ] | - | - | - | - |

## Combination of SNR and Spatial scalability, 3layers

The current SHVC supports combination of SNR and Spatial scalability. Figure 4 is an example of the combination.

Figure 4 Combination of SNR and spatial scalability, 3 layers

Like the example in Section 3.2, both the current and the proposed enable two ways of signaling.

In the example, it is emphasized that the proposal enables to set rep_format_idx_present_flag as 0 though only two rep_formats are signaled. It is easier for decoders to understand the possible resolutions combination from the value of vps_num_rep_formats_minus1, keeping the default rep_format-layer mapping capability.

When rep_format_idx_present_flag is 1, the current and the proposal are same.

Table 5 Comparison of signaled/inferred value for the case in Section 3.3

| Syntax element | Current |  | Proposed |  |
| --- | --- | --- | --- | --- |
| rep_format_idx_present_flag | 1 | 0 | 1 | 0 |
| vps_num_rep_formats_minus1 | 1 | (inferred as 2) | 1 | 1 |
| 0th rep_format() | W*H | W*H | W*H | W*H |
| 1st rep_format() | 2W*2H | 2W*2H | 2W*2H | 2W*2H |
| 2nd rep_format() | - | 2W*2H | - | - |
| vps_rep_format_idx[ 0 ] | (inferred as 0) | (inferred as 0) | (inferred as 0) | (inferred as 0) |
| vps_rep_format_idx[ 1 ] | 1 | (inferred as 1) | 1 | (inferred as 1) |
| vps_rep_format_idx[ 2 ] | 1 | (inferred as 2) | 1 | (inferred as 1) |

## Spatial scalability, 2layers and 1 potential rep_format

As stated in the introduction, the important role of the rep_format signaling in VPS is to let decoders know the possible set of the whole bitstream representations. An enhancement layer may change the resolution after VPS is signalled. Figure 5 shows an example of the case. At first, two layers are W*H and 2W*2H, then the enhancement layer may change the resolution to 3W*3H later.

Figure 5 Spatial scalability 2layers and 1 potential rep_format

Table 6 shows a comparison between the current and the proposed.

Currently, rep_format_idx_present_flag shall be 1 in the example. In the proposed, rep_format_idx_present_flag can be 0 due to the change of the semantics. It is believed that the capability is beneficial when the number of layers is increased. In the extreme case, vps_num_rep_formats_minus1 is 255 and MaxLayersMinus1 is 62. Then, vps_rep_format_idx[ i ] should be signaled 62 times without the capability.

When rep_format_idx_present_flag equal to 1, the signaled values are same as the current.

Table 6 Comparison of signaled/inferred value for the case in Section 3.4

| Syntax element | Current |  | Proposed |  |
| --- | --- | --- | --- | --- |
| rep_format_idx_present_flag | 1 | 0 | 1 | 0 |
| vps_num_rep_formats_minus1 | 2 | Not possible | 2 | 2 |
| 0th rep_format() | W*H |  | W*H | W*H |
| 1st rep_format() | 2W*2H |  | 2W*2H | 2W*2H |
| 2nd rep_format() | 3W*3H |  | 3W*3H | 3W*3H |
| vps_rep_format_idx[ 0 ] | (inferred as 0) |  | (inferred as 0) | (inferred as 0) |
| vps_rep_format_idx[ 1 ] | 1 |  | 1 | (inferred as 1) |
| vps_rep_format_idx[ 2 ] | - |  | - | - |

# Conclusion

In the contribution, rep_format signaling is discussed. Because it is believed that the number of rep_format and the contents are more important than the rep_format-layer mapping information, the changes of syntax and semantics are proposed. In short, vps_num_rep_formats_minus1 should be a dominant syntax element in terms of the design consistency.

# Reference

- JCTVC-P1008, “High efficiency video coding (HEVC) scalable extension Draft 5”

- JCT3V-G1004, “MV-HEVC Draft Text 7”

- JCTVC-N0092, “MV-HEVC/SHVC HLS: Representation format information in VPS”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).