| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0177 |
| --- | --- |

| Title: | 3D-CE1: Cross-check of Simplification of 3D Merge list construction (JCT3V-H0097) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of merge candidate construction simplification. The experiment results perfectly match with those provided by the proponent. It is reported the BD-rate increase on average is 0.06 %, 0.04 % and 0.09 % in texture, video and synthesis respectively.

# Experimental results

Table 1. Test results (anchor: HTM10r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.1% | 0.08% | 0.06% | 0.11% | 99.2% | 99.2% | 101.4% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.04% | 0.00% | 0.11% | 98.8% | 99.2% | 99.1% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.01% | -0.03% | 0.08% | 99.0% | 96.3% | 100.2% |
| GT_Fly | 0.0% | 0.5% | 0.4% | 0.08% | 0.08% | 0.13% | 99.0% | 98.1% | 99.9% |
| Poznan_Hall2 | 0.0% | 0.3% | -0.3% | 0.06% | 0.02% | -0.04% | 99.2% | 101.0% | 100.3% |
| Poznan_Street | 0.0% | 0.2% | 0.2% | 0.05% | 0.05% | 0.08% | 98.9% | 99.0% | 99.8% |
| Undo_Dancer | 0.0% | 0.6% | 0.5% | 0.14% | 0.13% | 0.19% | 99.1% | 99.9% | 99.9% |
| Shark | 0.0% | 0.2% | 0.1% | 0.02% | 0.00% | 0.04% | 98.7% | 100.2% | 100.4% |
| 1024x768 | 0.0% | 0.2% | 0.0% | 0.04% | 0.01% | 0.10% | 99.0% | 98.2% | 100.2% |
| 1920x1088 | 0.0% | 0.4% | 0.2% | 0.07% | 0.06% | 0.08% | 99.0% | 99.6% | 100.1% |
| average | 0.0% | 0.3% | 0.1% | 0.06% | 0.04% | 0.09% | 99.0% | 99.1% | 100.1% |

# References

[1] J. Y. Lee, M. W. Park, C. Kim, “3D-CE1: Simplification of 3D Merge list construction”, JCT3V-H0097, JCT3V 8th Meeting: Valencia, ES, 29 March – 4 April 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).