| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0189 |
| --- | --- |

| Title: | 3D-HEVC: Harmonization of depth lookup table for intra prediction modes for intra depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Houari Sabirin Kei Kawamura Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ho-sabirin@kddilabs.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

Current adopted methods of utilizing depth look-up table (DLT) for depth intra coding have different handling for different intra prediction modes. This contribution proposes some alternatives of harmonization of utilizing the DLT in depth intra coding by taking into account the equal treatment for all intra prediction modes. It is shown that coding gain can be achieved.

# Introduction

Applying DLT into intra modes of depth map was introduced at the Vienna meeting [1] and the utilization of DLT for intra modes for depth data was adopted at the Geneva meeting [2]. The utilization of DLT has provided bit savings up to 0.2% and 0.3% for random access and all-intra configuration, respectively.

Also at the last meeting the quantization of differences between original and predicted pixels in the residual domain was proposed [3] in which the coding gain can be achieved down to -0.2% for CTC and -0.4% for AI conditions.

In this contribution, the utilization of DLT is simplified by two alternatives: 1) applied to only non-angular intra prediction modes (DMM, RBC, DC, vertical, horizontal and planar); or 2) applied to all prediction modes.

# Proposed Method

This contribution proposes two methods that reflect the harmonization of utilizing depth lookup table for different intra prediction modes in depth coding.

## Method 1: applying DLT for planar intra prediction mode

This method proposes applying depth lookup table for planar prediction mode (PLANAR_IDX) along with already adopted DMM, RBC, DC, vertical and horizontal prediction modes.

## Method 2: applying DLT for all intra prediction mode

In this method, the depth lookup table is simply applied for all intra prediction modes in depth intra coding.

# Experimental Results

The proposed method is integrated into HTM 10.0 reference software and three tests are conducted under random access (CTC) and intra-only configuration

## Test 1: Results of the proposed method I

The proposed method I provides -0.03% and -0.011% gain for CTC and AI cases, respectively.

Table 1. Performance of the proposed method I in CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.088% | 0.013% | 0.027% | 0.010% | 0.175% | 97.625% | 97.692% | 95.265% |
| Kendo | 0.000% | -0.039% | -0.106% | -0.041% | -0.182% | -0.163% | 98.561% | 97.920% | 95.575% |
| Newspaper_CC | 0.000% | 0.078% | -0.032% | 0.003% | -0.036% | 0.056% | 97.693% | 97.944% | 95.040% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.044% | 97.582% | 90.648% | 88.796% |
| Poznan_Hall2 | 0.000% | 0.037% | -0.246% | -0.023% | -0.029% | -0.161% | 97.441% | 98.608% | 95.974% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.088% | 98.538% | 99.711% | 91.939% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.042% | 97.203% | 95.326% | 89.492% |
| Shark | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 3.746% | 97.158% | 96.687% | 89.856% |
| 1024x768 | 0.000% | 0.042% | -0.042% | -0.004% | -0.069% | 0.023% | 97.960% | 97.852% | 95.294% |
| 1920x1088 | 0.000% | 0.007% | -0.049% | -0.005% | -0.006% | 0.752% | 97.584% | 96.196% | 91.212% |
| average | 0.000% | 0.020% | -0.046% | -0.004% | -0.030% | 0.478% | 97.725% | 96.817% | 92.742% |

Table 1. Performance of the proposed method I in AI

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.000% | 0.000% | 0.000% | -0.032% | 0.370% | 99.232% | 104.863% | 104.401% |
| Kendo | 0.000% | 0.000% | 0.000% | 0.000% | -0.044% | 0.210% | 99.211% | 102.407% | 101.372% |
| Newspaper_CC | 0.000% | 0.000% | 0.000% | 0.000% | -0.020% | 0.326% | 98.420% | 100.962% | 98.682% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.145% | 99.151% | 98.047% | 95.587% |
| Poznan_Hall2 | 0.000% | 0.000% | 0.000% | 0.000% | 0.005% | 0.021% | 100.089% | 102.376% | 102.514% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.623% | 98.093% | 97.721% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.095% | 100.443% | 103.172% | 105.209% |
| Shark | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.347% | 97.688% | 97.506% |
| 1024x768 | 0.000% | 0.000% | 0.000% | 0.000% | -0.032% | 0.302% | 98.954% | 102.744% | 101.485% |
| 1920x1088 | 0.000% | 0.000% | 0.000% | 0.000% | 0.001% | 0.052% | 99.730% | 99.875% | 99.707% |
| average | 0.000% | 0.000% | 0.000% | 0.000% | -0.011% | 0.146% | 99.439% | 100.951% | 100.374% |

## Test 2: Results of the proposed method II

The proposed method II provides -0.068% and -0.050% gain for CTC and AI cases, respectively.

Table 3. Performance of the proposed method II in CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.062% | -0.091% | 0.010% | -0.077% | 0.125% | 98.662% | 94.448% | 91.562% |
| Kendo | 0.000% | -0.059% | -0.057% | -0.019% | -0.313% | -0.145% | 99.449% | 94.396% | 91.307% |
| Newspaper_CC | 0.000% | -0.001% | -0.103% | -0.014% | -0.128% | 0.014% | 98.130% | 92.287% | 89.579% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.044% | 100.074% | 93.955% | 90.239% |
| Poznan_Hall2 | 0.000% | 0.095% | -0.382% | -0.015% | -0.027% | -0.027% | 97.711% | 95.987% | 91.872% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 97.990% | 94.343% | 87.596% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 97.798% | 90.857% | 82.836% |
| Shark | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 98.870% | 93.391% | 86.605% |
| 1024x768 | 0.000% | 0.000% | -0.084% | -0.008% | -0.173% | -0.002% | 98.747% | 93.710% | 90.816% |
| 1920x1088 | 0.000% | 0.019% | -0.076% | -0.003% | -0.005% | 0.004% | 98.489% | 93.707% | 87.830% |
| average | 0.000% | 0.012% | -0.079% | -0.005% | -0.068% | 0.001% | 98.586% | 93.708% | 88.950% |

Table 4. Performance of the proposed method II in AI

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.000% | 0.000% | 0.000% | -0.113% | 0.199% | 99.100% | 98.953% | 96.798% |
| Kendo | 0.000% | 0.000% | 0.000% | 0.000% | -0.154% | 0.324% | 99.193% | 99.530% | 99.376% |
| Newspaper_CC | 0.000% | 0.000% | 0.000% | 0.000% | -0.099% | 0.139% | 98.629% | 99.957% | 98.243% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 98.644% | 95.098% | 87.378% |
| Poznan_Hall2 | 0.000% | 0.000% | 0.000% | 0.000% | -0.034% | 0.554% | 99.815% | 98.319% | 95.754% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.623% | 98.093% | 97.721% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.698% | 100.359% | 100.724% |
| Shark | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 99.779% | 107.224% | 105.229% |
| 1024x768 | 0.000% | 0.000% | 0.000% | 0.000% | -0.122% | 0.220% | 98.974% | 99.480% | 98.139% |
| 1920x1088 | 0.000% | 0.000% | 0.000% | 0.000% | -0.007% | 0.111% | 99.512% | 99.819% | 97.361% |
| average | 0.000% | 0.000% | 0.000% | 0.000% | -0.050% | 0.152% | 99.310% | 99.692% | 97.653% |

# Proposed Text Modifications

The followings shall be updated in the working draft document to denote the proposed methods.

## Proposed method I

- I.8.4.4.1	General decoding process for intra blocks

- …

      - If DltFlag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_CPREDTEX, INTRA_PLANAR, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

          - idx = DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ]		(Error: Reference source not found-1)

          - SL[ xTb0 + i ][ yTb0 + j ] = 
	Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] − 1, idx ) ]	(Error: Reference source not found-2)

      - Otherwise, the following applies:

## Proposed method II

- I.8.4.4.1	General decoding process for intra blocks

- …

      - If DltFlag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_CPREDTEX, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

          - idx = DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ]		(Error: Reference source not found-3)

          - SL[ xTb0 + i ][ yTb0 + j ] = 
	Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] − 1, idx ) ]	(Error: Reference source not found-4)

      - Otherwise, the following applies:

# Conclusions

This contribution proposes harmonization of utilizing depth lookup table for intra prediction modes for depth intra coding. The proposed method reaches up to -0.068% and -0.050% for CTC and all-intra configuration.

# References

- H. Liu, J. Jia, “CE 6.h related: Applying Depth Look-up Table to Intra Modes of Depth Map,” JCT3V-E0157, Vienna, AT, July 2013.

- J. Heo, J. Jia, and S. Yea, “CE5: Fast depth lookup table application method to intra modes for depth data,” JCT3V-F0159, Geneva, CN, Oct 2013.

- J. Y. Lee, M. W. Park, B. Choi, Y. Cho, and C. Kim, “3D-CE5 related: Depth quantization in intra residual domain,” JCT3V-G0103, San Jose, US, Jan 2014.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).