| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0247 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0206 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On frame-field related indications (follow-up of parts of JCTVC-Q0183/JCT3V-H0082 and JCTVC-Q0078/JCT3V-H0026) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution responds to the frame-field indication related issues that were identified in JCTVC-Q0183/JCT3V-H0082 and JCTVC-Q0078/JCT3V-H0026.

A new SEI message is proposed to resolve the ambiguity to convey the frame-field related information for a layer set that contains layers of different frame-field information.

A sub-proposal of JCTVC-Q0078/JCT3V-H0026 (item 3 of Section 2.7) is repeated, in which

- layer_progressive_source_flag and layer_interlaced_source_flag are included in the SPS when profile_tier_level( ) is not present in the SPS.

- The constraints on and the inference of the value of frame_field_info_present_flag (in SPS VUI) is derived on the basis of general_progressive_source_flag and general_interlaced_source_flag, if they are present in the SPS, and on the basis of layer_progressive_source_flag and layer_interlaced_source_flag, otherwise.

# Problem Statement

The following paragraphs quote the frame-field indication related issues that were identified in JCTVC-Q0183/JCT3V-H0082 and JCTVC-Q0078/JCT3V-H0026.

## JCTVC-Q0183/JCT3V-H0082

According to the current SHVC/MV-HEVC specifications, particularly the syntaxes and semantics of the picture timing SEI message and the scalable nesting SEI message, when there is more than one layer in a layer set, within an AU, the values of the syntax elements pic_struct, source_scan_type, and duplicate_flag for all pictures in the AU have to be the same, restrictively. This is due to that:

- The field-frame information and the picture timing information are both included in the picture timing SEI message.

- It is specified that, when a picture timing SEI message is nested, it shall apply to one or more operation point, and a non-nested timing SEI message applies to the operation point corresponding to the entire bitstream. In other words, the field-frame information cannot be nested or non-nested in a way that different information can apply to different layers.

That means, for each layer set, all pictures in an AU must be either all frame pictures or all field pictures. Is this intended? If not, what to do with it?

## JCTVC-Q0078/JCT3V-H0026

Presently, it is asserted that the use of indications related to source scanning type (progressive or interlaced) and picture type (frame or field) in MV-HEVC/SHVC is unclear, because:

- general_progressive_source_flag and general_interlaced_source_flag are included in the profile_tier_level( ) syntax structure. In MV-HEVC/SHVC, the profile_tier_level( ) syntax structure is associated with an output layer set. Yet, the semantics of general_progressive_source_flag and general_interlaced_source_flag refer to the CVS – which supposedly means all layers, not just the layers of the output layer set which the profile_tier_level( ) syntax structure is associated with.

- In the absence of SPS VUI, general_progressive_source_flag and general_interlaced_source_flag are used to infer the value of frame_field_info_present_flag, which specifies whether the pic_struct, source_scan_type, and duplicate_flag syntax elements are present in the picture timing SEI messages. However, general_progressive_source_flag and general_interlaced_source_flag are absent in SPSs with nuh_layer_id greater than 0, so it is unclear which profile_tier_level( ) syntax structure is in the inference of general_interlaced_source_flag.

It is noted that one of the envisioned types of scalability for SHVC is interlaced-to-progressive scalability. Consequently, it may be common to have interlaced source content represented by the base layer and progressive source content represented by the enhancement layer.

# Overview of the proposals

Section 2.1 proposes a new SEI message to resolve the ambiguity to convey the frame-field related information for a layer set that contains layers of different frame-field information.

Section 2.2 repeats a sub-proposal of JCTVC-Q0078/JCT3V-H0026 (item 3 of Section 2.7), in which

- layer_progressive_source_flag and layer_interlaced_source_flag are included in the SPS when profile_tier_level( ) is not present in the SPS.

- The constraints on and the inference of the value of frame_field_info_present_flag (in SPS VUI) is derived on the basis of general_progressive_source_flag and general_interlaced_source_flag, if they are present in the SPS, and on the basis of layer_progressive_source_flag and layer_interlaced_source_flag, otherwise.

The detailed proposal is indicated with change marks in the attached specification text document.

## pic_struct, source_scan_type and duplicate_flag for layered media

If the picture timing SEI message is nested in a scalable nesting SEI message, the semantics of pic_struct, source_scan_type, and duplicate_flag are unspecified.

The following syntax and semantics of the frame-field information SEI message are proposed:

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| ffinfo_pic_struct | u(4) |
| ffinfo_source_scan_type | u(2) |
| ffinfo_duplicate_flag | u(1) |
| } |  |

The frame-field information SEI message may be used to indicate how the associated picture should be displayed, the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

When a non-nested frame-field information SEI message is included in an SEI NAL unit with nuh_layer_id assocLayerId, it is associated to the picture within nuh_layer_id equal to assocLayerId within the same access unit as the SEI NAL unit. When a frame-field information SEI message is included in a scalable nesting SEI message, it is associated to each picture with nuh_layer_id assocLayerId equal to nestingLayerIdList[ i ] for all values of i in the range of 0 to nesting_num_layers_minus1, inclusive.

When a picture with nuh_layer_id equal to assocLayerId is present in an access unit, frame_field_info_present_flag is equal to 1 in the active SPS for the layer with nuh_layer_id assocLayerId, and a non-nested picture timing SEI message is not present in an SEI NAL unit with nuh_layer_id equal to assocLayerId, a frame-field information SEI message associated with nuh_layer_id equal to assocLayerId shall be present in the access unit.

The semantics of ffinfo_pic_struct, ffinfo_source_scan_type and ffinfo_duplicate_flag apply layer-wise to each value of assocLayerId.

ffinfo_pic_struct has the same semantics as the pic_struct syntax element in the picture timing SEI message.

ffinfo_source_scan_type has the same semantics as the source_scan_type syntax element in the picture timing SEI message.

ffinfo_duplicate_flag has the same semantics as the duplicate_flag syntax element in the picture timing SEI message.

## Indications of source scan type and inference of frame_field_info_present_flag

The following items, which were originally presented in item 3 of Section 2.7 of JCTVC-Q0078/JCT3V-H0026, are proposed:

- layer_progressive_source_flag and layer_interlaced_source_flag, which are present in the SPS when profile_tier_level( ) is not present in the SPS. These syntax elements specify the source scanning type similarly to how general_progressive_source_flag and general_interlaced_source_flag in the SPS with nuh_layer_id equal to 0 specify the source scanning type for the base layer.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } else { |  |
| layer_progressive_source_flag | u(1) |
| layer_interlaced_source_flag | u(1) |
| } |  |

- When general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag appear in an SPS, they apply to the pictures to which the SPS is an active SPS. (This item has already been agreed as part of JCTVC-Q0078/JCT3V-H0026.)

- When general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag appear in a profile_tier_level( ) syntax structure associated with an output layer set, they apply to output layers and alternative output layers, if any, of the output layer set. (This item has already been agreed as part of JCTVC-Q0078/JCT3V-H0026.)

- The constraints on and the inference of the value of frame_field_info_present_flag (in SPS VUI) is derived on the basis of general_progressive_source_flag and general_interlaced_source_flag, if they are present in the SPS, and on the basis of layer_progressive_source_flag and layer_interlaced_source_flag, otherwise.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).