| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0223_v2 |
| --- | --- |

| Title: | BoG report on High Level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met 27 March 2014.

The BoG met again on 28 March 2014.

# Recommendations

The BoG recommends adopting the following:

- JCTVC-Q0108 Proposal #1: Remove restriction on alignment of TSA and STSA with dependent layers

- JCTVC-Q0108 Proposal #2: Allow STSA pics of non-base layers to have TemporalId equal to 0

- JCTVC-Q0158 Proposal #2: Change range of values for HighestTid in Annex C to reduce number of encoder conformance tests

- JCTVC-Q0146 Proposal 1.1: Change inference rule in semantics of poc_msb_val_present_flag

- JCTVC-Q0146 Proposal 1.4, Add an inference value of 0 when poc_reset_info_present_flag is not present

- JCTVC-Q0060 Proposal 2: Add a condition to the derivation of refLayerPicIdc of (TemporalId == 0)

- JCTVC-Q0165 proposal 1.a), add a constraint to disallow an empty layer set

- JCTVC-Q0165 proposal 2. change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.

- JCTVC-Q0195 proposal 1: restructure syntax table for sending of rep_format_idx_present_flag and vps_num_ref_formats_minus1

- JCTVC-Q0195 proposal 2: add a semantic constraing on the value of update_ref_format_flag

- JCTVC-Q0177 proposal 1: modify inference rule for sps_temporal_id_nesting_flag when it is not present

- JCTVC-Q0177 proposal 2: clarification of description of end of sequence NAL unit

The BoG recommends the following activities take place:

- Revisit JCTVC-Q0105 in track, “On temporal enhancement layers and diagonal inter-layer prediction”

- Revisit JCTVC-Q0166 in track, “On scaled reference layer offset”

- Revisit JVTVC-Q0170 in track, “Resampling need for a scalable layer”

The BoG refers the following to the editors:

- Several editorial issues regarding HRD operation from JCTVC-Q0055.  Review text modifications with contribution author.

- Editorial improvements are needed for the association of NAL units within an access unit and access unit boundary detection.  Language in JCTVC-0091 may be useful to study.

- JCTVC-Q0146, proposal 1.2, consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described, and proposal 1.3 specification bug fix

- JCTVC-Q0054 proposal #2, bug fix in the semantics of vps_profile_present_flag[ ]

# Document review details

## 7.5.1 Generic HLS issues

JCTVC-Q0055 MV-HEVC/SHVC HLS: Comments on general decoding process and selection of CPB operation in the HRD operation [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

This contribution proposes modifications and improvements of texts in the subclause 8.1 General decoding procedure and C.1 General of Annex C with the following items:

First proposal: Re-definition of ‘output layer’

Refer to editors

Second Proposal: to substitute ‘TargetOptLayerSetIdx’ with ‘TargetOutputLayerSetIdx’

Seems to be an editorial bug. Refer to editors.

Third Proposal; Modification of TargetOutputLayerSetIdx setting procedure

Refer to editors to improve C.1 description of setting the value of TargetOutputLayerSetIdx

Fourth Proposal: Modification of HighestTid setting procedure

Refer to editors.  Suggest to introduce a variable which is set to vps_max_sub_layers_minus1 when nuh_layer_id > 0.  (Base layer uses SPS value, other layer use VPS value)

Fifth Proposal: Modification of NoRaslOutputFlag setting procedure

Proposal seems to change technical behavior.  No action.

Sixth proposal: Modification of selecting CPB operation for a bitstream conformance test

Refer to the editors to clarify this section regarding “both” bitstream specific and bitstream partition specific conformance

If possible, desirable to have editorial changes made during this meeting, and reviewed by the proponent.

JCTVC-Q0091 MV-HEVC/SHVC HLS: On access unit definition and allowing different decoding orders in different layer trees [M. M. Hannuksela (Nokia)]

It is asserted that it may be desirable to allow different decoding orders of pictures in different layer trees to provide flexibility for encoders to use different prediction structures in different layer trees and to enable a leaner sub-DPB capacity requirement for some layer trees, such as auxiliary picture layers.

The following aspects are proposed regardless of whether the access unit is specified as proposed in this contribution:

- Definition of a layer tree (which, when phrased in a simplified manner, consists of the layers that have dependency relations with each other).

- Allocation of layers to sub-DPBs in a manner that different layer trees are allocated to different sub-DPBs. Consequently, the extraction of a layer tree does not affect the sub-DPB parameters or operation of another layer tree even if the layer trees would have the same resolution, bit-depth and chroma format.

Revisit after other related topics are reviewed.

Additionally, the contribution proposes the following items related to relaxing the access unit definition:

- Specification of the order of NAL units and coded pictures and association to access units for MV-HEVC/SHVC.

Such specification has been missing so far in MV-HEVC/SHVC, which have instead referred to HEVC version 1 specification text, which is asserted to be wrong or incomplete in many aspects for MV-HEVC/SHVC.

- Relaxation of the access unit definition so that an access unit is required to include coded pictures associated with the same output time and belonging to the same layer tree. When the bitstream has multiple layer trees, an access unit may but is not required to include coded pictures associated with the same output time and belonging to different layer trees.

- A VPS VUI flag cross_layer_dec_order_aligned_flag. When the flag is equal to 1, access units are constrained to include all coded pictures associated with the same output time.

- A bitstream conformance constraint that pictures associated with the same output time shall be within the same POC resetting period and shall have the same value of PicOrderCntVal. This constraint is needed to make picture output order of the output order conforming decoder the same as that of the output timing conforming decoder.

Aspects a-b and 1-4 can be considered separately.

It was asserted that the proposal gives encoders more flexibility, and that it could allow more efficient sub-DPB optimization.

A participant questioned what interaction this proposal would have with systems specifications.  It was suggested that independent layer trees would need to be placed in different tracks.

Access unit boundary detection might be made more complex, but not clear.

A question was raised about interaction with SEI message persistence and scalable nesting SEI messages.

Refer to editors: It was noted that editorial improvements are needed for the association of NAL units with an access unit, and access unit boundary detection, independent from this proposal.  The language in this contribution may be useful to study.

JCTVC-Q0105 MV-HEVC/SHVC HLS: On temporal enhancement layers and diagonal inter-layer prediction [M. M. Hannuksela (Nokia)]

This contribution asserts that diagonal inter-layer prediction would be useful in the following use cases:

- An enhancement layer provides a temporal enhancement, possibly along with spatial or quality enhancement, relative to the base layer, where the picture rate ratio is non-dyadic, e.g. a 24-Hz base layer, a 30-Hz first enhancement layer, and a 120-Hz second enhancement layer.

- An SHVC-coded temporal enhancement layer is provided for an AVC or HEVC base layer.

- Interlaced-to-progressive scalability, when the base layer contains coded frames.

It is proposed to enable the use of diagonal inter-layer reference pictures as summarized in the following:

- A gating flag in SPS multilayer extension specifies if diagonal inter-layer prediction is enabled in the slice header level.

- A reference-layer short-term RPS syntax structure can be included in the slice header for a direct reference layer. The reference-layer short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the reference-layer short-term RPS syntax structure for the current picture.

New syntax structure is proposed.  It was noted that the earlier proposal in JCTVC-P0141 was simpler and involved less change to the specification.

Revisit in track, with updated contribution to also provide simpler option.

JCTVC-Q0108 MV-HEVC/SHVC HLS: On TSA and STSA pictures [M. M. Hannuksela (Nokia)]

The contribution includes the following proposals:

- It is proposed to remove the constraints on aligning TSA and STSA pictures within an access unit due to the following asserted reasons:

  - The alignment constraints do not guarantee that sub-layer up-switching would be possible across all layers of the target output layer set, when a base-layer TSA or STSA picture at the desired switch-to sub-layer appears. In other words, layer-wise consideration of sub-layer up-switching is needed in any case.

  - Without the alignment constraints, it would be possible to adapt the highest transmitted TemporalId of the highest layer separately from other layers and hence achieve finer-grain bitrate adjustment.

  - Without the alignment constraints, reference pictures for inter prediction can be selected differently across layers, allowing some layers to be coded with more frequent sub-layer up-switch points and some layers with a larger number of reference pictures for inter prediction (aiming at better rate-distortion performance).

- It is proposed to allow STSA pictures with nuh_layer_id greater than 0 to have TemporalId equal to 0. Such STSA pictures enable layer up-switching where a subset of sub-layers of the highest layer can be decoded starting from the up-switch point. It is asserted that the proposed change does not affect the decoding process.

Agreed to remove restriction from #1, to no longer require TSA or STSA alignment of dependent layers with the current layer.

Agreed to remove restriction from proposal #2, to allow STSA pics of non-base layers to have TemporalId equal to 0.

JCTVC-Q0109 MV-HEVC/SHVC HLS: On TemporalId constraints [M. M. Hannuksela (Nokia)]

This contribution proposes that pictures with different TemporalId values are allowed in the same access unit. It is asserted that the following use cases or benefits are achieved with the proposal:

- It provides encoders flexibility in determining prediction hierarchies differently across layers, allowing some layers to be coded with a greater number of TemporalId values and frequent sub-layer up-switch points and some layers with a prediction hierarchy aiming at a better rate-distortion performance.

- It provides flexibility to encode layer trees of the same bitstream independently from each other.

A question was raised of interaction with SEI messages.

No action.

JCTVC-Q0158 MV-HEVC/SHVC HLS: On Highest TemporalId [S. Deshpande (Sharp)]

In this document:

- In proposal 1 it is proposed to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList. It is proposed to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause F.8.1.4.1 based on the derived HighestTemporalIdList during the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.4.

It is asserted that without this proposed modification currently in a mixed frame rate and mixed number of temporal sub-layers scenario the marking process for sub-layer non-reference pictures not needed for inter-layer prediction in F.8.1.4.1 will not get invoked for layers which have lower frame rate and lower number of temporal sub-layers as the highest TemporalId value in those layers is less than HighestTid value.

- In proposal 2 a modification is proposed to the range of values for HighestTid in Annex C.

Proposal 1 asserts efficiency improvement for sub-DPB optimization.  One pic in the DPB could potentially be saved under particular use cases – an example was given of 3 layers, with 30 fps, 30fps, and 60 fps of the 3 layers. It was suggested that there would only be a benefit for shared sub-DPB.

Some non-proponent support for proposal 1 if shared sub-DPB is kept.  Revisit once shared sub-DPB decision is made.  Offline discussion about text for handling an additional use case.

Proposal 2 is asserted to reduce the number of tests than an encoder has to make for conformance.  Agreed to Proposal 2.

JCTVC-Q0163 MV-HEVC/SHVC HLS: On decoding non-output/non-reference layers [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution proposes to modify derivation process of the layer identifier list TargetDecLayerIdList to exclude decoding of non-output / non-reference layers.

It is asserted that the proposal is beneficial to provide a capability to ignore unnecessary layers for target output layers in decoding process. Considering base layer existence is set to be inevitable at any layer sets regardless of base layer usage, the proposed modification is assertively critical to signal the actual HDR parameter that depends on decoding process.

In the revision 2, Option 1 and Option 2 are updated.

It was suggested that JCTVC-Q0078 is related.

Need to consider the larger question about should a conformance point be defined which doesn’t contain a backwards compatible base layer, and if there should be a normatively defined re-writing process for an independent enhancement layer to be rewritten as a base layer.

Option 2 changes the sub-bitstream extraction process, but Option 1 does not.

Contribution specification text appears to be incomplete, because other areas would be affected.

Revisit after review of related contributions and discussion of larger questions.

### 7.5.2 IRAP alignment and POC derivation (2X)

JCTVC-Q0057 MV-HEVC/SHVC HLS: Comments on IRAP alignments and POC value derivation [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, S. Lee, C. Kim (Samsung)]

- Several restrictions on the IRAP picture alignment and the POC value derivation are proposed. With the proposed restrictions for IRAP alignments, the behaviour of POC value derivation can be simplified.

- In the first proposal, a flag layerwise_start_up_disable_flag disabling a layerwise start-up is proposed.  It was questioned whether this was proposing a change in the decoding process or a constraint on the bitstream.  The proposal text doesn’t describe a change in decoding process. No action.

- In the second proposal, when all_layers_idr_aligned_flag is equal to 1, vps_poc_lsb_aligned_flag shall be equal to 1 and poc_reset_info_present_flag shall be equal to 0.  Spec text was not provided.  No action.

- In the third proposal, replacing the restriction that “the value of poc_reset_idc of all pictures in an access unit shall be the same” with “the value of poc_reset_idc of all slices in a layer shall be the same” is proposed.  It was questioned if the proposal would impact error resilience.   No action.

- In MV-HEVC profile (stereo profile), cross_layer_pic_type_aligned_flag and cross_layer_irap_aligned_flag are equal to 1 by default.  No action.

JCTVC-Q0146 MV-HEVC/SHVC HLS: On picture order count and related [A. K. Ramasubramonian, Hendry, Y.-K. Wang]

This document proposes several changes to the semantics of syntax elements related to POC, and also proposes a constraint to disallow an enhancement layer RASL picture of a CRA picture to refer to pictures that precede, in decoding order, a POC resetting picture that precedes the associated CRA picture in decoding order and that is in the same layer as the CRA picture.

Proposal 1.1 changes semantics of poc_msb_val_present_flag, inference rule when not present.  Agreed.

Proposal 1.2 proposes a semantic constraint.  It was suggested that if a splicer removed the RASL pictures associated with the IRAP pic at the upswitch point, this constraint would not be necessary.  Refer to editors to consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described.  JCTVC-Q0177 may be related.

Proposal 1.3 spec bug fix, refer to editors.

Proposal 1.4 proposes an inference value of 0 when poc_reset_info_present_flag is not present.  Agreed.

### 7.5.3 RPS signalling and derivation (3)

JCTVC-Q0060 MV-HEVC/SHVC HLS : On inter-layer RPS signalling and derivation [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes two changes for MV-HEVC and SHVC related to inter-layer RPS signalling in the slice segment header and derivation process. The first change targets to improve the inter-layer RPS signalling in the slice segment header by taking into account the number of direct reference layers presented in the same access unit as the current picture, which is indicated by the ‘numRefLayerPics’, instead of the number of direct reference layers indicated by the ‘NumDirectRefLayers’. The second change targets to improve the derivation process of ‘numRefLayerPics’ and ‘refLayerPicIdc’ to support the lightweight option scenario in that syntax element ‘max_tid_il_ref_pics_plus’ is set equal to 0.

In r1 revision, the simplified version is added for Item 2 and comments on JCTVC-Q0079, which includes the conceptually same proposals as this, are added.

Proposal 1 could provide some minor bit savings in the slice segment header under certain conditions.  JCTVC-Q0079 is related.

Proposal 2 adds a condition to the derivation of the value of refLayerPicIdc.  Agreed to add a condition with (TemporalId == 0). Also proposed as option 1 in JCTVC-Q0079.

JCTVC-Q0079 MV-HEVC/SHVC HLS: max_tid_il_ref_pics_plus1 in inter-layer RPS syntax and semantics [M. M. Hannuksela (Nokia)]

The contribution proposes changes for the following two asserted problems or shortcomings:

- When max_tid_il_ref_pics_plus1[ ][ ] is equal to 0 (i.e., when only the IRAP pictures of the reference layer may be used as reference for inter-layer prediction), the index of the reference layer is not included in the list of layer index values of the potential inter-layer reference pictures, refLayerPicIdc[ ], even though it should be included for a correct operation.

- max_tid_il_ref_pics_plus1[ ][ ] is not used in the inter-layer RPS syntax and semantics to prune the potential reference layers for inter-layer prediction.

Two options are proposed, where option 1 is asserted to fix the asserted bug only and option 2 is asserted to fix both issues. The proponent prefers option 2.

See notes for JCTVC-Q0060.

Could potentially gain a few bits per slice under certain conditions, but no estimate of gains.

JCTVC-Q0100 MV-HEVC/SHVC HLS: Constraints for Reference Picture Set Parameters [S. Deshpande (Sharp)]

In this document bitstream constraints are proposed for short-term reference picture set parameters and long-term reference picture set parameters for SHVC and MV-HEVC. These bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameters signalled in VPS which are used for the DPB operation.

It seems that the existing constraints are not clear for the layered extensions, and should be addressed.  Revisit after offline discussion and refinement of proposed text.

### 7.5.5 Parameter sets (5)

JCTVC-Q0054 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

Modifications of semantics of dimension_id_len_minus1[ j ] and vps_profile_present_flag[ i ] are proposed.

In the first proposal to modify Semantics of dimension_id_len_minus1, no action.

- In the second proposal, refer to editors a bug fix in the semantics of vps_profile_present_flag[ i ]

JCTVC-Q0061 MV-HEVC/SHVC HLS: On max_tid_il_ref_pics_plus1 in the VSP extension [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes the extra syntax elements to indicate the default value for syntax element “max_tid_il_ref_pics_plus1” to remove signalling redundancy.

This change could save some bits in the VPS for the case when max tid is used, but the same value was used for all sub-layers.  Two options proposed.

It was suggested that we should be concentrating on design stability and bug fixes at this stage rather than optimizations.

JCTVC-Q0110 Support for out-of-band signaling in VPS to enable future layer additions [S. Narasimhan, A. Luthra (Arris)]

Presenter not available.

JCTVC-Q0165 MV-HEVC/SHVC HLS: Clean up for output layer set [T. Tsukuba, T. Yamamoto, T. Ikai, S. Deshpande (Sharp)]

This contribution presents several proposals regarding output layer set as follows:

- Prohibits 1) the case no output layer exists in an output layer set, and 2) duplicated output layer sets

- Clean up signaling condition related to num_add_output_layer_sets and default_target_output_layer_set.

- Add a flag, ptl_dpb_info_present_flag, indicating whether or not to explicitly signal profile_tier_level_idx and DPB parameters for additional output layer sets

In revision 1, the proposed text for proposal 1 is updated. In addition, bit-estimation analysis on proposal 3 is added. According to the analysis, it is asserted that proposal 3 can reduce bits with negligible overhead.

In revision 2, the proposed texts for proposal 3 is updated.

In revision 3, the proposed text for proposal 3 is further updated.

Proposal 1.a) Add a constraint to disallow an empty output layer set. Agreed.

Proposal 1.b) Proposes to add a constraint to disallow duplicated output layer sets.   No action.

Proposal 2.  Change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.  Seen as a more logically consistent design.  Agreed.

Proposal 3.  Could provide bitrate savings.  Use case is not clear.  No action.

JCTVC-Q0211 MV-HEVC/SHVC HLS: On HighestTid and MaxSubLayersInLayerSetMinus1 [T. Ikai (Sharp)] [late]

The contribution proposes a modification related to maximum sub-layer value, called MaxSubLayersInLayerSetMinus1[ i ], in the output layer set which is used to signals sub-layer base DPB parameters and restrict HighestTid in the range of 0 and MaxSubLayersInLayerSetMinus1[ i ]. The modification uses maximum sub-layer value of output layers instead of maximum sub-layer value of decoding layers. It is asserted that the modification are useful to reduce DPB size and remove unnecessary DPB parameters.

It was questioned if there is an interaction with the alternate output layer process.  It was noted that this was a late contribution, so little opportunity for study in advance of meeting.

The proposal could save some bits in the VPS in certain conditions.

JCTVC-Q0195 MV-HEVC/SHVC HLS: On representation format signaling [S. Hattori, O. Nakagami, T. Suzuki (Sony)] [late]

This contribution discusses representation format (rep_format) signaling in VPS extension.

First, the default mapping between the rep_format and layer is discussed. Currently, this happens when rep_format_idx_present_flag is equal to 0. In this case, the number of rep_formats is constrained to be equal to the number of layers (MaxLayersMinus1). In the contribution, more flexible method is studied and proposed to release this constraint.

Second, a case in which vps_num_rep_formats_minus1 is equal to 0 is discussed. The contribution proposes a clean-up signaling vps_num_rep_formats_minus1. The contribution indicates that the update in SPS extension by update_rep_format_flag is redundant in this case.

The contribution provides comparison of signalled parameter in the bitstream with examples which are based on the use-cases from CTC in MV-HEVC and SHVC.

For proposal 1, it is asserted that the new design is cleaner and easier to understand.  Agreed to restructure rep_format_idx_present_flag and vps_num_ref_formats_minus1.

For proposal 2, a semantic constraint on the value of update_ref_format_flag.  Agreed.

### 7.5.7 Miscellaneous HLS topics (8)

JCTVC-Q0142 MV-HEVC/SHVC HLS: On extraction of independent non-base layer [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

This contribution asserts the presence of two problems related to the semantics of poc_lsb_not_present_flag[ i ] and the use of long-term reference pictures. The asserted problems can be summarized as follow:

- When poc_lsb_not_present_flag[ i ] is equal to 1 and the IDR picture in layer i is in the same access unit with a base layer picture that has slice_pic_order_cnt_lsb not equal to 0, the bitstream may become non-conformant as the derived picture order count (POC) for that IDR may be different from other pictures in the same access unit.

- When an independent non-base layer is extracted for input for HEVC decoder, if pictures in layer uses long-term reference picture (LTRP) and the original bitstream has non-cross layer aligned IDR, a problem may occur such that HEVC decoder cannot locate the correct LTRP.

The contribution also asserts that the proposed constraints to the semantics of poc_lsb_not_present_flag[ i ] and base_layer_parameter_set_compatibility_flag[ i ] would solve the problems above.

Regarding proposal 1, it was suggested to revise the language to make it more clear that if IDR alignment is indicated, the constraint does not apply.  Revisit after offline discussion.

Regarding proposal 2, it was suggested that the proposed language was overly restrictive.  It was also questioned if the functionality of the existing VUI indictor flag should be expanded to include a general indication of rewritability, rather than just the parameter set aspect.  Revisit.

JCTVC-Q0177 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [Hendry, A. K. Ramasubramonian, Y.-K. Wang, V. Seregin (Qualcomm)] [late]

This document proposes several changes to the current MV-HEVC/SHVC design. The proposed changes include the following:

- In proposal 1, Modifying of the inference rule for sps_temporal_id_nesting_flag when it is not present.  This is seen as an editorial bug.  Agreed.

- In proposal 2, Clarification of the description related to the end of sequence NAL unit and the end of bitstream NAL unit.  It was suggested the behavior for how a decoder handles an EOS with layer id > 0 should be disallowed in this version of the spec.  Agreed, subject to text revision to address this suggestion.

- In proposal 3, Signalling of the presence of discardable pictures in the bitstream.  Proposes to add a flag in the slice segment header, and a gating flag in the VPS.  It is asserted to save bits in some cases, but no numbers were provided.  No action.

JCTVC-Q0102 MV-HEVC/SHVC HLS: Comments on HEVC Extensions [S. Deshpande (Sharp)]

Only proposals 3 and 4 belong to this category.

- Proposal 3: It is proposed to signal the bit rate and picture rate information only up to the maximum temporal sub-layers in the corresponding layer set.

- It was suggested that an existing variable could be used, MaxSubLayersInLayerSetMinus1. Some non-proponent support.  Revisit after revised text with that change is available.

In r1 revision following proposal 4 is added:

- Proposal 4: A modification to the derivation of numRefLayerPics, and refLayerPicIdc[ i ] to handle discardable pictures is proposed.

It is asserted that the bits could be saved in some cases, when there are discardable pics in reference layers.  No bit rate savings numbers are provided.  No action.

JCTVC-Q0160 SHVC/MV-HEVC HLS: On alternative output layer flag [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes to introduce a gating flag for alt_output_layer_flag. The proposed change is asserted to reduce parsing complexity as well as to assure the extensibility of the usage of the alternative output layer. In addition, it is proposed to restrict usage of the alternative output layer in Stereo Main profile.

It was suggested the extensibility could alternatively be provided in an extension to the VPS extension.

Proposed to restrict stereo main profile from using the alt output layer.  Agreed in spirit, but using alternative text, which will be provided by M. Hannuksela.  Revisit to review provided text.

JCTVC-Q0166 MV-HEVC/SHVC HLS: On scaled reference layer offset [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes change on scaled reference offset information signalling. It is proposed to signal the offset information in both VPS extension and slice header extension.

Contribution did not provide decoding process modification text.  Some support. Revisit in track when text of semantics and decoding process is available.

JCTVC-Q0170 SHVC HLS: Resampling need for a scalable layer [K. Andersson, J. Samuelsson, R. Sjöberg, J. Ström]

Currently a decoder may according to information given on a high level e.g. VPS, draw wrong conclusions about the need for resampling process or not for decoding of a layer of a scalable video bitstream. The need for resampling process could be useful for a decoder to know since it varies the amount of actions for the decoder to perform.

We propose to add a layer wise syntax element to indicate if resampling process is needed for decoding of a layer of a HEVC extension bitstream. The indication can also distinguish between resampling for fractional and integer sample positions or if the need for resampling is unknown.

Proposes new syntax elements in the VUI.

Revisit in track to consider if the indication functionality is considered useful.

JCTVC-Q0189 MV-HEVC/SHVC HLS: On slice_temporal_mvp_enabled_flag [M. M. Hannuksela (Nokia)] [late]

JCTVC-Q0078 MV-HEVC/SHVC HLS: On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers [M. M. Hannuksela (Nokia)]