| Joint Collaborative Team on 3D Video Coding Extension of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0002 |
| --- | --- |

| Title: | JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-3V |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Gerhard Tech (Fraunhofer HHI) Krzysztof Wegner (Poznan Univ. of Tech.) Jill Boyce (Vidyo) Ying Chen (Qualcomm Incorporated) Teruhiko Suzuki (Sony) Sehoon Yea (LGE) Jens-Rainer Ohm (RWTH Aachen) Gary Sullivan (Microsoft) | Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl jill@vidyo.com cheny@qti.qualcomm.com teruhikos@jp.sony.com sehoon.yea@lge.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-3V ad hoc group on MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) between the 8th meeting in Valencia and the 9th JCT-3V meeting in Sapporo.

# Mandates

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) - Produce and finalize JCT3V-H1001 3D-HEVC Draft Text 4. - Produce and finalize JCT3V-H1002 MV-HEVC Draft Text 8. - Produce and finalize JCT3V-H1003 3D-HEVC and MV-HEVC Test Model 8. - Gather and address comments for refinement of these documents. - Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |

# Introduction

The fourth 3D-HEVC draft, the eighth Test Model of 3D-HEVC and MV-HEVC and the eighth MV-HEVC draft were developed from the third 3D-HEVC draft, the seventh 3D-HEVC Test Model and the seventh MV-HEVC draft, respectively, following the decisions taken at the 8th JCT-3V meeting in Valencia.

Three editorial teams were formed to work on the three documents that were to be produced:

- H1001 3D-HEVC Draft Text 4

- Gerhard Tech, Ying Chen, Krzysztof Wegner, Sehoon Yea

- H1002 MV-HEVC Draft Text 8

- Gerhard Tech, Miska Hannuksela, Ying Chen, Jill Boyce, Krzysztof Wegner

- Ye-Kui Wang, Adarsh Ramasubramonian

- H1003 3D-HEVC Test Model 8

- Ying Chen, Gerhard Tech, Krzysztof Wegner, Sehoon Yea

# Status

## 3D-HEVC Draft 4

Two versions of H1001 have been published by the editing AHG following the 8th JCT-3V meeting in Valencia, the last version has been submitted as study text to the MPEG secretariat. The 3D-HEVC draft text has been updated to be aligned with MV-HEVC draft 8.

### Incorporated changes

All adoptions of the last meeting have been incorporated. Most of the changes have been minor modifications. Existing text has been revised and improved.

Changes (23) of H1001 relative to G1001 are:

HLS (4)

- (3DN-08/JCT3V-H0091) EC Problem2: Add a constraint that the bit depth signaling in PPS should not contradict the value signaled in SPS. Decision: Adopt this aspect.

- (3DN-13/JCT3V-H0137) Control of the availability of advanced inter-view coding predictions Decision: Adopt (second part)

- (3DN-18/JCT3V-H0095) HLS: On SDC signaling Decision: Editors will put the encoder restriction..

- (3DN-23/JCT3V-H0204) BoG report on intra depth slice Decision: Adopt (H0204)

DMM (3)

- (3DN-02/JCT3V-H0092) CE3: Results on simplified DMM mode coding; Decision: Adopt H0092.

- (3DN-04/JCT3V-H0119) CE3: Simplification on CABAC contexts for the syntax related to depth intra mode. Decision: Adopt  solution from H0119 Test 2 and H0135

- (3DN-25/JCT3V-H0136) Constraints for depth modeling modes Decision(BF): Adopt.

SDC (3)

- (3DN-01/JCT3V-H0131) CE3: Delta DC coding for SDC and DMM modes. Decision: Adopt H0131 Test 2

- (3DC-19/Fix sdc_flag, intra_chroma_pred_mode) Added missing condition of intra_chroma_pred_mode parsing from sdc_flag.)

- (3DN-26/JCT3V-H0108) Clean-up on DMM and SDC DC prediction Decision: Adopt.

DBBP (5)

- (3DN-11/JCT3V-H0104) Partition boundary filtering in DBBP Decision: Adopt

- (3DN-14/JCT3V-H0072) Bug-fix of depth-based block partitioning Decision: Adopt (H0072, Method 2)

- (3DN-15/JCT3V-H0057) Sub-PU Restriction for DBBP Decision: Adopt

- (3DN-21/JCT3V-H0094) Improvement on the signaling of DBBP Decision: Adopt (Solution 1)

- (3DN-28/JCT3V-H0094) Editor's note regarding potentially missing part.

DLT (2)

- (3DN-09/JCT3V-H0105) DLT-based residual coding; Remove DLT based residual coding

- (3DN-20/JCT3V-H0091) On DLT signaling 1.) Encoder constraint to predict DLT only from existing reference. Decision: Adopt;

Others (6)

- (3DN-03/JCT3V-H0070) CE1: Results on Adaptive Disabling Inter-view Motion Vector Candidates; Decision: Adopt simplified version as per WD text in v2 of the contribution.

- (3DN-10/JCT3V-H0103) Vertical DV restriction after depth-based refinement Decision: Adopt.

- (3DN-16/JCT3V-H0077/H0099/H0111/H0133) Sub-PU MPI Decision: Adopt (H0077/H0099/H0111/H0133(Option 1) – all are identical

- (3DN-17/JCT3V-H0205) Disallow bi-prediction in case of 4x8 and 8x4 sub PU sizes Decision: Adopt (disallow B prediction in case of 4x8 and 8x4 sub PU sizes as provided in H0205)

- (3DN-27/JCT3V-H0062) CE1: Simplification of merge candidate construction Decision: Adopt

- (3DN-24/JCT3V-H0205) Editor's note regarding potentially missing part.

### Open issue

- A list of other minor issues is listed in the bug tracking system.

- Subsequent to the Valencia meeting, proponents of H0094 and H0205 provided revised specification text. However, additional changes have neither mentioned in the original input proposals nor in the meetings notes such that they have not been incorporated into the draft. Further clarification might be required since the changes have been integrated to the software.

## 3D-HEVC Test Model 8

One version of H1003 has been published by the editing AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

### Incorporated changes

All adoptions of the last meeting have been incorporated. Moreover existing text has been revised and improved and missing text from previous meeting has been added.

Changes (11) of H1003 relative to G1005 are:

- (3DN-01/JCT3V-H0077/JCT3V-H0099/JCT3V-H0111/JCT3V-H0133) Sub-PU MPI

- (3DN-02/JCT3V-H0070) CE1: Results on Adaptive Disabling Inter-view Motion Vector Candidates

- (3DN-03/JCT3V-H0062) CE1: Simplification of merge candidate construction

- (3DN-04/JCT3V-H0205) Disallow bi-prediction in case of 4x8 and 8x4 sub PU sizes and other relevant edits

- (3DN-05/JCT3V-H0057) Sub-PU Restriction for DBBP

- (3DN-06/JCT3V-H0072) Bug-fix of depth-based block partitioning

- (3DN-07/JCT3V-H0104) Partition boundary filtering in DBBP

- (3DN-08/JCT3V-H0103) Vertical DV restriction after depth-based refinement

- (3DN-09/JCT3V-H0105) Remove DLT based residual coding

- (3DN-10/JCT3V-H0086) Low-latency illumination compensation (IC) encoding algorithm (Non CTC)

- (3DN-11/JCT3V-H0108) Clean-up on DMM and SDC DC prediction

## MV-HEVC Draft Text 8

Five versions of H1002 were published by the AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

### Incorporated changes

All adoptions of the last meeting have been incorporated. Moreover existing text has been revised and improved and missing text from previous meeting has been added.

Changes (47) of H1004 relative to F1004 are:

Generic HLS issues (4)

- #12 (GEN/Q0108/TSA,STSA alignment) Agreed to remove restriction from 1, to no longer require TSA or STSA alignment of dependent layers with the current layer.

- #13 (GEN/Q0108/STSA TemporalId) Agreed to remove restriction from proposal 2, to allow STSA pics of non-base layers to have TemporalId equal to 0.

- #14 (GEN/Q0158/Highest Tid Annex C) In proposal 2 a modification is proposed to the range of values for HighestTid in Annex C. Proposal 2 is asserted to reduce the number of tests than an encoder has to make for conformance. Agreed to Proposal 2.

- #23 (GEN/Q0183/SEI clean-ups) numerous small clean-ups on SEI messages.

POC alignment and derivation (3)

- #15 (IRAP,POC/Q0146/inference of poc_msb_val_present_flag) Proposal 1.1: Change inference rule in semantics of poc_msb_val_present_flag

- #16 (IRAP,POC/Q0146/inference poc_reset_info_present_flag) Proposal 1.4, Add an inference value of 0 when poc_reset_info_present_flag is not present

- #32 (POC/Q0142/poc_lsb_not_present_flag) v2: Add semantic constraints to poc_lsb_not_present_flag.

Auxiliary pictures (4)

- #2 (AUX/Q0081/primary) Decision: Remove the constraint that an alpha picture must be accompanied by a primary picture.

- #39 (AUX/Q0078/conformance): mechanism for signaling a profile/tier/level conformance point for auxiliary pictures

- #40 (AUX/NODOC/primary pic) Clarify that an auxiliary picture can be associated with more than one primary picture. Consider if the language associating an alpha auxiliary picture with a primary picture in the semantics of dimension_id[ ][ ] near the AuxId derivation could be moved to the alpha SEI message.

- #44 (AUX/Q0078/concepts) Auxiliary picture concepts:

Profiles (4)

- #31 (PROF/NODOC/Profile constraint) Add a profile constraint to the Scalable Main, Scalable Main 10, and Stereo Main profiles against allowing layers with duplicate values of DependencyId (or ViewOrderIdx) when AuxId equal to 0.

- #33 (PROF/Q0160/alt_output_flag) v2: Add constraint to stereo main profile that it must contain exactly two texture views, and add a note to state that the constraint implies a restriction that alt_output_flag equal to 0.

- #45 (PROF/H0126/Stereo main) Phrasing used in specifying the Stereo Main profile.

- #50 (PROF/Monochrome) Add Monochrome 8-bit profile

Parameter sets (6)

- #18 (PS/Q0165/disallow an empty layer set) proposal 1.a), add a constraint to disallow an empty layer set

- #19 (PS/Q0165/presence of num_add_output_layer_sets) proposal 2. change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.

- #20 (PS/Q0195/syntax table rep format) proposal 1: restructure syntax table for sending of rep_format_idx_present_flag and vps_num_ref_formats_minus1

- #21 (PS/Q0195/constraint update_ref_format_flag) proposal 2: add a semantic constraint on the value of update_ref_format_flag

- #48 (SPS/PPS extension cleanups) Alignment with RExt

- (PPS/reserved flag): Add a flag in PPS for SHVC color gamut scalability

Hypothetical reference decoder (HRD) (8)

- #4 (HRD/Q0101/Bitstream part buffer) On Bitstream Partition Buffer. Decision (BF/Cleanup): Adopt (sub-proposals 1–11, refined as described).

- #5 (HRD/Q0182/Bitstream part buffer) Decision (BF/Cleanup/Ed): Adopted (such that we use the main proposal for sub-proposal 1, and alternative 1 for sub-proposal 2).

- #6 (HRD/Q0182/hrd_parameters) Sub-proposal 2 Alternative 1: Clarify that the VPS hrd_parameters( ) syntax structure that applies to the layer set which is associated with the bitstream partition initial arrival time SEI message is used to determine the lengths of the nal_initial_arrival_delay[ i ] and vcl_initial_arrival_delay[ i ] syntax elements. Decision (BF/Cleanup/Ed): Adopted alternative 1 for sub-proposal 2

- #8 (HRD/Q0102/sps_max_dec_pic_buffering_minus1) Sub-proposal 2: A semantics bug fix is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix. In discussion, the first option was preferred. Decision (BF/Cleanup/Ed.): Adopt.

- #9 (HRD/Q0102/NoOutputOfPriorPicsFlag) It was suggested that also the separate_colour_plane_flag should affect inference of NoOutputOfPriorPicsFlag. Decision (Ed.): Agreed (affects RExt text).

- #10 (HRD/Q0154/DPB Flushing and parameters) On picture flushing and DPB parameters Decision: Adopted (some details to be discussed further in BoG).

- #38 (DBP/Q0154/VPS DPB) Proposal in C.5.2.1: Add in the decoding process that when a new VPS is activated, all pictures in the DPB are marked as unused for reference

- #42 (DBP/NODOC/sharing) Remove sub-DPB sharing and processes that mark inter-layer reference pictures as "unused for reference"

SEI (6)

- #11 (SEI/Q0045/Overlay) Proposal for an SEI message on selectable overlays. Decision: Adopt (modified for variable-length strings).

- #35 (SEI/Q0189/SEI message for indicating constraints on TMVP) Proposal 2.3,  SEI message for indicating constraints on TMVP

- #37 (SEI/Q0253/layer not present), modified semantics of layers not present SEI message to correct bug introduced during editing

- #51 (SEI/Added Multiview scene information SEI message)

- #52 (SEI/Added Multiview acquisition information SEI)

- #53 (SEI/Added Multiview view position SEI message)

Miscellaneous (6)

- #22 (MISC/Q0177/inference sps_temporal_id_nesting_flag) proposal 1: modify inference rule for sps_temporal_id_nesting_flag when it is not present

- #24 (MISC/Q0078/scan and pic type) , Items 3 b,c and 4, clarifying which pictures in an output layer sets are applied the values of general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag.

- #30 (MISC/Q0102/loop index) proposal 3, change the max loop index for signaling bit rate and pic rate info to MaxSubLayersInLayerSetMinus1

- #34 (MISC/Q0189/slice temporal mvp enabled flag) Proposal 2.2, clarification of semantics of slice temporal mvp enabled flag

- #47 (MISC/Q0177/EOS NAL) proposal 2: clarification of description of end of sequence NAL unit

- #49 (MISC/Q0247/frame-field information SEI message)

Hybrid Scalability (1)

- #3 (HS/Q0041/hybrid scalability) The proposed text was endorsed, with non-editorial open issues considered as follows …:

Reference picture sets (2)

- #17 (RPS/Q0060/condition refLayerPicIdc) Proposal 2: Add a condition to the derivation of refLayerPicIdc of (TemporalId == 0)

- #36 (RPS/Q0100/constraint to semantics) v3, add constraint to RPS semantics

Fixes (8)

- #1 (ED.FIX/Q0081/alpha channel persist) On reuse of alpha planes in auxiliary pictures. It was asked why there would not be a presumption that the alpha channel content would simply persist, without needing the flag to indicate it. Decision (Ed.): Delegated to editors to clarify, as necessary, that the alpha channel content persists until cancelled or updated in output order.

- #25 (ED.FIX/Q0055/Rename TargetOutputLayerSetIdx) Second proposal, substitute ‘TargetOptLayerSetIdx’ with ‘TargetOutputLayerSetIdx’

- #25 (ED.FIX/Q0055/Set TargetOutputLayerSetIdx) Third Proposal; Modification of TargetOutputLayerSetIdx setting procedure. Refer to editors to improve C.1 description of setting the value of TargetOutputLayerSetIdx

- #26 (ED.FIX/Q0091/NAL, AU) It was noted that editorial improvements are needed for the association of NAL units with an access unit, and access unit boundary detection, independent from this proposal.  The language in this contribution may be useful to study.

- #27 (ED.FIX/Q0146/HandleCraAsBlaFlag and EOS) proposal 1.2, consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described

- #28  (Ed.Fix/Q0054/vps_profile_present_flag) proposal bug fix in the semantics of vps_profile_present_flag

- #29 (ED.FIX/Q0116/Recovery point SEI) , consider adding a note regarding how random accessibility is affected by the recovery point SEI message

- #46 (Ed.Fix/Q0146/slice_segment_header_extension_data_bit) proposal 1.3 specification bug fix

### Open issues

Open issues in H1002:

- Some text bugs reported in the bug tracking system or indicated by editor's comments.

# Recommendations

The recommendations of the MV-HEVC / 3D-HEVC Draft and Test Model editing AHG are to:

- Approve the edited documents H1001, H1002 and H1003 as JCT-3V outputs.

- Continue to edit documents to ensure that all agreed elements of 3D- and MV-HEVC are fully described.

- Compare the documents with the HTM-software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the documents.

- Ensure that properly drafted candidate text is available prior to making any decision to change the specifications.

- Discuss reported open issues.