| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0006 |
| --- | --- |

| Title: | JCT-3V AHG report: 3D High level syntax (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Boyce and M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) | Email: | jill@vidyo.com miska.hannuksela@nokia.com |
| Source: | AHG6 |  |  |

_____________________________

# Abstract

This report summarizes the activities of the ad-hoc group of 3D high level syntax (AHG6).

# Mandates

The following mandates for AHG6 are specified in JCT3V-H_Notes_d9.doc:

- Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC.

- Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs.

- Study SEI messages and VUI syntax designs needed for HEVC extensions.

- Study mechanisms to indicate decoding capabilities/requirements for auxiliary picture layers

- Investigate mechanisms for picture referencing across CRA pictures

- Study and compare mechanisms for enabling temporal enhancement layers and diagonal inter-layer prediction

- Study the operation of the Hypothetical Reference Decoder for multi-layer operation.

- Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs.

- Coordinate efforts with JCT-VC AHG9 on high-level syntax issues in relation to 3D extensions.

However, it is asserted that mandates 4 to 6, inclusive, are copied from the earlier AHG period (between the 7th and 8th meetings) and were no longer valid in the current AHG period (between 8th and 9th meetings).

# Activities

## Activities on email reflector

The following contribution was submitted on June 11th, and its availability was announced in the JCT-VC and JCT-3V reflectors:

JCTVC-R0049 MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 [M. M. Hannuksela (Nokia)]

Abstract: This contribution proposes allowing a VPS with a non-zero nuh_layer_id to be delivered to the decoder in addition to a "base" VPS with nuh_layer_id equal to 0, both with the same vps_video_parameter_set_id value. The VPS with a non-zero nuh_layer_id is required to be a semantic superset of the base VPS and may for example include information about additional layers. The VPS with the highest nuh_layer_id value among the VPSs with the same vps_video_parameter_set_id value is activated.

No other emails were exchanged over the JCT-3V reflector.

## AHG meeting

JCT-3V AHG6 has met collocated with JCT-VC on 1 – 2 July, 2014. The JCT-VC meeting notes document the agreements that have been reached.

## Submitted contributions

Most high-level syntax input contributions have been submitted to both JCT-VC and JCT-3V. These input contributions have been categorized in the JCT-VC meeting notes.

In addition there six high-level syntax contributions submitted only to JCT-3V. These input contributions have been categorized correctly in the initial JCT-3V meeting notes. Out of these six contributions:

- 5 contributions concern the HLS of 3D-HEVC (out of which one contribution is a cross-check)

- 1 contribution concerns the MV-HEVC specific SEI messages