| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0011 |
| --- | --- |

| Title: | CE1 Summary Report: Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Hongbin Liu Tomohiro Ikai | Tel: Email: | 86-10-5776-0696 hongbinl@qti.qualcomm.com 81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment 1 (CE1) on Residual Prediction. During the 8th JCT-3V meeting, two contributions have been proposed to improve the coding efficiency or reduce the memory bandwidth of residual prediction in 3D-HEVC and they were considered for further study in a Core Experiment. The goal of this CE is to evaluate them in the context of the 3D-HTM 11.0 for the potential memory bandwidth reduction and/or coding performance improvement.

In summary, 7 companies were involved in this CE, with 2 CE proposals and 2 CE-related proposals. Several input proposals related to ARP but not mainly for memory bandwidth reduction or coding efficiency are not coordinated under this CE.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Hongbin Liu Ying Chen | hongbinl@qti.qualcomm.com cheny@qti.qualcomm.com | P/C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com |  |
| Samsung | M. W. Park Jin Young Lee | m.w.park@samsung.com jinyoung79.lee@samsung.com | P |
| LG | Sehoon Yea | sehoon.yea@lge.com |  |
| NTT | Shiori Sugimoto Shinya Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp |  |
| ETRI | Gun Bang | gbang@etri.re.kr |  |

# Tools under investigation

## Simplification of Advanced Residual Prediction [JCT3V-I0051]

Two aspects are proposed in this contribution:

- For bi-predictively coded PUs, the number of accessed blocks used in ARP is reduced from 4 to 3:

  - When current PU is predicted from one temporal reference picture and one inter-view reference picture, the temporal reference block of base block in inter-view ARP is identified with the coded motion vector.

  - When current PU is predicted from two inter-view reference pictures, the temporal reference block in the ARP target reference picture associated with reference picture list 1 is set to the one associated with reference picture list 0.

- It is proposed to disable bi-predictive 4x4 chroma blocks with ARP mode by applying uni-prediction from reference picture list 0.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0155.

## Results of ARP simplification [JCT3V-I0104]

It is proposed to disable ARP for 4x4 chroma blocks to reduce ARP’s bandwidth.

This proposal was crosschecked by Qualcomm, and detail information can be referred to JCT3V-I0166.

## Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for CE proposals. In this table, memory bandwidth of depth (in DoNBDV) is not considered since both proposals only focus on residual prediction.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP in 3D-HEVC |  | CTC |  | 122% | 135% | 128% | 158% |
| JCT3V-I0051 #1 | 0.00% | -0.04% | -0.01% | 101% | 113% | 106% | 132% |
| JCT3V-I0151 #1 + #2 | 0.30% | 0.31% | 0.08% | 94% | 103% | 96% | 119% |
| JCT3V-I0104 | 0.41% | 0.41% | 0.11% | 105% | 95% | 88% | 108% |

Note that for both JCT3V-I0051 #2 and JCT3V-I0104, coding loss of chroma components is much higher than luma component. For example, the coding loss of Cb (Cr) PSNR/video bitrate on video 1 is 0.46% (0.42%) and 0.51% (0.68%) for JCT3V-I0051 #2 and JCT3V-I0104 respectively.

# Related proposals

## Alignment of disparity vector in inter-view motion and residual predictions for ARP [JCT3V-I0052]

This contribution proposes to align the disparity vector used in inter-view motion prediction and residual prediction. In this way, DoNBDV is not performed when ARP applies and memory bandwidth can be saved for ARP.

This proposal was crosschecked by Hisilicon and Sharp, and detail information can be referred to JCT3V-I0071 and I0159.

## Reduction of Worst Case Memory Bandwidth for ARP [JCT3V-I0081]

This contribution proposes to use NBDV instead of DoNBDV for the inter-view merge candidates when the ARP is enabled so that depth memory fetch is not needed anymore for ARP in the worst case.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0157.

Based on the idea description, memory bandwidth analysis and coding performance results, JCT3V-I0052 and JCT3V-I0081 are identical.

## Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for CE1 related subset A proposals. In this table, memory bandwidth of depth (in DoNBDV) is considered since both proposals try to remove depth access when ARP applies. Here, it is assumed that the whole depth block is fetched in DoNBDV.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP + DoNBDV in 3D-HEVC |  | CTC |  | 132% | 148% | 139% | 172% |
| JCT3V-I0052 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |
| JCT3V-I0081 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |