| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0012 |
| --- | --- |

| Title: | CE2 Summary Report on Depth Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Ying Chen Yi-Wen Chen | Tel: Email: | +1 858 845 6589 cheny@qti.qualcomm.com yiwen.chen@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment 2 (CE2) on Depth Intra Coding. During the 8th JCT-3V meeting, two contributions have been proposed to improve the coding efficiency or reduce the memory bandwidth of residual prediction in 3D-HEVC and they were considered for further study in a Core Experiment. The goal of this CE is to evaluate them in the context of the 3D-HTM 11.0.

In summary, 9 companies were involved in this CE, with 3 CE proposals and 7 CE-related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| LG | Sehoon Yea Jungdong Seo | sehoon.yea@lge.com jungdong.seo@lge.com | P/C |
| MediaTek | Yi-Wen Chen Jian-Liang Lin | yiwen.chen@mediatek.com jl.lin@mediatek.com | P/C |
| Samsung | M. W. Park Jin Young Lee | m.w.park@samsung.com jinyoung79.lee@samsung.com | P/C |
| Qualcomm | Hongbin Liu Ying Chen | hongbinl@qti.qualcomm.com cheny@qti.qualcomm.com | C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp |  |
| ETRI | Gun Bang | gbang@etri.re.kr |  |
| RWTH | Fabian Jäger | jaeger@ient.rwth-aachen.de | C |
| HHI | Philipp Merkle | philipp.merkle@hhi.fraunhofer.de | C |

# Tools under investigation

## Summary of the proposals

### CE2: Advanced boundary chain coding for depth intra mode [JCT3V-I0118]

This proposal provides the complexity assessment of the advanced boundary chain coding (ABC). The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following a contour described by a chain code. The method of complexity assessment is based on JCT3V-H0127 which provides a complexity analysis for depth intra coding modes. This proposal also presents the experimental results of ABC mode with and without CABAC (coded bins). The proposed method with CABAC reportedly shows 0.3% and 0.3% BDR gains in synthesized views under CTC and AI configurations, respectively. The coding gains of ABC without CABAC are 0.2% and 0.2% under CTC and AI configurations, respectively.

This proposal is cross checked by JCT3V-I0140 (MediaTek).

For the chain coding method which was removed as proposed in JCT3V-F0147,  the results are as follows:

Results on removal of RBC (CTC)

### 3D-CE2: Single depth intra mode for 3D-HEVC [JCT3V-I0095]

This contribution presents the results of CE2 in which the single depth mode is proposed to code the smooth area within a depth map. A CU-level flag is signaled to indicate whether a CU is coded as single depth mode. When a CU is coded as single depth mode, it is reconstructed by filling this CU with one single depth value. Additionally, an index is transmitted to select one out of several available depth sample candidates derived from the spatial neighboring pixels. Compared to the anchor HTM-11.0, the proposed method reportedly shows overall 0.3% BD-rate savings under common test conditions.

This proposal is cross checked by JCT3V-I0168 (Qualcomm).

The tests related to this category are listed as below.

Test 1: Test the coding performance of the proposed single depth mode. The proposed scheme in H0087 has been further simplified by reducing the size of the candidate list from 5 to 2 and replacing the corner spatial candidates with the candidates located at the center of the PU boundary.

Several additional experiments have also been tested to address several mandates of CE2, including the following.

Test 2: Test the coding performance of the current intra SDC scheme with modified encoder design. The encoder is modified by additionally testing the RDO performance for those DC values equal to the sample candidates of single depth mode. For each block coded as intra SDC mode, as shown in figure 1, the values of the neighboring pixels An-1, Bn-1, B0, A0, B-1 are additionally tested as the DC value at the encoder to see whether the coding performance could be further improved.

Figure 1. The spatial sample candidates.

Test 3: Test the coding performance of inserting DDD value as a new candidate for the single depth mode.

Test 4: Test the coding performance of applying residual coding to single depth mode.

### 3D-CE2: Residual DC quantization in intra SDC [JCT3V-I0082]

This contribution presents the results of CE2 in which residual DC quantization is proposed to reduce the magnitude of the residual DC for the general modes in intra SDC, instead of DLT. DLT signals an index instead of the DC value in intra SDC. Hence, to reconstruct the residual DC in a decoder side, DC of the predicted pixels should be first calculated and then re-mapping from the DLT index to the value is performed. In order to simplify these operation, the proposed method quantizes the residual DC with one shift operation to reduce the magnitude of the residual DC, instead of using DLT. The results shows that the proposed method achieves the coding gain of -0.1% and removes the DC calculation and re-mapping processes.

Test#1: DLT is applied to all the sequences by removing a threshold with 128

Test#2: The residual quantization is implemented without a decoder change by using the existing DLT function. When DLT is not constructed by the threshold, it stores every even depth pixel value as an index increases from 0 to 127.

Test#3: The residual DC quantization with default QP equal to 0

Test#4: The residual DC quantization with default QP equal to 1

This proposal is cross checked by JCT3V-I0122 Test#2/#3 (RWTH) and JCT3V-I0135 Test#4 (Sharp).

## Results of the proposals

Although there might be interactions between the three proposals especially on the overlaps of the coding gains. The results of these proposals are separately presented, although each of them has already been investigated in a way that the more closely related tools have been tested.

### Results for JCT3V-I0118: CE2: Advanced boundary chain coding for depth

|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| JCT3V-I0118 #1 | -0.11% | -0.05% | 0.14% | -0.28% | 100.8% | 98.4% |
| JCT3V-I0118 #2 (w/o CABAC) | -0.11% | -0.09% | 0.14% | -0.21% | 100.9% | 99.7% |
| AI |  |  |  |  |  |  |
| JCT3V-I0118 #1 | 0.00% | 0.00% | 0.02% | -0.29% | 101.4% | 100.4% |
| JCT3V-I0118 #2 (w/o CABAC) | 0.00% | 0.00% | 0.04% | -0.22% | 101.8% | 97.3% |

### Results for JCT3V-I0095: 3D-CE2: Single depth intra mode for 3D-HEVC

|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0095 Test 1 | 0.01% | 0.07% | -0.07% | -0.29% | 101.7% | 98.2% |
| I0095 Test 2 | 0.02% | 0.01% | 0.00% | 0.00% | 104.7% | 98.9% |
| I0095 Test 3 | 0.01% | 0.07% | -0.06% | -0.26% | 100.3% | 97.1% |
| I0095 Test 4 | -0.01% | 0.01% | -0.06% | -0.29% | 105.0% | 98.4% |
| AI |  |  |  |  |  |  |
| I0095 Test 1 | 0.00% | 0.00% |  | -0.34% | 99.2% | 96.0% |
| I0095 Test 2 | 0.00% | 0.00% |  | -0.01% | 123.8% | 101.7% |
| I0095 Test 3 | 0.00% | 0.00% |  | -0.34% | 98.9% | 96.6% |
| I0095 Test 4 | 0.00% | 0.00% |  | -0.35% | 106.3% | 99.0% |

### Results for JCT3V-I0082: 3D-CE2: Residual DC quantization in intra SDC

|  | Video PSNR / video bitrate | Video PSNR / total bitrate | Video PSNR | Synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.04% | 100.0% | 99.9% |
| I0082 #2 | 0.02% | -0.01% | -0.01% | 0.07% | 100.3% | 99.8% |
| I0082 #3 | -0.01% | -0.07% | -0.07% | -0.07% | 100.2% | 99.6% |
| I0082 #4 | 0.01% | -0.07% | -0.07% | -0.06% | 100.2% | 100.0% |
| AI |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.13% | 99.6% | 99.7% |
| I0082 #2 | 0.00% | -0.05% | -0.05% | 0.19% | 100.3% | 99.8% |
| I0082 #3 | 0.00% | -0.07% | -0.07% | -0.07% | 100.1% | 99.6% |
| I0082 #4 | 0.00% | -0.11% | -0.11% | -0.08% | 99.7% | 99.5% |

# Related proposals

## DMM related

Two proposals are related to simplification of the two DMM modes, respectively.

### CE2 related: Simplification of DMM4 [JCT3V-I0066]

This contribution proposes two modification of the DMM4 depth intra coding mode: simplification of threshold calculation as well as encoder optimization and simplification.

Currently the threshold value for contour segmentation derivation is calculated as the mean value of all depth samples, requiring N²-1 additions and one shift operation for an NxN block. Similar as in proposal JCT3V-H0058, the threshold calculation is simplified by using the mean of the four corner values as the threshold value for contour segmentation, requiring 3 additions and one shift, independent of the block size.

Currently, intra modes in inter slices are only tested under certain conditions in order to limit the encoder complexity. It is proposed that this encoder restriction is removed for DMM4 mode, i.e. always testing it in inter slices.

### Lookup table size reduction in DMM1 [JCT3V-I0110]

This contribution proposes a method to reuse wedgelet pattern of 16x16 block for 32x32 block instead of specifying 32x32 wedgelet pattern in order to reduce the storage size for wedgelet pattern in DMM1. Actually, a pattern for a 16x16 block is scaled to form a 32x32 block. Experimental result reportedly shows that 0.06% / 0.03% coding loss for synth PSNR vs. total bitrate in the CTC / all-intra configuration, respectively. It is asserted that this proposed method can achieve storage size reduction by 79.5%.

## SDC related

### 3D-CE2 related: Allowing zero DC candidate in inter SDC [JCT3V-I0083]

This proposal suggests two solutions to change the condition of depth_dc_flag, indicating whether the DC values are present or not. One solution leads to slight coding loss and the other doesn’t introduce coding loss.

### 3D-CE2 related: Fast SDC DC offset decision [JCT3V-I0084]

Multiple DC candidates were adopted to improve the performance of inter and intra SDC. At an encoder, SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through exhaustive comparison. In order to reduce the encoding complexity, the proposed method introduces a fast SDC DC offset decision method. Results demonstrate that the proposed method save the encoding time without coding loss.

## Other inputs

### CE2 related: Reduction of the number of intra prediction modes for depth coding [JCT3V-I0117]

This contribution presents an intra prediction mode for depth map coding. The proposed method reduces the number of intra prediction modes for depth map because the depth map has different characteristics compared to the texture video: 1) little variation within a block 2) simple or smooth region. It means that 35 intra prediction modes used in the conventional intra coding is unnecessarily expensive for depth intra coding. Therefore, this proposal reduced the number of conventional intra prediction modes for depth map coding. The proposed method has minor coding gains under the common test condition (CTC) and all-intra configuration (AI) with negligible encoding/decoding complexity.

### CE2 related: Remove depth_dc_flag signalling in DMM cases [JCT3V-I0120]

It is proposed that the depth_dc_flag (indicating whether residual value of segments to be 0 in case of sdc_flag equal to 1 and whether dc offset exists in case of when sdc_flag is equal to 0) is conditionally signalled when sdc_flag (indicating whether SDC is used) is unequal to 0. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

### 3D-CE2 related: Separate enabling flag for intra coding tools [JCT3V-I0085]

Intra coding tools such as intra SDC and DMM (DMM1 and DMM4) are enabled and disabled by a single flag ‘vps_depth_modes_flag’, while inter coding tools are individually selectable by their corresponding flags. When intra SDC is on and DMM is off, a DMM flag need to be signalled in each CU, which may be redundant. This contribution proposes to define a separate flag for DMM and intra SDC. In addition, it is suggested to change the names of DMM1 and DMM4 into DMM and IVP (Intra-view prediction), respectively.