| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0044 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0024 |
| --- | --- |

| Title: | AHG10: On decoder capability indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes a number of items related to indication of decoder capabilities.

# Proposals

## Number of layers as a decoder capability

It is proposed that the decoder capability indications include the number of layers or the number of inter-layer processing operations supported by the decoder. The number of resampling units in a HW decoder implementation might be limited – hence, it would be important to include it as decoder capability.

Adding the number of layers as a decoder capability would enable for example indicating that a decoder is capable of decoding a Scalable Main bitstream with two layers.

For example, the following specification text (with the added part in bold) could be used:

Capabilities of video decoders conforming to this Specification are specified in terms of the ability to decode video streams conforming to the constraints of profiles, tiers and levels. When expressing the capabilities of a decoder for a specified profile, the tier and level supported for that profile should also be expressed. When expressing the capabilities of a decoder to decode output layer sets conforming to a profile specified in Annex G or H, the capabilities of a decoder also include the number of layers that the decoder has the ability to decode. When a decoder is indicated to be capable of decoding a certain profile, a certain tier, the level levelIdc and the number of layers numLayers, the decoder shall be capable of decoding any output layer set of that profile and tier and with a level less than or equal to levelIdc and with the number of layers in the output layer set less than or equal to numLayers.

## Inferred profile decoding capability

It is proposed that it is explicitly specified which profiles shall be supported by a decoder, when support for decoder capability for a "layered" profile is indicated. While the exact lists of profiles that shall be supported are probably outside the scope of the mandate of AHG10, examples (in terms of specification text) are given here to make the intent of this proposal clearer:

When the ability to decode the Scalable Main 10 profile is indicated for a decoder, the decoder shall also have the ability to decode the Scalable Main profile, the Main profile, the Main Still Picture profile and the Main 10 profile.

When the ability to decode the Scalable Main profile or the Stereo Main profile in indicated for a decoder, the decoder shall also have the ability to decode the Main profile and the Main Still Picture profile.

It is asserted that such specification has the following benefits:

- It may reduce the signalling needed to indicate decoder capability, as all supported profiles need not be separately listed.

- Profile downgrading in mode negotiation, such as in SDP Offer/Answer procedure, becomes clear. For example, if an offerer offers a Scalable Main 10 profile but the answerer can only support Scalable Main profile, the answerer can downgrade the profile in the answer to the Scalable Main profile. Without the clearly specified downgrading of profiles, the offerer would have to list each profile separately in its offer.

## Assignment on bitstream partitions to processing units (comment on JCTVC-R0043/JCT3V-I0022)

JCTVC-R0043/JCT3V-I0022 proposes to indicate decoder capabilities as a set of profile, tier and level triplets denoted by (Pi , Ti , Li) for i in the range of 0 to q – 1, inclusive, where q is a positive integer essentially indicating the number of processing units or cores in the decoder.

It is remarked that the specification text of JCTVC-R0043/JCT3V-I0022 assumes that bitstream partitions can be assigned to any processing units as long as the processing core is indicated to be capable of decoding the required profile-tier-level combination. For a computing architecture of a decoder, this arbitrary assignment might require a certain type of a data access approach between the processing units, e.g. a bus. A conscious decision should be made whether such an assignment scheme of bitstream partitions is fine. An alternative might be to require certain order of assigning bitstream partition to processing units.

It is noted that in order to confirm whether an output layer set is decodable with certain decoding capabilities, potentially a large number of combinations of how to assign bitstream partitions to processing units has to be tested in brute-force manner. That is because no ordering of assigning a processing unit to a bitstream partition has been specified in the algorithm in G.11.3, but rather any available processing unit fulfilling the profile-tier-level requirement for the bitstream partition can be taken. The phrasing "if( there is a non-empty UnassignedProfileTierLevelTriples[ x ] such that" has been used in the algorithm in subclause G.11.3. It is suggested to consciously decide whether such an approach is fine or whether bitstream partition assignment to processing units should be performed in an unambiguously specified order.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).