| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0046 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0026 |
| --- | --- |

| Title: | AHG10: Comments on JCTVC-R0043/JCT3V-I0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution includes some technical comments on JCTVC-R0043/JCT3V-I0022, proposes some changes and identifies some topics that should be studied further.

# Comments

## On partition-wise conformance testing

JCTVC-R0043 proposes bitstream conformance to be tested partition-wise, summarized in JCTVC-R0043 as follows:

The bitstream conformance tests are specified to test the conformance of each partition, in a manner that decoded pictures and other necessary information for all direct or indirect reference layer of the layers in the partition are assumed to be available similarly as the current specification for the base layer when it is externally provided.

This design choice is asserted to have several challenges, including:

- The properties of the reference-layer pictures affect the decoding process, e.g. in VPS activation and layer-wise start-up. The exhaustiveness of the present specification text in this regard is not felt to be sufficient.

- The joint HRD behavior of the bitstream partitions is not controlled. This leads to similar issues as reported in JCTVC-Q0188 for the joint HRD conformance checking of the external base layer and HEVC enhancement layers. In particular, the buffering for bitstream partitions should be such that

- The pictures in the referenced bitstream partition have to be decoded before they are needed as inter-layer references in a "predicted" bitstream partition.

- The pictures in the referenced bitstream partition must still remain in the DPB when they are needed as inter-layer references in a "predicted" bitstream partition.

When conformance of each partition is tested separately, the above-mentioned cross-partition constraints are not checked and cannot be verified.

- It is allowed to create output layer sets and bitstream partitions such that pictures of more than bitstream partition of an output layer set are output. The output of pictures of the same access unit should be synchronized, and bitstream and decoder conformance specifications should guarantee such synchronized output. However, when partition-wise bitstream conformance testing is performed, bitstream and decoder conformance specifications do not guarantee such synchronized output.

To overcome the above-described challenges, it is proposed to specify bitstream conformance to be tested for the entire sub-bitstream of the output layer set using the bitstream partition buffering (BPB) operation.

## On level specifications

It is remarked that in JCTVC-R0043 level limits are essentially specified as if all pictures of a partition unit would be handled as one big picture. For example, partitionSizeinSamplesY is derived as a sum of PicSizeInSamplesY and many of the constraints of Annex A apply without changes other than referring to partitionSizeInSamplesY rather than PicSizeInSamplesY. For instance, partitionSizeinSamplesY is limited by MaxLumaPs of the level.

The way levels are specified in JCTVC-R0043 has some nice consequences, such as the ability to downgrade a profile in session negotiation while keeping the level unchanged.

As a general remark, it is noted that JCTVC-R0043 lacks the intuitive association of levels to specific spatial resolutions of individual pictures or layers. Options that could be considered instead include:

- Include a scale factor in the level limits, as was done e.g. in MVC (mvcScaleFactor is equal to 2) and MVC+D (mvcdScaleFactor is equal to 2.5). The intuitive association of levels to specific spatial resolutions would then be maintained with the typical/target number of layers and the typical/target spatial resolution scaling factor.

- Include a scale factor in the level limits that is a function or a multiple of the number of layers in the output layer sets. The intuitive association of levels to specific spatial resolutions would then be maintained with the typical/target spatial resolution scaling factor.

In addition, it is suggested that the following details of the level specifications proposed in JCTVC-R0043 require further studying:

- Picture aspect ratio constraints.

The following picture aspect ratio constraints are proposed in JCTVC-R0043:

For each layer in the partition, the value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

For each layer in the partition, the value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

The above specifications allow more extreme picture aspect ratios compared to those allowed by Annex A profiles. It should be considered whether picture aspect ratio limitations should be kept similar to or the same as in Annex A.

- CtbSizeY constraint.

The following constraint on CtbSizeY is proposed in JCTVC-R0043:

For level 5 and higher levels, the value of CtbSizeY for each layer in the partition shall be equal to 32 or 64.

It is believed that the above constraint does not overcome problem 1 described in JCTVC-Q0145 as follows:

If more than 4 SNR layers of 720p resolution (or equivalent number of luma pixels e.g. more layers with combined spatial and SNR layers) are needed, then Level 5 or above would have to be used. Consequently, the value of CtbSizeY shall be equal to 32 or 64 (per item e of the general limits). For resolution like 720p or lower, this restriction disallows the use of smaller coding tree block sizes such as 16x16 and can therefore result in sub-optimal coding efficiency.

It is suggested that the importance problem should be agreed by AHG10, and, if considered important, a change in the CtbSizeY constraint should be designed to solve the problem.

- Constraints on the rate or number of tiles

In JCTVC-R0043 the number of tiles (or, more precisely, the rate of tiles) is proposed to be limited for partition units rather than for pictures. Such a limitation would allow, for example, a partition unit where an EL picture is coded as one tile and a BL picture with large number of tiles. It should be considered whether constraints to disallow radically uneven distribution of tiles across layers could cause problems for decoder's capability of handling all bitstream partitions conforming to a certain level. If so, it is suggested to design suitable picture-wise limits for the number or rate of tiles.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).