| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0053r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0029r1 |
| --- | --- |

| Title: | AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following proposals are made in this contribution:

- It is proposed that the decoding process of the current picture (subclause 8.1, subclause F.8.1, subclause G.8.1 or subclause H.8.1) is selected on the basis of nuh_layer_id of the current picture and the profile to which the bitstream partition containing the current picture conforms.

- It is proposed that the general decoding process in subclause 8.1 selects TargetPartitioningScheme based on external means or HRD, and if no other means is available TargetPartitioningScheme is set to 0. The BitstreamToDecode derived in subclause 8.1 is proposed to consist of the partitions containing target output layers, the partitions containing the direct and indirect reference layers of the layers in the partitions containing the target output layers and the partition containing the base layer.

- The following items, which are asserted to be bug fixes or editorial improvements, are also proposed:

  - It is clearly indicated that the processing for the external base-layer pictures is invoked at the beginning of an access unit, and the processing is separated into its own subclause (F.8.1.6).

  - The invocations of the decoding process for the current picture is gathered in one location (subclause 8.1) to make the text easier to understand. Related subclauses F.8.1, F.8.1.2, G.8.1 and H.8.1 are proposed to be deleted (but their child subclauses are kept).

# Proposals

The motivation and an overview of each of the proposals is given in the following sub-sections. The detailed proposal is provided with the change marks in the accompanying document relative to the AHG10 output specification text. The proposed text changes are identical in both variants of the AHG10 output specification text, and hence only one accompanying document is provided.

## Invocation of the decoding process for the current picture

### Motivation

It is asserted that the invocation of the correct decoding process for the current picture is not clearly specified in the current MV-HEVC/SHVC specification. Furthermore, now that AHG10 recommends profiles to be specified on bitstream partition basis, it is not clear which decoding process (HEVC v1, Annex F base layer decoding, MV-HEVC, SHVC, 3D-HEVC) to use on layer basis.

It is further remarked that scalability dimensions may be not be indicative of the decoding process to be used. For example, both MV-HEVC and 3D-HEVC will use the ViewOrderIdx scalability dimension but use a different decoding process.

### Proposal

It is proposed that the decoding process (subclause 8.1, subclause F.8.1, subclause G.8.1 or subclause H.8.1) is selected on the basis of nuh_layer_id of the current picture and the profile to which the bitstream partition containing the layer conforms.

The following text is proposed for subclause 8.1:

The following applies for the decoding of the current picture:

–	If the current picture has nuh_layer_id equal to 0 and TargetOlsIdx is equal to 0, the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7 and the decoding process for a coded picture with nuh_layer_id equal to 0 as specified in subclause Error: Reference source not found is invoked.

–	Otherwise if the current picture has nuh_layer_id equal to 0, the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause Error: Reference source not found and the decoding process for a coded picture with nuh_layer_id equal to 0 as specified in subclause Error: Reference source not found is invoked.

[Ed. (MH): If the decoding capability of independent non-base layers is supported "natively" without rewriting, one should add here the invocation of the respective decoding process.]

–	Otherwise, the following applies:

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	Let lIdx be equal to such value for which the nuh_layer_id value of the current picture is equal to LayerSetLayerIdList[ OlsIdxToLsIdx[ TargetOlsIdx ] ][ lIdx ].

–	Let pIdx be equal to such value for which layer_included_in_partition_flag[ TargetOlsIdx ][ TargetPartitioningScheme ][ pIdx ][ lIdx ] is equal to 1.

–	If general_profile_idc for the pIdx-th bitstream partition in the TargetPartitionScheme-th partitioning scheme of the TargetOlsIdx-th OLS is equal to [Ed. add MV-HEVC profile_idc value], the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause Error: Reference source not found and the decoding process of subclause Error: Reference source not found is invoked.

–	Otherwise, general_profile_idc for the pIdx-th bitstream partition in the TargetPartitionScheme-th partitioning scheme of the TargetOlsIdx-th OLS is equal to [Ed. add SHVC profile_idc values], the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause H.7 and the decoding process of subclause H.8.1.1 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

## Selection of the partitioning scheme and bitstream partitions to be decoded

### Motivation

In the previous section it is proposed that the selection of the decoding process for the current picture would be done on the basis of the profile indicated to the bitstream partition containing the layer of the current picture. Consequently, the partitioning scheme that is in use should be known or selected by the general decoding process.

The profile conformance in G.10 (for the Stereo Main profile) in the AHG10 output text is specified for a sub-bitstream consisting of the bitstream partition conforming the profile and all the direct and indirect reference layers of the layers in the bitstream partition.

This proposal also responds to the following comment in the second AHG10 teleconference:

It was further discussed whether it is possible to have other (non-auxiliary) pictures that are included in an output layer set but are not target output layers of the output layer set and are not in layers that are needed for decoding the target output layers (e.g., an independently coded layer).

Hannuksela indicated that he believed this is allowed, and that layer sets can include any layers, and output_layer_flag[ i ][ j ] can be set without constraints.

It seemed that further study of those semantics and that question would be needed.

It was asked whether the extraction process can include such "extra" non-target output layers in an extracted bitstream (after recent modifications).

It was suggested that the profile/tier/level information for such an output layer set would include all the layers in the output layer set in the CPB operation but would not include the "unnecessary" layers in its decoding process requirements.

It was remarked that the current specification assumes that the full decoding process is applied to the "unnecessary" layers as well as to the "necessary" layers.

Future extensibility was suggested to be a use case that would motivate this behaviour.

It was remarked that we may not have adequate text language currently regarding identification of which layers the profile/tier/level information applies to. It was remarked that it should be possible to draft such language, although it is not currently there.

In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. However, there was no clear agreement that this needed to be supported, pending further study.

It was asked whether the "necessary" layer subset should be explicitly identified rather than determined from dependency tree information, to ease the identification of the relevant bitstream subset.

It is remarked non-output layers that are not required for decoding of any of the target output layers of an OLS may be included in the OLS to account for the bitrate and CPB requirements including the non-output layers too. Consequenlty, picture timing, buffering period, and other HRD parameters can be provided for an OLS where some of the layers would not be required to be decoded. It is further noted that a bitstream partition may be such that none of its layers is needed for output or decoding. For example, a 3-view bitstream could be like follows:

- The base layer is Main profile compatible, sub-bitstream of the base layer and layer 1 is Stereo Main profile compatible, and the entire bitstream is 3D-HEVC profile compatible (i.e. layer 2 uses 3D-HEVC coding tools).

- A first OLS is specified to contain all 3 layers (for providing HRD parameters for the entire bitstream), but only the base layer and layer 1 are output layers. A bitstream partitioning is included in VPS such that a first partition consists of the base layer and layer 1, while a second partition consists of layer 2. Stereo Main decoders should be able to decode this OLS, as layer 2 is not required for output or decoding.

- A second OLS is specified to contain all 3 layers (for providing HRD parameters for the entire bitstream), and all layers are output layers. Stereo Main decoders cannot decode this OLS, as layer 2 is required for output and decoding.

The proposed change omits the decoding of "unnecessary" bitstream partitions not needed for decoding of the bitstream partitions containing output layers or needed for decoding output layers.

### Proposal

It is proposed that the general decoding process in subclause 8.1 selects TargetPartitioningScheme based on external means or HRD, and if no other means is available TargetPartitioningScheme is set to 0. The BitstreamToDecode derived in subclause 8.1 is proposed to consist of the partitions that contain target output layers as well as the partitions that contain direct and indirect reference layers for any layers of the partitions containing target output layers. Additionally, bitstream partition containing the base layer is included in the BitstreamToDecode, because it can impose cross-layer impacts even when the decoded base-layer pictures were not used as reference for inter-layer prediction.

The following text is proposed into subclause 8.1:

The variable TargetPartitioningScheme, which specifies the partitioning scheme of the the OLS with index TargetOlsIdx, is specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetPartitioningScheme, TargetPartitioningScheme is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, TargetPartitioningScheme is set as specified in subclause C.1.

–	Otherwise, TargetPartitioningScheme is set equal to 0.

When TargetPartitioningScheme is selected by external means, it is a requirement for the external means that TargetPartitioningScheme is in the range of 0 to NumPartitioningSchemes[ TargetOlsIdx ] − 1, inclusive.

The variable TargetDecLayerSetIdx, the layer identifier list TargetOptLayerIdList, which specifies the list of nuh_layer_id values, in increasing order of nuh_layer_id values, of the pictures to be output, and the layer identifier list TargetDecLayerIdList, which specifies the list of nuh_layer_id values, in increasing order of nuh_layer_id values, of the NAL units to be decoded, are specified as follows:

–	The variables are initialized as follows:

TargetDecLayerSetIdx = OlsIdxToLsIdx[ TargetOlsIdx ]
lsIdx = TargetDecLayerSetIdx
psIdx = TargetPartitioningScheme
for( partCnt = 0; partCnt <= num_partitions_in_scheme_minus1[ psIdx ]; partCnt++ )
	for( k = 0, j = 0; j < NumLayersInIdList[ lsIdx ]; j++ )
		if( RequiredPartitionFlag[ TargetOlsIdx ][ psIdx ][ partCnt ]  &&  
		     layer_included_in_partition_flag[ TargetOlsIdx ][ psIdx ][ partCnt ][ j ] ) {
			TargetDecLayerIdList[ j ] = LayerSetLayerIdList[ lsIdx ][ j ]
			if( OutputLayerFlag[ TargetOlsIdx ][ j ] )
				TargetOptLayerIdList[ k++ ] = LayerSetLayerIdList[ lsIdx ][ j ]
		}

-

–	The list TargetDecLayerIdList is sorted so that its entries appear in increasing nuh_layer_id order.

–	The list TargetOptLayerIdList is sorted so that its entries appear in increasing nuh_layer_id order.

## Bug fixes and editorial improvements

The following items, which are asserted to be bug fixes or editorial improvements, are also proposed:

- (The sub-proposal a of version 1 of contribution is redundant, as this item was already included in the AHG10 output text).

- It is clearly indicated that the processing for the external base-layer pictures is invoked at the beginning of an access unit, and the processing is separated into its own subclause (F.8.1.6).

- The invocations of the decoding process for the current picture is gathered in one location (subclause 8.1) to make the text easier to understand. Related subclauses F.8.1, F.8.1.2, G.8.1 and H.8.1 are proposed to be deleted (but their child subclauses are kept).

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).