| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014 | Document: JCTVC-R0054 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0030 |

| Title: | AHG10: Comments on JCTVC-R0041/JCT3V-I0023 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution provides some comments on JCTVC-R0041/JCT3V-I0023.

# Introduction

The contribution provides some comments on JCTVC-R0041/JCT3V-I0023. Primarily, the proposal in JCTVC-R0041/JCT3V-I0023 version 4 is commented, while some comments are also made on JCTVC-R0041/JCT3V-I0023 version 3. The comments are marked with "[Ed. (MH)" within the specification text proposed in JCTVC-R0041/JCT3V-I0023, which is copied below.

The following general remarks are made, summarizing some of the specific comments below:

- It is asserted that a new sub-bitstream extraction process is needed such that keeps the essential NAL units from the base layer (VPS, EOB, EOS) and otherwise keeps the layers of the OLS with the independent non-base layer or additional layer set. Such extraction is similar to rewriting proposed in JCTVC-R0042/JCT3V-I0021, which raises the question why the rewriting process should be avoided.

- The semantic constraints, such as those proposed below, could potentially make it easier for an encoder to write an independent non-base layer with "INBLD" capability. However, they could disallow the encoder to use some features that are desirable, such as sending poc_resetting_period_id for error resilience or discardable_flag for scalability/thinning indications. Furthermore, it remains unclear whether the decoder can use version 1 decoding processes for decoding the layers with "INBLD" property. It is suggested that the rewriting process, such as that proposed in JCTVC-R0042/JCT3V-I0021, makes it very clear that the version 1 decoding process is applicable as such, after the nuh_layer_id values of the NAL unit have been changed according to the rewriting process.

# Version 4 proposed semantics changes, with additional profile constraints and removal of rewriting process and defining Independent non base layer decoding (IMBLD) capability

The language is based upon JCT3V-H1002_v5.

- F.11 Independent non base layer decoding capability

This subclause specifies an independent non base layer decoding (INBLD) capability for applications that support decoding of independent layers with the value of nuh_layer_id greater than 0, which otherwise conform to one or more of the profiles specified in Annex A. [Ed. (MH): It is not very clear to what "otherwise" is relative.] This specification enables the construction of bitstreams that contain multiple independently coded layers and the decoding of sub-bitstreams extracted from such bitstreams.

An OLS that conforms to a profile specified in Annex A [Ed. (MH): As per one variant of the AHG10 output text, it might not be indicated which profile an entire profile conforms to, but rather profile conformance is indicated bitstream partition wise. The text here needs to be revised accordingly (also when it comes to the derivation which sub-bitstream INBLD applies to, as specified below).] with INBLD capability shall satisfy the constraints on a derived sub-bitstream for the OLS as follows. [Ed. (MH): Do these constraints apply only to OLSs for which the PTL syntax structure indicates INBLD? Or do these constraints also apply to CVSs for which the PTL syntax structure in the active SPS indicates INBLD? Or is it required that whenever there is an active SPS with PTL syntax structure indicating INBLD, there shall also be the respective OLS in the VPS with PTL syntax structure indicating INBLD?]  Let olsIdx be the OLS index of an OLS, with a layerIdListTarget containing a single value of nuh_layer_id, with nuh_layer_id > 0.  A sub-bitstream for the OLS is derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with the following inputs: the bitstream, tIdTarget equal to 6, and layerIdListTarget that is included in the layer set with the index layerSetIdx. [Ed. (MH): The sub-bitstream extraction of F.10.1 is not going to work here, because it will remove VPS_NUT, EOB_NUT and EOS_NUT, which have nuh_layer_id equal to 0. Something similar to what if proposed in F.10.3 in JCTVC-R0042 is needed. In other words, a sub-bitstream extraction process has to be specified, which keeps the essential NAL units from the base layer, such as VPS_NUT, EOB_NUT and EOS_NUT, and otherwise keeps the data among the layers of the additional layer set.] [Ed. (MH): A conscious decision should be made whether the SPSs for the independent non-base layer need to have nuh_layer_id among layerIdListTarget only or whether it can also have nuh_layer_id equal to 0.]

- general_profile_space shall be equal to 1 in the profile_tier_level( ) syntax structure applying to the olsIdx-th OLS [Ed. (MH): why not to use one of the general_reserved_zero_44bits instead? Seems that those bits would be less scarce than general_profile_space bits.] [Ed. (MH): Another possibility has been proposed in R0042: indicate the decoder capability for INBLD separately from the PTL, while the bitstream property of INBLD can be concluded from the fact whether the PTL applies to the base layer or to an independent non-base layer.] [Ed. (MH): In general, the question on how to indicate INBLD as decoder capability is different from the question how to specify the constraints for an OLS and/or an independent non-base layer that is said to conform to INBLD.]

- VPSs shall have poc_lsb_not_present_flag[ LayerIdxInVPS[ nuh_layer_id ] ] when nuh_layer_id contained in layerIdListTarget

- SPSs shall have V1CompatibleSPSFlag equal to 1 only

- Slice segments header extensions shall have poc_reset_idc equal to 0 only, when present [Ed. (MH): As a consequence, encoders cannot send poc_reset_period_id, which could help in loss resilience even if it is not needed for the decoding process. A better constraint would be to state that POC decoding of 8.3.1 can be used.]

- Slice segments header extensions shall have poc_msb_val_present_flag equal to 0 only, when present

- [Ed. (MH): The following is present in F.8.3.1: "If the current picture is not a RASL picture, a RADL picture or a sub-layer non-reference picture, and the current picture has TemporalId equal to 0 and discardable_flag equal to 0, PrevPicOrderCnt[ lId ] is set equal to PicOrderCntVal." The requirement on having discardable_flag equal to 0 disables the use of discardable pictures as POC anchor pictures. However, this constraint is not present in subclause 8.3.1. Should there be a requirement here that discardable_flag shall be equal to 0? Such a requirement might be undesirable, though, when the independent non-base layer is used as a reference layer for predicted layers. A better constraint would be to state that POC decoding of 8.3.1 can be used.]

Decoders with INBLD capability that conform to a profile specified in Annex A at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all extracted sub-bitstreams [Ed. (MH): Extracted based on what?] for which all of the following conditions apply [Ed. (MH): Specify in which syntax structures, VPS and/or SPS, and in which PTL structures these conditions apply]:

- general_profile_space equal to 1

- general_profile_idc or general_profile_compatibility_flag[ i ] indicate a profile defined in Annex A

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

general_profile_space specifies the context for the interpretation of general_profile_idc and general_profile_combatibility_flag[ i ] for all values of i in the range of 0 to 31, inclusive. The value of general_profile_space shall be in the range of 0..1 in bitstreams conforming to this version of this Specification. Other values for general_profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the CVS when general_profile_space is not in the range of 0..1.

general_profile_space equal to 0 specifies that constraints may be placed on an OLS conforming to  some values of general_profile_idc such that the target layer list only include the layer with nuh_layer_id equal to 0. general_profile_space equal to 1 specifies that additional constraints may be imposed for some values of general_profile_idc for layers contained in the target layer list of an OLS with nuh_layer_id > 0.

general_profile_idc, when general_profile_space in the range of 0..1, indicates a profile to which the CVS conforms as specified in Annex A or subclause Error: Reference source not found. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A or subclause Error: Reference source not found. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

# Version 3 proposed semantics changes, with additional profile constraints and removal of rewriting process

The language is based upon JCT3V-H1002_v5.

[Ed. (MH): Most of the comments for v4 apply also to v3.]

general_profile_space specifies the context for the interpretation of general_profile_idc and general_profile_combatibility_flag[ i ] for all values of i in the range of 0 to 31, inclusive. The value of general_profile_space shall be in the range of 0..1 in bitstreams conforming to this version of this Specification. Other values for general_profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the CVS when general_profile_space is not in the range of 0..1.

general_profile_space equal to 0 specifies that constraints may be placed on an OLS conforming to  some values of general_profile_idc such that the target layer list only include the layer with nuh_layer_id equal to 0. general_profile_space equal to 1 specifies that additional constraints may be imposed for some values of general_profile_idc for layers contained in the target layer list of an OLS with nuh_layer_id > 0.

general_profile_idc, when general_profile_space in the range of 0..1, indicates a profile to which the CVS conforms as specified in Annex A or subclause Error: Reference source not found. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A or subclause Error: Reference source not found. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

Replace subclause "A.3.2 Main profile” with the following.

A.3.2 Main profile

v1 compatible Bitstreams containing OLSs conforming to the Main profile shall obey the following constraints on the derived sub-bitstream for each of the OLSs, with OLS index olsIdx and layerSetIdx being the layer set for the OLS, and the sub-bitstream being derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with the following inputs: the bitstream, tIdTarget equal to 6, and layerIdListTarget containing one value of nuh_layer_id that is included in the layer set with the index layerSetIdx.

Bitstreams conforming to the Main profile shall obey the following constraints:

- SPSs shall have chroma_format_idc equal to 1 only.

- SPSs shall have bit_depth_luma_minus8 equal to 0 only.

- SPSs shall have bit_depth_chroma_minus8 equal to 0 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- If general_profile_space equal to 0, the value of nuh_layer_id contained within the layerIdListTarget shall be equal to 0

- Otherwise, the following constraints apply

  - VPSs shall have poc_lsb_not_present_flag[ LayerIdxInVPS[ nuh_layer_id ] ] when nuh_layer_id contained in layerIdListTarget

  - SPSs shall have V1CompatibleSPSFlag equal to 1 only

  - Slice segments header extensions shall have poc_reset_idc equal to 0 only, when present

  - Slice segments header extensions shall have poc_msb_val_present_flag equal to 0 only, when present

- [Ed. (JB): Possibly add a note to indicate that decoding process in 8.1.1 rather than F.8.1.2 can be followed] [Ed. (MH): F.8.1.2 acts as a parent for invoking either the MV-HEVC or SHVC decoding process, and in the future also the 3D-HEVC decoding process. There should rather be a decoding process for independent non-base layer that is independent of the type of scalability that might or might not be applied on top of it.]

Conformance of a bitstream to the Main profile is indicated by general_profile_idc being equal to 1 or general_profile_compatibility_flag[ 1 ] being equal to 1.

NOTE – When general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] should also be equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams [add something about sub-bitstream] [Ed. (MH) No, only the INBLD capable decoders shall be capable of handling sub-bitstreams] for which all of the following conditions apply [Ed. (MH) Add: "in the active SPSs for the base layer"]:

- general_profile_compatibility_flag[ 1 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).