| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0062 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0031 |
| --- | --- |

| Title: | On monochrome coding (concerning multi-layer profiles and auxiliary pictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following alternative proposals are made:

- Require monochrome sampling support and support of the Monochrome 8 profile for the independent layer in the Stereo Main, Scalable Main and Scalable Main 10 profiles.

- Allow the use of three color components with alpha and depth auxiliary layers in a manner that chroma samples are required to be equal to 1  <<  ( BitDepthC − 1 ).

The proponent prefers alternative 2.

# Proposal

A related excerpt from the AHG10 meeting notes is copied below:

It was noted that monochrome is not a capability within Main profile, which creates an issue with regard to whether a decoder that can decode such pictures could be constructed from existing Main profile decoders. Using the pseudo-monochrome syntax found in version 1 might be a way to avoid this.

Multi-layer monochrome coding is needed particularly for depth layers, which may also be inter-layer predicted from each other. Currently, monochrome coding is not enabled by Stereo Main, Scalable Main and Scalable Main 10 profiles.

The following alternative proposals are made:

- Require monochrome sampling support and support of the Monochrome 8 profile for the independent layer in the Stereo Main, Scalable Main and Scalable Main 10 profiles.

- Require only the Main profile support for the independent layer in the Stereo Main, Scalable Main and Scalable Main 10 profiles.

Allow the use of three color components with alpha and depth auxiliary layers as follows:

It is a requirement of bitstream conformance that when AuxId[ lId ] is equal to AUX_ALPHA or AUX_DEPTH, either of the following applies:

–	chroma_format_idc is equal to 0 in the active SPS for the layer with nuh_layer_id equal to lId.

–	The value of all decoded chroma samples is equal to 1  <<  ( BitDepthC − 1 ).

Consequently, when an output layer set consisting of multiple auxiliary layers is indicated to conform to the Stereo Main, Scalable Main or Scalable Main 10 profile, the 4:2:0 chroma format is in use in the layers.

The proponent prefers alternative 2.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).