| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0071r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0032r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On cross-layer impacts of IRAP and EOS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contributions asserts that the lack of cross-layer impacts as response to end of sequence (EOS) NAL unit and IRAP pictures with an indicated broken link is undesirable and proposes the following:

- It is proposed that if the current picture is the first picture that follows an end of sequence NAL unit (with nuh_layer_id equal to 0) in decoding order, NoClrasOutputFlag is set equal to 1. (This causes marking of all pictures in the DPB as "unused for reference" as well as setting LayerInitializedFlag and FirstPicInLayerDecodedFlag equal to 0 for all layers, which initializes the layer-wise start-up process.)

Item 1 is asserted to be functionally identical to an aspect in JCTVC-R0224.

- It is proposed that when nuh_layer_id is greater than 0, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause the following:

- LayerInitializedFlag and FirstPicInLayerDecodedFlag is set equal to 0 for the current layer and all layers directly or indirectly predicted from the current layer. (Consequently, layer-wise start-up is enforced for these layers.)

- Each picture in the DPB for the current layer and all layers directly or indirectly predicted from the current layer are marked as "unused for reference".

- It is proposed to allow EOS NAL unit with nuh_layer_id greater than 0.

It is proposed that the decoding of the EOS NAL unit with nuh_layer_id greater than 0 causes the same impacts as those above (2a and 2b) for the subsequent IRAP picture with the same nuh_layer_id. Furthermore, for the semantics of SEI messages applying for nuh_layer_id greater than 0, it is proposed that EOS NAL unit with nuh_layer_id greater than 0 is treated similarly to an end of sequence for the persistence of the SEI message. When present, an EOS NAL unit with a particular nuh_layer_id value shall be the last NAL unit with that particular nuh_layer_id value within an access unit, except an end of bitstream NAL unit (when present).

In revision 2 of the contribution, the following additional constraint is proposed to avoid access units and picture units without any VCL NAL units resulting from an extraction process:

An EOS NAL unit with a particular nuh_layer_id value shall not be present in an access unit, unless a coded picture with that particular nuh_layer_id value is present in the same access unit.

- As part of the HRD specification, NoOutputOfPriorPicsFlag and predLayerBufferEmptyFlag are derived for IRAP pictures of independent layers with nuh_layer_id greater than 0 similarly to NoOutputOfPriorPicsFlag and crossLayerBufferEmptyFlag for base-layer IRAP pictures. When NoOutputOfPriorPicsFlag and predLayerBufferEmptyFlag are equal to 1, the pictures in the independent layer and all layers directly or indirectly predicted from the independent layers are not output.

It is noted that as proposed in JCTVC-R0235/JCT3V-I0048, when a bitstream without a base layer (vps_base_layer_available_flag equal to 0) is proposed the layer with smallest nuh_layer_id value is treated in the decoding process the same way as the base layer. Among other things, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause a cross-layer impact.

It is asserted that the proposed item 2 makes BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 of the lowest layer of a layer tree (without a base layer) to cause a cross-layer impact both when the layer tree is decoded as part of a bitstream with a base layer and when the layer tree is extracted (with subclause F.10.3 of the AHG10 output text) to become a bitstream without a base layer.

It is asserted that the proposed item 3 makes the IRAP picture following an EOS NAL unit in the same non-base layer cause a cross-layer impact both when the layer tree is decoded as part of a bitstream with a base layer and when the layer tree is extracted (with subclause F.10.3 of the AHG10 output text) to become a bitstream without a base layer.

# Introduction

## Background: handling of BLA and CRA pictures in MV-HEVC/SHVC

The current MV-HEVC/SHVC draft handles BLA pictures with nuh_layer_id greater than 0 by setting the variable NoRaslOutputFlag equal to 1. Similarly, the current MV-HEVC/SHVC draft allows to set the value of the variable HandleCraAsBlaFlag by external means, and the variable NoRaslOutputFlag for a CRA picture with nuh_layer_id greater than 0 is set equal to the value of HandleCraAsBlaFlag. NoRaslOutputFlag equal to 1 has the following consequences:

- When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, all reference pictures (if any) that have nuh_layer_id equal to nuh_layer_id value of the current picture and that are currently in the DPB are marked as "unused for reference".

- When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is not equal to 0 and the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures is invoked, which enables decoding of RASL pictures associated with the IRAP picture (while the RASL pictures are likely to have incorrect sample values).

- The RASL pictures associated with the BLA or CRA picture with NoRaslOutputFlag equal to 1 are not output (i.e. PicOutputFlag of those RASL pictures is set equal to 0).

## Identified issues

### EOS not causing the cross-layer impacts

It is asserted that the end of sequence (EOS) NAL unit (which can only have nuh_layer_id equal to 0) has no cross-layer impact. It is merely required that the access unit following an EOS NAL unit is an IRAP access unit:

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit.

The layers with nuh_layer_id greater than 0 can have inter prediction dependencies from pictures preceding the EOS NAL unit in decoding order to the pictures following the EOS NAL unit in decoding order. As an EOS NAL unit is supposed to specify an end of a CVS (across layers), it is suggested that the EOS NAL unit should have the same impact as IRAP pictures with NoClrasOutputFlag equal to 1.

### BLA with nuh_layer_id greater than 0 not causing cross-layer impacts

It is remarked that BLA pictures (or CRA pictures with HandleCraAsBlaFlag equal to 1) at layers with nuh_layer_id greater than 0 cause no impact on the layers predicted from them. The proponent thinks it would be logical to infer a cross-layer impact of the BLA pictures to the predicted layers.

Additionally, the following issues were identified related to BLA/CRA pictures and layer switching performed by middle-boxes.

- State machine needed for layer up-switching

When there is a middle-box that selects the number of layers to be forwarded, layer down-switching can take place at any access unit, but layer up-switching requires the following steps:

- Layer up-switching can take place only at an IRAP picture at the switch-to layer and all the reference layers of the IRAP picture need to be decodable at the point of the IRAP picture. The middle-box needs to therefore i) find IRAP pictures at the switch-to layer, and ii) keep track which reference layers are correctly decodable (similarly to maintaining the LayerInitializedFlag[ layerId ] variable of the MV-HEVC/SHVC decoding process).

- If layer up-switching is about to take place at a CRA picture, the middle-box has to rewrite the CRA NAL unit type to BLA NAL unit type (or use some external means to convey the information of the up-switching at the CRA picture to the decoder).

- The RASL pictures associated with the IRAP picture of the up-switch point are not decodable correctly. The middle-box can reduce the bitrate of the forwarded stream by omitting these RASL pictures from the forwarded stream.

The steps above need to be performed for each potential switch-to layer separately.

Examples of layer up-switching are presented with the example bitstream illustrated in Figure 1. It is assumed that each picture uses inter-layer prediction from the lower layer (EL1 predicts from BL, and EL2 predicts from EL1). It is further assumed that hierarchical inter prediction is in use with GOP size of 8. This bitstream is the output of the encoder and the input of the middle-box prior to any layer down- or up-switching operations.

Figure 1. An example of a 3-layer bitstream.

In Figure 2, the middle-box determines to switch down from forwarding three layers to forwarding only one layer at picture 9. Subsequently, the middle-box determines to switch-up layers at access unit H. As only EL1 contains an IRAP picture at that point, the middle-box starts to forward EL1 but not EL2. The middle-box can convert the CRA NAL unit type of picture H at EL1 to a BLA NAL unit type, or pass information to the decoder that HandleCraAsBlaFlag shall be set to 1 for the CRA picture (the latter option is illustrated in the figure). The middle-box can but needs not omit the forwarding of the associated RASL pictures (pictures I to O of EL1 in the figure). The latter one of these options is illustrated in the figure.

Figure 2. An example of a forwarded bitstream resulting into a proper layer up-switch.

Subsequently, the middle-box may also up-switch back to EL2, as illustrated in Figure 3 (now illustrated with the option to change the CRA nal_unit_type to BLA nal_unit_type).

Figure 3 An example of a forwarded bitstream resulting into a proper layer up-switch.

Figure 4 illustrates a case where a middle-box forwards pictures carelessly, resulting into a decoding error. As in the previous example, the middle-box determines to switch down from forwarding three layers to forwarding only one layer at picture 9. Subsequently, the middle-box determines to switch-up layers at access unit H. It is assumed that the middle-box contains insufficient up-switching logic and it therefore starts to forward pictures of both EL1 and EL2 at access unit H. Even if the middle-box handled EL1 correctly by turning the CRA picture H into a BLA picture by changing its NAL unit type or through external means to the decoder, pictures H onwards of EL2 are not decodable. In fact, some references pictures of picture H onwards of EL2 are missing and consequently the RPS decoding of picture H onwards of EL2 is likely to lead to a detected error.

Figure 4. An example of a forwarded bitstream resulting into an error in decoding.

- Decoded picture buffer is used inefficiently.

When pictures of a layer are no longer encoded or transmitted, the pictures within the layer that are marked as "used for reference" remain marked as "used for reference" and will not be removed from the DPB. Only the next picture in the same layer or a base-layer BLA, IDR with cross_layer_bla_flag equal to 1, end-of-bitstream or end-of-sequence NAL unit can cause the marking of these pictures as "unused for reference". Such inefficient use of DPB space may happen for example when layer down-switching is performed by a middle-box. In the case of layer down-switching, pictures of the former highest layer will remain to be marked as "used for reference". While such inefficient use of the DPB causes no violation of the level limits or the indicated DPB size, it disables more efficient dynamic use of the DPB memory.

- Decoder not capable of concluding whether an intentional temporary layer down-switch or an unintentional picture loss has happened.

The SPS VUI flags fixed_pic_rate_general_flag[ i ] and fixed_pic_rate_within_cvs_flag[ i ] enable indicating that the picture rate is fixed. If a decoder does not receive a picture so that the fixed picture output rate is maintained, the decoder could conclude that the picture has been lost in transmission and perform loss resilience actions, such sending of feedback messages and/or requesting of retransmission. However, in scalable bitstreams a picture might be intentionally missing due to a layer down-switching operation performed by a middle-box, in which case the decoder should not perform loss resilience actions. Hence, it is desirable to provide decoders means to conclude between accidental loss and intentional layer down-switching.

## On layers not present SEI message

The layers not present SEI message provides a mechanism for signalling that VCL NAL units of particular layers indicated by the VPS are not present in a particular set of access units. The target access units are defined as the set of access units starting from the access unit containing the layers not present SEI message up to but not including the next access unit, in decoding order, that contains a layers not present change SEI message or the end of the CVS, whichever is earlier in decoding order. When a first layer is indicated to be absent with the SEI message, all layers that may be predicted from the first layer (according to the layer dependency information provided in the VPS) are also required to be indicated to be absent.

When it comes to the identified issues, the layers not present SEI message was found to be insufficient due to the following reasons (the enumeration matches that in the section above):

- The layers not present SEI message has no normative impact in the decoder. Hence, when performing layer up-switching, the middle-box has to omit sending of pictures prior to the next IRAP picture in the switch-to layer and, if that IRAP picture is a CRA picture, change its nal_unit_type to indicate a BLA picture.

- The layers not present SEI message has no normative decoding process and hence causes no change in marking of pictures, i.e. pictures are not marked as "unused for reference". However, decoders making use of the SEI message could free the DPB picture buffers of the layers that are no longer present.

- The layers not present SEI message can be used for determining between intentional layer down-switching and accidental picture losses.

# Proposal

## Overview

The proposal can be summarized as follows:

- It is proposed that if the current picture is the first picture that follows an end of sequence NAL unit (with nuh_layer_id equal to 0) in decoding order, NoClrasOutputFlag is set equal to 1. (This causes marking of all pictures in the DPB as "unused for reference" as well as setting LayerInitializedFlag and FirstPicInLayerDecodedFlag equal to 0 for all layers, which initializes the layer-wise start-up process.)

Item 1 is asserted to be functionally identical to an aspect in JCTVC-R0224.

- It is proposed that when nuh_layer_id is greater than 0, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause the following:

- LayerInitializedFlag and FirstPicInLayerDecodedFlag is set equal to 0 for the current layer and all layers directly or indirectly predicted from the current layer. (Consequently, layer-wise start-up is enforced for these layers.)

- Each picture in the DPB for the current layer and all layers directly or indirectly predicted from the current layer are marked as "unused for reference".

It is asserted that with these changes, the identified issue 1 is diminished to the extent that the middle-box can freely forward any layers after the first IRAP picture used for up-switching.

- It is proposed to allow EOS NAL unit with nuh_layer_id greater than 0.

It is proposed that the decoding of the EOS NAL unit with nuh_layer_id greater than 0 causes the same impacts as those above (2a and 2b) for the subsequent IRAP picture with the same nuh_layer_id. Furthermore, for the semantics of SEI messages applying for nuh_layer_id greater than 0, it is proposed that EOS NAL unit with nuh_layer_id greater than 0 is treated similarly to an end of sequence for the persistence of the SEI message. When present, an EOS NAL unit with a particular nuh_layer_id value shall be the last NAL unit with that particular nuh_layer_id value within an access unit, except an end of bitstream NAL unit with that nuh_layer_id value (when present).

In revision 2 of the contribution, the following additional constraint is proposed to avoid access units and picture units without any VCL NAL units resulting from an extraction process:

An EOS NAL unit with a particular nuh_layer_id value shall not be present in an access unit, unless a coded picture with that particular nuh_layer_id value is present in the same access unit.

- As part of the HRD specification, NoOutputOfPriorPicsFlag and predLayerBufferEmptyFlag are derived for IRAP pictures of independent layers with nuh_layer_id greater than 0 similarly to NoOutputOfPriorPicsFlag and crossLayerBufferEmptyFlag for base-layer IRAP pictures. When NoOutputOfPriorPicsFlag and predLayerBufferEmptyFlag are equal to 1, the pictures in the independent layer and all layers directly or indirectly predicted from the independent layers are not output.

The motivation for this change is to have a cross-layer impact caused by IRAP pictures specified consistently also in the HRD specification, where NoOutputOfPriorPicsFlag is derived and its impacts are enforced. The cross-layer impact is limited to an independent non-base layer and the layers predicted from it.

It is noted that as proposed in JCTVC-R0235/JCT3V-I0048, when a bitstream without a base layer (vps_base_layer_available_flag equal to 0) is proposed the layer with smallest nuh_layer_id value is treated in the decoding process the same way as the base layer. Among other things, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause a cross-layer impact.

It is asserted that the proposed item 2 makes BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 of the lowest layer of a layer tree (without a base layer) to cause a cross-layer impact both when the layer tree is decoded as part of a bitstream with a base layer and when the layer tree is extracted (with subclause F.10.3 of the AHG10 output text) to become a bitstream without a base layer.

It is asserted that the proposed item 3 makes the IRAP picture following an EOS NAL unit in the same non-base layer cause a cross-layer impact both when the layer tree is decoded as part of a bitstream with a base layer and when the layer tree is extracted (with subclause F.10.3 of the AHG10 output text) to become a bitstream without a base layer.

## Proposed specification text

### Item 1

A key part of the proposed specification text for the proposed item 1 is copied below (the added part indicated in turquoise highlight), while the detailed proposal is presented with change marks in the accompanying specification text document.

When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies:

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is the first picture that follows an end of sequence NAL unit with nuh_layer_id equal to 0 in decoding order, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture or a CRA picture with HandleCraAsBlaFlag equal to 1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is an IDR picture with cross_layer_bla_flag is equal to1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means, not specified in this Specification, is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

### Items 2 and 3

A key part of the proposed specification text for the proposed items 2 and 3 is copied below, while the detailed proposal is presented with change marks in the accompanying specification text document.

The variables LayerResetFlag and dolLayerId are derived as follows:

–	If the current picture is an IRAP picture and has nuh_layer_id nuhLayerId greater than 0, the following applies:

–	If the current picture is the first picture, in decoding order, that follows an end of sequence NAL unit with nuh_layer_id equal to nuhLayerId, LayerResetFlag is set equal to 1 and dolLayerId is set equal to the nuh_layer_id value of the current NAL unit.

–	Otherwise, if the current picture is a CRA picture with HandleCraAsBlaFlag equal to 1, an IDR picture with cross_layer_bla_flag is equal to 1 or a BLA picture, LayerResetFlag is set equal to 1 and dolLayerId is set equal to the nuh_layer_id value of the current NAL unit.

–	Otherwise, LayerResetFlag is set equal to 0.

–	When LayerResetFlag is equal to 1, the following applies:

–	The values of LayerInitializedFlag and FirstPicinLayerDecodedFlag are updated as follows:

dolLayerIdx = LayerIdxInVps[ dolLayerId ]
for( i = 0; i < NumPredictedLayers[ dolLayerId ]; i++ ) {
	iLayerId = PredictedLayerId[ dolLayerIdx ][ i ]
	LayerInitializedFlag[ iLayerId ] = 0
	FirstPicinLayerDecodedFlag[ iLayerId ] = 0
}

–	Each picture that is in the DPB and has nuh_layer_id equal to dolLayerId or, when NumPredictedLayers[ dolLayerId ] is greater than 0, any value of PredictedLayerId[ dolLayerId ][ i ] for the values of i in the range of 0 to NumPredictedLayers[ dolLayerId ] – 1, inclusive, is marked as "unused for reference".

–	Otherwise, LayerResetFlag is equal to 0.

    - Item 4

The process appears similarly in subclauses C.3.3 and C.5.2.2. Below a copy of the proposed additions in C.3.3 are presented:

–	When the current picture is an IRAP picture with any non-zero nuh_layer_id value indepLayerId such that NumDirectRefLayers[ indepLayerId ] is equal to 0, and with NoRaslOutputFlag equal to 1, the following ordered steps are applied:

1.	The variable predLayerBufferEmptyFlag is derived as follows:

–	If LayerResetFlag is equal to 1 for the current picture or an end of sequence NAL unit preceding the current picture, predLayerBufferEmptyFlag is set equal to 1.

–	Otherwise, predLayerBufferEmptyFlag is set equal to 0.

2.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

3.	When the value of NoOutputOfPriorPicsFlag derived for the decoder under test is equal to 1, the following applies for the HRD:

–	If the value of predLayerBufferEmptyFlag is equal to 1, all non-empty picture storage buffers in the sub-DPBs of the layer with nuh_layer_id equal to indepLayerId and all its direct and indirect predicted layers are emptied without output of the pictures they contain, and the sub-DPB fullness of the sub-DPBs of the layer with nuh_layer_id equal to indepLayerId and all its direct and indirect predicted layers is set equal to 0.

–	Otherwise (the value of predLayerBufferEmptyFlag is equal to 0), all non-empty picture storage buffers that contain pictures with nuh_layer_id equal to indepLayerId are emptied without output of the pictures they contain, and the sub-DPB fullness of the sub-DPB associated with nuh_layer_id equal to indepLayerId is decremented by the number of picture storage buffers that were emptied.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).