| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0154 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0036 |
| --- | --- |

| Title: | On Alternative Output Layer and Default Output Layer Indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document following is proposed:

- Proposal 1: A modification is proposed to the semantics of alt_output_layer_flag[ i ] to infer the value when it is not present. This is because alt_output_layer_flag[ i ] is only signaled for an output layer set when NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, however the value of alt_output_layer_flag[ i ] is used in the specification for any output layer set.

Additionally a modification is proposed where derivation of the variables AltOptLayerFlag[ i ] is no longer necessary.

- Proposal  2: A modification is proposed to the semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures. In r1 revision an alternative option is added for this based on JCTVC meeting discussion about allowing auxiliary layers to be output layers.

# Introduction

In JCTVC-Q1008 [3] and JCT3V-H1002 [2], alt_output_layer_flag[ i ] and  default_output_layer_idc  syntax elements are signalled in VPS extension.

In this document following is proposed:

- Proposal 1: A modification is proposed to the semantics of alt_output_layer_flag[ i ] to infer the value when it is not present. This is because alt_output_layer_flag[ i ] is only signaled for an output layer set when NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, however the value of alt_output_layer_flag[ i ] is used in the specification for any output layer set (e.g. in section F.7.4.4 in the semantics of  general_profile_compatibility_flag[ j ], and in section G.11.1.2)

Additionally a modification is proposed where derivation of the variables AltOptLayerFlag[ i ] is no longer necessary.

- Proposal  2: A modification is proposed to the semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures.

# Proposal

Proposal 1

A modification is proposed to the semantics of alt_output_layer_flag[ i ] to infer the value when it is not present. This is because alt_output_layer_flag[ i ] is only signaled for an output layer set when NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, however the value of alt_output_layer_flag[ i ] is used in the specification for any output layer set (e.g. in section F.7.4.4 in the semantics of  general_profile_compatibility_flag[ j ], and in section G.11.1.2).

Additionally a modification is proposed where derivation of the variables AltOptLayerFlag[ i ] is no longer necessary.

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| splitting_flag | u(1) |
| … |  |
| if( NumIndependentLayers > 1 ) |  |
| num_add_layer_sets | ue(v) |
| for( i = 0; i < num_add_layer_sets; i++ ) |  |
| for( j = 1; j < NumIndependentLayers; j++ ) |  |
| highest_layer_idx_plus1[ i ][ j ] | u(v) |
| if( NumLayerSets > 1 ) { |  |
| num_add_output_layer_sets | ue(v) |
| default_target_output_layer_idc | u(2) |
| } |  |
| NumOutputLayerSets = num_add_output_layer_sets + NumLayerSets |  |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( i >= NumLayerSets ) |  |
| layer_set_idx_for_output_layer_set_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ defaultTargetOutputLayerIdc = = 2 ) |  |
| for( j = 0; j < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| profile_level_tier_idx[ i ] | u(v) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |

The proposed modifications are highlighted in color below.

Additional modifications related to AltOptLayerFlag[ i ] are highlighted in color below.

alt_output_layer_flag[ i ] equal to 0 specifies that an alternative output layer is not used for any output layer in the i-th output layer set. alt_output_layer_flag[ i ] equal to 1 specifies that an alternative output layer may be used for the output layer in the i-th output layer set.

When not present, the value of alt_output_layer_flag[ i ] is inferred to be equal to 0.

–	If NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, the variable AltOptLayerFlag[ i ] is set equal to alt_output_layer_flag[ i ].

–	Otherwise, the variable AltOptLayerFlag[ i ] is set equal to 0.

AltOptLayerFlag[ 0 ] is set equal to 0.

NOTE 1 – When AltOptLayerFlagalt_output_layer_flag[ olsIdx ] is equal to 0, pictures that are not at the target output layers of the output layer set with index olsIdx are not output. When AltOptLayerFlag[ olsIdx ] is equal to 1 and a picture at the target output layer of the output layer set with index olsIdx is not present in an access unit or has PicOutputFlag equal to 0, a picture with highest nuh_layer_id among those pictures of the access unit for which PicOutputFlag is equal to 1 and which has nuh_layer_id value among the nuh_layer_id values of the direct and indirect reference layers of the target output layer is output.

For each value of olsIdx in the range of 0 to NumOutputLayerSets − 1, inclusive, the following applies:

–	When AltOptLayerFlagalt_output_layer_flag[ olsIdx ] is equal to 1, the value of pic_output_flag shall be the same in the slice headers of an access unit that have nuh_layer_id value equal to OlsHighestOutputLayerId[ olsIdx ] or equal to the nuh_layer_id value of any direct or indirect reference layer of the layer with nuh_layer_id equal to OlsHighestOutputLayerId[ olsIdx ].

–	Let olsBitstream be the output of the sub-bitstream extraction process with inputs of the current bitstream, TemporalId equal to 7 and layerIdListTarget equal to LayerSetLayerIdList[ LayerSetIdxForOutputLayerSet[ olsIdx ] ]. Let truncatedOlsBitstream be olsBitstream or be formed from the olsBitstream by removing access units preceding, in decoding order, any access unit with an IRAP picture having nuh_layer_id equal to 0. It is a requirement of bitstream conformance that when AltOptLayerFlagalt_output_layer_flag[ olsIdx ] is equal to 1, a bitstream that is formed by removing, from the truncatedOlsBitstream, any coded picture that is not used as a reference for prediction for any other picture and is not the only coded picture of an access unit is a conforming bitstream.

NOTE 2 – When AltOptLayerFlagalt_output_layer_flag[ olsIdx ] is equal to 1, encoders are required to set the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] such that these values suffice also when pictures of an alternative output layer are marked as "needed for output" in the HRD.

Sections F.7.4.4, and section G.11.1.2 are provided below for reference.

- F.7.4.4 Profile, tier and level semantics

If vps_base_layer_internal_flag is equal to 0 and the profile_tier_level( ) syntax structure is the first profile_tier_level( ) syntax structure in the VPS, all bits in the syntax structure shall be equal to 0 and decoders shall ignore the syntax structure. Otherwise, the semantics of the profile_tier_level( ) syntax structure are specified by the remaining part of the current subclause.

The profile_tier_level( ) syntax structure provides profile, tier and level to which an output layer set conforms. When the profile_tier_level( ) syntax structure is included in a vps_extension( ) syntax structure, the profile_tier_level_idx[ i ] syntax element of the vps_extension( ) syntax structure specifies which profile_tier_level( ) syntax structure applies to the i-th output layer set. When num_add_layer_sets is greater than 0 and i is in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive, the profile_tier_level( ) syntax structure identified by profile_tier_level_idx[ i ] applies to the output of the non-base layer subtree extraction process of subclause Error: Reference source not found with the input variable lsIdx set equal to LayerSetIdxForOutputLayerSet[ i ]. When the profile_tier_level( ) syntax structure is included in a VPS, but not in a vps_extension( ) syntax structure, it applies to the 0-th output layer set. When the profile_tier_level( ) syntax structure is included in an active SPS for the base layer, it applies to the 0-th output layer set. When the profile_tier_level( ) syntax structure is included in an active SPS for an independent non-base layer with nuh_layer_id equal to layerId, it applies to the 0-th output layer set in an output bitstream of the non-base layer subtree extraction process of subclause Error: Reference source not found with an input parameter lsIdx such that AssignedBaseLayerId[ lsIdx ] is equal to layerId.

For interpretation of the following semantics, CVS refers to the CVS subset associated with the layer set to which the profile_tier_level( ) syntax structure applies.

When the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_reserved_zero_44bits are not present, they are inferred to be equal to the corresponding values of the ( ptlIdx − 1 )-th profile_tier_level( ) syntax structure in the VPS extension.

When the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_interlaced_source_flag[ i ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_reserved_zero_44bits[ i ] are not present, they are inferred to be equal to the corresponding values of ( ptlIdx − 1 )-th profile_tier_level( ) syntax structure in the VPS extension.

The specifications in subclause 7.4.4 apply, with following modifications.

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex A or subclause Error: Reference source not found.

general_profile_idc, when general_profile_space is equal to 0, indicates a profile to which the CVS conforms as specified in Annex A or subclause Error: Reference source not found. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A or subclause Error: Reference source not found. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

general_profile_compatibility_flag[ j ] equal to 1, when general_profile_space is equal to 0, indicates that the CVS conforms to the profile indicated by general_profile_idc equal to i as specified in Annex A or subclause Error: Reference source not found. When general_profile_space is equal to 0, general_profile_compatibility_flag[ general_profile_idc ] shall be equal to 1. The value of general_profile_compatibility_flag[ j ] shall be equal to 0 for any value of j that is not specified as an allowed value of general_profile_idc in Annex A or subclause Error: Reference source not found.

A sequence of pictures picSeq is derived as follows:

–	If the profile_tier_level( ) syntax structure is included in an SPS, picSeq consists of the pictures in the CVS for which the SPS is the active SPS.

–	Otherwise, if the profile_tier_level( ) syntax structure is included in a VPS, but not in the vps_extension( ) syntax structure, picSeq consists of the pictures with nuh_layer_id equal to 0 in the CVS.

–	Otherwise, if the profile_tier_level( ) syntax structure is associated with an output layer set with alt_output_layer_flag[ i ] equal to 1, picSeq consists of the pictures of the output layer and its direct and indirect reference layers within the CVS.

–	Otherwise, picSeq consists of the pictures of the output layers of the associated output layer set within the CVS.

- G.11.1 Profiles

- G.11.1.1 General

- G.11.1.2 Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

–	VPSs shall have vps_base_layer_internal_flag equal to 1 only.

–	VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

Bitstreams containing output layer sets conforming to the Stereo Main profile shall obey the following constraints on the derived sub-bitstream for each of the output layer sets, with output layer set index olsIdx and layerSetIdx being the layer set for the output layer set, and the sub-bitstream being derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with the following inputs: the bitstream, tIdTarget equal to 6, and layerIdListTarget containing the nuh_layer_id values of the layers that are included in the layer set with the index layerSetIdx and are primary picture layers.

Bitstreams containing output layer sets conforming to the Stereo Main profile shall also obey the following constraints on the base layer bitstream derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with the bitstream, tIdTarget equal to 6, and with layerIdListTarget containing only one nuh_layer_id value that is equal to 0 as inputs.

The base layer bitstream derived from bitstreams conforming to the Stereo Main profile shall obey the following constraints:

–	The base layer bitstream shall obey all constraints of the Main profile specified in subclause A.3.2.

–	SPSs of the base layer bitstream shall have general_profile_idc equal to 1 or general_profile_compatibility_flag[ 1 ] equal to 1.

The derived sub-bitstream for an output layer set conforming to the Stereo Main profile shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have chroma_format_idc equal to 1 only.

–	CtbLog2SizeY derived from any active SPS of the sub-bitstream shall be in the range of 4 to 6, inclusive.

–	ScalabilityId[ j ][ smIdx ] shall be equal to 0 for any smIdx value not equal to 1 and for any value of j for the sub-bitstream.

–	The number of the layers in the sub-bitstream shall be equal to 2.

–	output_layer_flag[ olsIdx ][ j ] shall be equal to 1 for all values of j in the range of 0 to 1.

NOTE – This restriction implies that alt_output_layer_flag[ olsIdx ] is equal to 0.

- When ViewScalExtLayerFlag[ i ] is equal to 1 for the layer with nuh_layer_id equal to i in the sub-bitstream, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS RBSP for that layer.

- When ViewScalExtLayerFlag[ i ] is equal to 1 for the layer with nuh_layer_id equal to i in the sub-bitstream, num_scaled_ref_layer_offsets shall be equal to 0 in each active SPS for that layer.

- When ViewScalExtLayerFlag[ i ] is equal to 1 for the layer with nuh_layer_id equal to i in the sub-bitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all direct reference layers of that layer.

–	All active SPSs for the sub-bitstream shall have sps_range_extensions_flag equal to 0 only and sps_extension_6bits equal to 0 only.

–	All active PPSs for the sub-bitstream shall have pps_range_extensions_flag equal to 0 only and pps_extension_6bits equal to 0 only.

–	All active SPSs for the sub-bitstream shall have bit_depth_luma_minus8 equal to 0 only.

–	All active SPSs for the sub-bitstream shall have bit_depth_chroma_minus8 equal to 0 only.

–	When AuxId[ i ] and AuxId[ j ] are both equal to 0 and i is not equal to j, ViewOrderIndex[ i ] shall not be equal to ViewOrderIndex[ j ].

Proposal 2

A modification is proposed to the semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures. Two options are presented.

Option 1:

It is proposed that when default_output_layer_idc is equal to 0 the auxiliary picture layers are not output layers of the OLS and only primary picture layers are output layers of OLS. This modification is proposed because there is no SHVC/ MV-HEVC profile defined that contains decoding capability for the auxiliary picture layers and that since default_output_layer_idc values of 0 and 1 are aimed towards MV-HEVC and SHVC. Additionally it is asserted that this modification makes the behavior when default_output_layer_idc value is equal 0 to be consistent with the beahvior when the default_output_layer_idc value is equal to 1.

The proposed text modification to JCT3V-H1002 is highlighted below:

default_output_layer_idc specifies the derivation of the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive. default_output_layer_idc equal to 0 specifies that all layers with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to 0 in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are output layers of their respective OLSs. default_output_layer_idc equal to 1 specifies that only the layer with the highest value of nuh_layer_id such that nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to 0 in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, is an output layer of its OLS. default_output_layer_idc equal to 2 specifies that the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are specified with the syntax elements output_layer_flag[ i ][ j ]. The value of 3 for default_output_layer_idc is reserved for future use by ITU-T | ISO/IEC. Although the value of default_output_layer_idc is required to be less than 3 in this version of this Specification, decoders shall allow a value of default_output_layer_idc equal to 3 to appear in the syntax.

The variable defaultOutputLayerIdc is set equal to Min( default_output_layer_idc, 2 ).

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th OLS is an output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th OLS is not an output layer.

The value of output_layer_flag[ 0 ][ 0 ] is inferred to be equal to 1.

When defaultOutputLayerIdc is equal to 0 or 1, for i in the range of 0 to vps_num_layer_sets_minus1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is derived as follows:

–	If defaultOutputLayerIdc is equal to 0 and LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ] is equal to nuhLayerIdA with AuxId[ nuhLayerIdA ] equal to 0 or LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ] is equal to nuhLayerIdA, with nuhLayerIdA being the highest value in LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ] with AuxId[ nuhLayerIdA ] equal to 0, OutputLayerFlag[ i ][ j ] is set equal to 1.

–	Otherwise, OutputLayerFlag[ i ][ j ] is set equal to 0.

For i in the range of ( defaultOutputLayerIdc  = =  2 ) ? 0 : ( vps_num_layer_sets_minus1 + 1 ) to NumOutputLayerSets − 1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is set equal to output_layer_flag[ i ][ j ].

The variable NumOutputLayersInOutputLayerSet[ i ] is derived as follows:

NumOutputLayersInOutputLayerSet[ i ] = 0
for( j = 0 ; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++) { 
	NumOutputLayersInOutputLayerSet[ i ] += OutputLayerFlag[ i ][ j ]
	if( OutputLayerFlag[ i ][ j ] )
		OlsHighestOutputLayerId[ i ] = LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ]

It is a requirement of bitstream conformance that NumOutputLayersInOutputLayerSet[ i ] shall be greater than 0 for i in the range of 0 to NumOutputLayers − 1, inclusive.

Option 2:

Based on JCTVC Sapporo meeting discussion about allowing auxiliary layers to be output layers this alternative option is proposed. In this option it is proposed that when default_output_layer_idc is equal to 1 the highest output layer could be an auxiliary picture layer. It is asserted that this modification makes the behavior when default_output_layer_idc value is equal to 1 to be consistent with the beahvior when the default_output_layer_idc value is equal to 0.

The proposed text modification with respect to JCT3V-H1002 is shown below:

default_output_layer_idc specifies the derivation of the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive. default_output_layer_idc equal to 0 specifies that all layers in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are output layers of their respective OLSs. default_output_layer_idc equal to 1 specifies that only the layer with the highest value of nuh_layer_id such that nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to 0 in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, is an output layer of its OLS. default_output_layer_idc equal to 2 specifies that the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are specified with the syntax elements output_layer_flag[ i ][ j ]. The value of 3 for default_output_layer_idc is reserved for future use by ITU-T | ISO/IEC. Although the value of default_output_layer_idc is required to be less than 3 in this version of this Specification, decoders shall allow a value of default_output_layer_idc equal to 3 to appear in the syntax.

The variable defaultOutputLayerIdc is set equal to Min( default_output_layer_idc, 2 ).

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th OLS is an output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th OLS is not an output layer.

The value of output_layer_flag[ 0 ][ 0 ] is inferred to be equal to 1.

When defaultOutputLayerIdc is equal to 0 or 1, for i in the range of 0 to vps_num_layer_sets_minus1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is derived as follows:

–	If defaultOutputLayerIdc is equal to 0 or LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ] is equal to nuhLayerIdA, with nuhLayerIdA being the highest value in LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ] with AuxId[ nuhLayerIdA ] equal to 0, OutputLayerFlag[ i ][ j ] is set equal to 1.

–	Otherwise, OutputLayerFlag[ i ][ j ] is set equal to 0.

For i in the range of ( defaultOutputLayerIdc  = =  2 ) ? 0 : ( vps_num_layer_sets_minus1 + 1 ) to NumOutputLayerSets − 1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is set equal to output_layer_flag[ i ][ j ].

The variable NumOutputLayersInOutputLayerSet[ i ] is derived as follows:

NumOutputLayersInOutputLayerSet[ i ] = 0
for( j = 0 ; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++) { 
	NumOutputLayersInOutputLayerSet[ i ] += OutputLayerFlag[ i ][ j ]
	if( OutputLayerFlag[ i ][ j ] )
		OlsHighestOutputLayerId[ i ] = LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ]

It is a requirement of bitstream conformance that NumOutputLayersInOutputLayerSet[ i ] shall be greater than 0 for i in the range of 0 to NumOutputLayers − 1, inclusive.

# Conclusion

This document proposes following:

- Proposal 1: A modification is proposed to the semantics of alt_output_layer_flag[ i ] to infer the value when it is not present.

Additionally a modification is proposed where derivation of the variables AltOptLayerFlag[ i ] is no longer necessary.

- Proposal  2: A modification is proposed to the semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures.

We propose to adopt the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 6,” JCTVC-Q1008, Valencia, May 2014.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 8,” JCT3V-H1002, Valencia, May 2014.