| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0221 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0041 |
| --- | --- |

| Title: | HEVCv1/MV-HEVC/SHVC HLS: On extensibility of nesting SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego CA 92121 | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is asserted that the current specification of nesting SEI messages (in HEVC and SHVC/MV-HEVC) limits the extensibility of these SEI messages using the extension mechanism provided in the sei_payload( ) structure, and also put limitations on the location of the SEI messages in an SEI NAL unit. This document also reports other observations made on the non-extensibility of a few of the non-nesting SEI messages. It is proposed that a note be added to the semantics of the scalable nesting SEI message, highlighting the limitations associated with the SEI message. It is also proposed that the syntax structure of the bitstream partition nesting SEI message and output layer set nesting SEI message be modified such that the extensibility feature of sei_payload( ) is applicable to these SEI messages.

# Introduction

SEI messages are contained in SEI NAL units, and each SEI NAL unit may contain one or more SEI messages. Some SEI messages are nesting SEI messages, and they contain one or more SEI messages; each SEI message contained in a nesting SEI message is applicable to a set of layers, operation points, output layer sets, or bitstream partitions as specified by the nesting SEI message.

The SHVC/MV-HEVC specification currently has three such nesting SEI messages:

- Scalable nesting SEI message

- Bitstream partition nesting SEI message

- Output layer set (OLS) nesting SEI message

# Syntax structure

The overall structure of an SEI NAL unit containing one or more SEI messages is illustrated in Figure 1. sei_rbsp( ) may contain one or more sei_message( ) structures and each sei_message( ) structure contains an sei_payload( ) structure. The syntax structure of an SEI message is contained in sei_payload( ) and represented as xxx( payloadSize ) in the figure. Optionally, sei_payload( ) may also contain payload extension which would be useful for future extensibility of any SEI message.

Figure 1: Basic structure of an SEI NAL unit. The structure xxx( payloadSize ) represents the syntax structure of any SEI message.

# Non-extensibility of nesting SEI messages (as specified currently)

## Limitations of the scalable nesting SEI message

The syntax structure of the scalable nesting SEI message is given below:

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bitstream_subset_flag | u(1) |
| … |  |
| nesting_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

In particular, the highlighted syntax allows one or more than one SEI message to be nested in the scalable nesting SEI message. However, the function more_rbsp_data( ) returns a value 1 until the current byte position in the RBSP is equal to the position of the last 1 in the RBSP, which corresponds to the 1 in the rbsp_trailing_bits( ) present at the end of the SEI RBSP. Therefore, if any data that is present in an SEI NAL unit that contains a scalable nesting SEI message, follows the sei_payload( ) containing the structure scalable_nesting( ) and precedes the rbsp_trailing_bits( ) structure would be considered as sei_message( ) in the scalable nesting SEI message and this would definitely lead to a parsing error. Hence, a bitstream cannot be parsed (and hence non-conforming) if any of the following is true for an SEI NAL unit that contains a scalable nesting SEI message and is present in the bitstream:

- The SEI NAL unit contains any SEI message that follows, in decoding order, the sei_payload( ) containing the scalable nesting SEI message.

- The sei_payload( ) containing the scalable nesting SEI message contains payload extension data i.e. any of the following syntax elements – reserved_payload_extension_data, payload_bit_equal_to_one, and payload_bit_equal_to_zero.

Consequently, the scalable nesting SEI message cannot be extended using the extension mechanism provided in the sei_payload( ) structure.

## Bitstream partition nesting SEI message

As specified currently, the syntax of bitstream partition nesting SEI message also has similar syntax structure as the scalable nesting SEI message. Hence, it shares the same limitations as described for the scalable nesting SEI message in Section 3.1.

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

## OLS nesting SEI message

Inclusion of multiple SEI messages inside the OLS nesting SEI message is done differently when compared to the scalable nesting SEI message and the bitstream partition nesting SEI message. The syntax structure is parsed as long as there is data in the payload (using the function more_data_in_payload( )), and hence no payload extension can be present in an sei_payload( ) structure that contains the ols_nesting( ) structure. However, there can be SEI messages that succeed the OLS nesting SEI message in the same SEI NAL unit, unlike the other two nesting SEI messages.

| ols_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| ols_flag | u(1) |
| num_ols_indices_minus1 | ue(v) |
| for( i = 0; i <= num_ols_indices_minus1; i++ ) |  |
| ols_idx[ i ] | ue(v) |
| while( !byte_aligned( ) ) |  |
| ols_nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_data_in_payload( ) ) |  |
| } |  |

# Using payloadSize information in the parsing of the SEI message

The variable payloadSize in the sei_message( ) structure denotes the number of RBSP bytes in the SEI message payload. This includes all the bytes of the sei_payload( ), including the payload extension, when present. However, several SEI message syntax structures include all the payloadSize bytes within the specific SEI message syntax structure such as recovery_point( ), thus leaving no room for any payload extension to be present. The SEI messages currently defined in the Specification that fall under this category are:

- Filler payload SEI message

- User data registered

- User data unregistered

- Reserved SEI message

- VPS rewriting SEI message

# Proposal

## Scalable nesting SEI message

The syntax structure of the scalable nesting SEI message is already implemented by HEVC version 1 decoders, and hence it would not be backward compatible to change the syntax structure of the scalable nesting SEI message. Hence, it is proposed that a note be added to the semantics of the scalable nesting SEI message to indicate that when a scalable nesting SEI message is included in an SEI NAL unit, it can only be the last SEI message in the SEI NAL unit and that scalable nesting SEI cannot contain any payload_extension_data_byte syntax elements. The proposed addition is given below:

NOTE – A scalable nesting SEI message can only be the last SEI message in the SEI NAL unit containing the scalable nesting SEI message. The sei_payload( ) of the scalable nesting SEI message cannot contain any reserved_payload_extension_data.

It is also noteworthy to mention that the restriction that the scalable nesting SEI message can only be the last SEI message in the SEI NAL unit containing the scalable nesting SEI message is also applicable to AVC.

## Changes to other nesting SEI messages

The semantics of the bitstream partition nesting SEI message and the output layer set nesting SEI message are proposed to be changed as follows so that these two SEI messages do not have the same limitations as the scalable nesting SEI message. With these changes, these two SEI messages are not constrained to be the last SEI messages of the SEI NAL unit containing these messages, and they may also be extended using the reserved_payload_extension_data syntax element.

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_bsp_minus1 | ue(v) |
| for( i = 0; i <= num_seis_in_bsp_minus1; i++ ) |  |
| sei_message( ) |  |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

num_seis_in_bsp_minus1 plus 1 specifies the number of sei_message( ) structures contained in the bsp_nesting( ) syntax structure. The value of num_seis_in_bsp_minus1 shall be in the range of 0 to 15, inclusive.

| ols_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| ols_flag | u(1) |
| num_ols_indices_minus1 | ue(v) |
| for( i = 0; i <= num_ols_indices_minus1; i++ ) |  |
| ols_idx[ i ] | ue(v) |
| while( !byte_aligned( ) ) |  |
| ols_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_ols_minus1 | ue(v) |
| for( i = 0; i <= num_seis_in_ols_minus1; i++ ) |  |
| sei_message( ) |  |
| do |  |
| sei_message( ) |  |
| while( more_data_in_payload( ) ) |  |
| } |  |

num_seis_in_ols_minus1 plus 1 specifies the number of sei_message( ) structures contained in the ols_nesting( ) syntax structure. The value of num_seis_in_ols_minus1 shall be in the range of 0 to 15, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).