| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0223 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0042 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses a few cases related to multi-layer POC signalling and derivation, and asserts that some changes to the POC-resetting related restrictions would be desirable. It is proposed to 1) handle missing pictures and discardable pictures in the same manner as different picture types across layers in requiring POC resetting, 2) allow mixing of CRA and BLA pictures without requiring POC resetting; and 3) explicitly spell out a restriction on the value of full_poc_reset_flag based on the value of poc_reset_idc of pictures in the first access unit in the same POC resetting period.

# Problem statements

It is asserted that the current specification related to POC signalling and derivation has some shortcomings as given in the following subsections.

## On absent pictures

Some constraints specified related to the value of poc_reset_idc are as follows:

It is a requirement of bitstream conformance that the following constraints apply:

–	The value of poc_reset_idc shall not be equal to 1 or 2 for a RASL picture, a RADL picture, a sub-layer non-reference picture, or a picture that has TemporalId greater than 0, or a picture that has discardable_flag equal to 1.

–	The value of poc_reset_idc of all pictures in an access unit shall be the same.

–	When the picture in an access unit with nuh_layer_id equal to 0 is an IRAP picture with a particular value of nal_unit_type and there is at least one other picture in the same access unit with a different value of nal_unit_type, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	When there is at least one picture that has nuh_layer_id greater than 0 and that is an IDR picture with a particular value of nal_unit_type in an access unit and there is at least one other picture in the same access unit with a different value of nal_unit_type, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	The value of poc_reset_idc of a CRA or BLA picture shall be less than 3.

–	When the picture with nuh_layer_id equal to 0 in an access unit is an IDR picture and there is at least one non-IDR picture in the same access unit, the value of poc_reset_idc shall be equal to 2 for all pictures in the access unit.

–	When the picture with nuh_layer_id equal to 0 in an access unit is not an IDR picture, the value of poc_reset_idc shall not be equal to 2 for any picture in the access unit.

Of all the above constraints, the case where an access unit (AU) that has pictures not present for some of the layers and has one IRAP picture in at least one layer is not covered. Figure 1 illustrates an example of an AU (i.e. AU D) containing an IDR picture in the base layer (BL) but no picture in the enhancement layer (EL). In such a situation, since the EL picture is missing but there is an IDR picture in the BL, POC reset is needed because the POC of the IDR picture in the BL must be equal to 0 and yet the POC chain in the enhancement layer continues. Failing to reset POC of the enhancement layer picture at AU D could result in derived POC being not cross-layer aligned for pictures in the subsequent AU. However, none of the above restrictions for the value of poc_reset_idc are applicable for this situation and hence POC reset is not mandated for AU D.

Figure 1 - The case of an AU with IRAP picture in the BL and missing picture in the EL

Figure 2 illustrates another example of an AU (i.e. AU D) containing an IDR picture in the EL but no picture in the BL. Again in such a situation, POC reset is needed because the POC MSB of the IDR picture in the EL will be reset to 0. However, currently POC reset is not mandated for AU D.

Figure 2 - The case of an AU with missing picture in the BL and IDR picture in the EL

## On discardable pictures

The above problem on absent pictures applies similarly to discardable pictures, which after being discarded would become absent pictures.

When IRAP pictures are cross-layer aligned, POC reset is not required by the current specification. However, there is no restriction that prohibits an IRAP picture to be marked as a discardable picture. When an access unit auA contains cross-layer aligned IRAP pictures and at least one of the IRAP pictures is discardable, derivation of POC for the pictures in the subsequent AUs, in decoding order, may be incorrect if the discardable IRAP picture in auA was removed as illustrated in Figure 3. When the EL IDR picture in AU E is discarded, the value of POC MSB of the EL picture in AU F will be derived to be equal to the value of POC MSB of the previous anchor picture (i.e. 256). This results in different POC values derived for pictures in the BL and the EL for AU F.

Figure 3 - POC derivation problem when one of IRAP in the AU that contained cross-layer aligned IRAP is a discardable picture.

## On full_poc_reset_flag

When poc_reset_idc is equal to 3, full_poc_reset_flag is signalled to indicate whether both POC MSB and LSB shall be reset or only POC MSB shall be reset. The value of full_poc_reset_flag indicates the value of poc_reset_idc of the first access unit in the same POC resetting period. If the wrong value of the full_poc_reset_flag is signalled, the decoder could decrement the POC of earlier pictures by the wrong value. Figure 4 illustrates an example where the encoder assigns an incorrect value for full_poc_reset_flag. Since the value of poc_reset_idc of the picture in AU E is 2, which means that both POC MSB and LSB are reset, the value of full_poc_reset_flag, when present, in the pictures that succeed, in decoding order, AU E and that belong to the same POC resetting period as AU E, must be 1 to guarantee that correct value of DeltaPocVal is derived to decrement the value of POC of earlier pictures. Since the value of full_poc_reset_flag is incorrectly signalled as 0 for the EL picture in AU G, the value use to decrement the POC of pictures of Layer 1 that precede, in decoding order, AU E is 64 instead of 102. The incorrect POC values of earlier pictures affect the RPS derivation process as the decoder cannot find the correct reference pictures for the current picture. In the illustrated example, during the RPS derivation of the EL picture of AU G, decoder cannot find reference pictures with POC equal to −1 and −2 and hence these reference pictures are marked "unavailable for reference."

Figure 4 - Example of problem when encoder assigns wrong value for full_poc_reset_flag.

## On mixing of CRA and BLA pictures

According to the current Specification, when a picture in an access unit has nuh_layer_id equal to 0 and is an IRAP picture with a particular value of nal_unit_type, and there is at least one other picture in the same access unit with a different value of nal_unit_type, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit. This means that when an access unit in a 2-layer bitstream contains a CRA picture in the BL and a BLA picture in the enhancement layer, POC reset, either resetting POC MSB only or resetting both POC MSB and LSB, is required. Such constraint is not good in some use-cases.

Figure 5 illustrates a layer switch-down and switch-up example in which layer switch-up is conducted at the AU that contains cross-layer aligned CRA pictures. As it is not required to do POC reset when nal_unit_types of IRAP pictures are cross-layer aligned, encoder may not signal any POC reset information (e.g., poc_reset_idc, poc_reset_period_id, etc) in the slice segment header extension in the CRA pictures of the AU. However, for layer switch-up, the system may change the NAL unit type of the EL picture of the AU from CRA to BLA. In this case, the constraint implies that, in addition to changing the NAL unit type, the system entity that performs the switch-up operation also needs to modify the slice segment header extension of the pictures to insert POC reset information. Such additional burden imposed on the system entity is unnecessary because POC reset is not really needed in the illustrated situation. This applies similarly for splicing bitstreams at cross-layer aligned CRA pictures where only the base layer CRA picture is changed to be a BLA picture.

Figure 5 - Example of layer switch-down and switch-up at cross-layered CRA.

# Summary of proposals

To solve the problems described in Section 1, the following changes are proposed:

- Update the constraints related to the value of poc_reset_idc to cover cases where poc_reset_idc has be equal to 1 or 2 for all pictures in an AU  when there is at least one missing picture or discardable picture in the AU that contains an IRAP picture at the BL or an IDR picture in an EL.

- Add a new constraint for full_poc_reset_flag to ensure that its value is synchronized with the value of poc_reset_idc of the first AU in the same poc resetting period.

- Relax the existing constraint for poc_reset_idc to ensure that when an AU contains only CRA or BLA pictures (but with any type of BLA picture), poc_reset_idc is not required to be equal to 1 or 2.

# Proposed text modifications

The text modifications to implement the above proposals are as follows:

poc_reset_idc equal to 0 specifies that neither the most significant bits nor the least significant bits of the picture order count value for the current picture are reset. poc_reset_idc equal to 1 specifies that only the most significant bits of the picture order count value for the current picture may be reset. poc_reset_idc equal to 2 specifies that both the most significant bits and the least significant bits of the picture order count value for the current picture may be reset. poc_reset_idc equal to 3 specifies that either only the most significant bits or both the most significant bits and the least significant bits of the picture order count value for the current picture may be reset and additional picture order count information is signalled. When not present, the value of poc_reset_idc is inferred to be equal to 0.

When an access unit auA has an IRAP picture with nuh_layer_id equal to 0 or an IDR picture with nuh_layer_id greater than 0, the variable absentPictureInAuFlag is derived as follows for the access unit auA:

–	If any of the following two conditions is true, the value of absentPictureInAuFlag is set equal to 1:

–	There is at least one picture with discardable_flag equal to 1 in the access unit auA.

–	Let auB be the first access unit that follows auA in decoding order and belongs to a different POC resetting period than the access unit auA, when present. Let auC be the first access unit that follows auA in decoding order and has an IRAP picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1, when present. Let auD be the first access unit that follows auA in decoding order and has an IRAP picture picture in each of the layers in the bitstream, when present. There is no picture with nuh_layer equal to any particular value of nuh_layer_id layerIdA in the access unit auA, there is a picture picA with nuh_layer_id equal to layerIdA that follows auA in decoding order, and either none of auB, auC, and auD exists in the bitstream or the picture picA precedes, in decoding order, the first of auB (when present), auC (when present), and auD (when present) in decoding order.

–	Otherwise, the value of absentPictureInAuFlag is set to be 0.

It is a requirement of bitstream conformance that the following constraints apply:

–	The value of poc_reset_idc shall not be equal to 1 or 2 for a RASL picture, a RADL picture, a sub-layer non-reference picture, or a picture that has TemporalId greater than 0, or a picture that has discardable_flag equal to 1.

–	The value of poc_reset_idc of all pictures in an access unit shall be the same.

–	When the picture in an access unit with nuh_layer_id equal to 0 is an IRAP picture, with a particular value of nal_unit_type and there is at least one other picture in the same access unit that is not an IRAP picture or absentPictureInAuFlag is equal to 1 with a different value of nal_unit_type, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	When there is at least one picture that has nuh_layer_id greater than 0 and that is an IDR picture with a particular value of nal_unit_type in an access unit and there is at least one other picture in the same access unit with a different value of nal_unit_type or absentPictureInAuFlag is equal to 1, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	The value of poc_reset_idc of a CRA or BLA picture shall be less than 3.

–	When the picture with nuh_layer_id equal to 0 in an access unit is an IDR picture and there is at least one non-IDR picture in the same access unit or absentPictureInAuFlag is equal to 1, the value of poc_reset_idc shall be equal to 2 for all pictures in the access unit.

–	When the picture with nuh_layer_id equal to 0 in an access unit is not an IDR picture, the value of poc_reset_idc shall not be equal to 2 for any picture in the access unit.

The value of poc_reset_idc of an access unit is the value of poc_reset_idc of the pictures in the access unit.

poc_reset_period_id identifies a POC resetting period. There shall be no two pictures consecutive in decoding order in the same layer that have the same value of poc_reset_period_id and poc_reset_idc equal to 1 or 2. When not present, the value of poc_reset_period_id is inferred as follows:

–	If the previous picture picA that has poc_reset_period_id present in the slice segment header in present in the same layer of the bitstream as the current picture, the value of poc_reset_period_id is inferred to be equal to the value of the poc_reset_period_id of picA.

–	Otherwise, the value of poc_reset_period_id is inferred to be equal to 0.

NOTE – It is not prohibited for multiple pictures in a layer to have the same value of poc_reset_period_id and to have poc_reset_idc equal to 1 or 2 unless such pictures occur in two consecutive access units in decoding order. To minimize the likelihood of such two pictures appearing in the bitstream due to picture losses, bitstream extraction, seeking, or splicing operations, encoders should set the value of poc_reset_period_id to be a random value for each POC resetting period (subject to the constraints specified above).

It is a requirement of bitstream conformance that the following constraints apply:

–	One POC resetting period shall not include more than one access unit with poc_reset_idc equal to 1 or 2.

–	An access unit with poc_reset_idc equal to 1 or 2 shall be the first access unit in a POC resetting period.

–	A picture that follows, in decoding order, the first POC resetting picture among all layers of a POC resetting period in decoding order shall not precede, in output order, another picture in any layer that precedes the first POC resetting picture in decoding order.

full_poc_reset_flag equal to 1 specifies that both the most significant bits and the least significant bits of the picture order count value for the current picture are reset when the previous picture in decoding order in the same layer does not belong to the same POC resetting period. full_poc_reset_flag equal to 0 specifies that only the most significant bits of the picture order count value for the current picture are reset when the previous picture in decoding order in the same layer does not belong to the same POC resetting period.

It is a requirement of bitstream conformance that when the value of poc_reset_idc of pictures in the first access unit in the same POC resetting period is equal to 1 or 2, the value of full_poc_reset_flag, when present, shall be equal to poc_reset_idc − 1.

# Study case

Figure 6Error: Reference source not found illustrates examples of cases of AUs that have missing pictures. AU A does not need to reset POC because the missing pictures are in Layer 2 and there are no pictures in that layer until AU B that does reset POC. AU B needs to reset POC because the second AU after AU B has picture in Layer 2, unless that second AU itself does POC reset (here we assume it does not). AU C needs to reset POC because it has IDR in Layer 0 but non-IRAP in other layers. AU D does not need to reset POC because the AU has cross-layer aligned IDR and no missing picture. Finally, AU E needs to reset POC because the second AU after AU E has picture in Layer 2, again unless that second AU itself does POC reset (here we assume it does not).

Figure 6 – Example of cases where POC reset is required and not required.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).