| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0224 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0043 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Clarifications to the specification of end of sequence (EOS) NAL unit |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121 | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is asserted that the end of sequence (EOS) NAL unit directly affects the decoding of pictures in more than one access unit that follows the EOS NAL unit in decoding order. It is argued that this is not the intended design and that the EOS NAL unit should only directly affect the decoding of pictures in the first access unit that follows the EOS NAL unit in decoding order. It is also asserted that in the derivation of the variable NoClrasOutputFlag for IRAP pictures with nuh_layer_id equal to 0, CRA pictures with HandleCraAsBlaFlag equal to 1 are currently treated differently from CRA pictures in the first AU that immediately follow an EOS NAL unit in decoding order. It is proposed that these two cases of CRA pictures should be treated the same way as for BLA pictures.

# Problem Statement

The parts of the Specification relevant to this discussion are given below.

- coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an initial IRAP access unit, followed by zero or more access units that are not initial IRAP access units, including all subsequent access units up to but not including any subsequent access unit that is an initial IRAP access unit.

...

- initial intra random access point (IRAP) access unit: An IRAP access unit in which the coded picture with nuh_layer_id equal to 0 has NoRaslOutputFlag equal to 1.

- ...

- intra random access point (IRAP) access unit: An access unit in which the coded picture with nuh_layer_id equal to 0 is an IRAP picture.

...

When the current picture is an IRAP picture, the variable HandleCraAsBlaFlag is derived as specified in the following:

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable HandleCraAsBlaFlag is set equal to the value provided by the external means.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0.

When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies:

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture or a CRA picture with HandleCraAsBlaFlag equal to 1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is an IDR picture with cross_layer_bla_flag is equal to1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means, not specified in this Specification, is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

–	When NoClrasOutputFlag is equal to 1, the variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive.

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and LayerInitializedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

In the section on order of access units and their association to CVS, the following constraint is specified.

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit.

From the above paragraphs, the following observations can be made for conforming bitstreams:

- The EOS NAL unit currently affects decoding of not just the pictures in the first access unit that succeeds the EOS NAL unit in decoding order, but also subsequent AUs. If an IRAP picture of a particular layer is the first picture in that layer following an EOS NAL unit, then the NoRaslOutputFlag of the picture is equal to 1 (per the above spec text highlighted in cyan) even that IRAP picture is contained in an AU that is not immediately following the EOS NAL unit in decoding order. In the bitstream illustrated in Figure 1, the presence of the EOS in AU n+1 results in the CRA picture in AU n+5 to have NoRaslOutputFlag equal to 1. This was not the intended design of the EOS NAL unit, and it would be better to restrict the EOS NAL unit to only directly affect the decoding of pictures in the first subsequent access unit, because if the AU n+2 is not a splicing point the RASL pictures at the AUs n+6 and n+7 can be correctly decoded. In the case where the AU n+2 is a splice point, the current derivation of NoRaslOutputFlag based on LayerInitializedFlag already ensures that the CRA picture in AU n+5 has NoRaslOutputFlag equal to 1 (per the above spec text highlighted in pink).

Figure 1: Decoding of CRA picture in AU n+5 is still affected by the present of EOS NAL unit in AU n+1.

- The text in grey shows that when a CRA picture with nuh_layer_id equal to 0 has HandleCraAsBlaFlag equal to 1, the value of NoClrasOutputFlag is derived to be equal to 1. However, when a CRA picture with nuh_layer_id equal to 0 is the first picture that succeeds an EOS NAL unit in decoding order, although the value of NoRaslOutputFlag is derived to be equal to 1, yet the value of NoClrasOutputFlag is not derived to be equal to 1, as illustrated in Figure 2 and Figure 3. We believe that this inconsistency is a bug due to an overlook, and the value of NoClrasOutputFlag should be also derived to be equal to 1 for a CRA picture with nuh_layer_id equal to 0 that is the first picture succeeding an EOS NAL unit in decoding order. CRA pictures with nuh_layer_id equal to 0 contained in the first IRAP AU following an EOS NAL unit in decoding order and CRA pictures with HandleCraAsBlaFlag equal to 1 should both be equivalently handled in the same way as a BLA picture.

Figure 2: For a CRA picture with HandleCraAsBlaFlag equal to 1, the value of NoClrasOutputFlag is derived to be equal to 1.

Figure 3: For a CRA picture that immediately follows an EOS NAL unit, the value of NoClrasOutputFlag is currently derived to be equal to 0.

# Solution

The following changes (highlighted in yellow) to the derivation of NoClrasOutputFlag and NoRaslOutputFlag are proposed to complete the Specification of EOS NAL unit and to make it more aligned with the design intent of only directly affecting the decoding of the first subsequent AU.

When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies:

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture or a CRA picture with HandleCraAsBlaFlag equal to 1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a CRA picture and the current access unit is the first access unit that follows an end of sequence NAL unit in decoding order, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is an IDR picture with cross_layer_bla_flag is equal to1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means, not specified in this Specification, is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

–	When NoClrasOutputFlag is equal to 1, the variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive.

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the access unit containing the picture with that particular value of nuh_layer_id is the first access unit that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and LayerInitializedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).