| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0230 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0046 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Unavailable and useless stuff |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Hendry Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com fhendry@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses unavailable base layer, operation points, and output operation points, and how to handle them in bitstream conformance tests. This document also discusses "useless" layers that are neither output layers nor needed directly or indirectly for decoding output layers.

The following are proposed:

- Define fully available (output) operation points and only test fully available (output) operation points in bitstream conformance tests.

- Remove the vps_base_layer_available_flag, and add an SEI message to indicate which (output) operation points are fully available as well as whether the base layer is available. The SEI message can be either provided in the bitstream or through external means. It is suggested to discuss whether to use the information included in the SEI message in the specification of bitstream conformance tests to test only those (output) operation points that are fully available.

# Discussion

The AHG10 output text in the attachment of JCTVC-R0010v3 includes the specification of conforming bitstreams with an unavailable base layer, as indicated by the syntax element vps_base_layer_available_flag equal to 0. Such a bitstream can be generated by extracting an additional layer set, which corresponds to an independent set of layers not containing the base layer specified in the VPS extension. During the extraction, vps_base_layer_available_flag is set equal to 0.

The VPS has been designed such that when some layers or sub-layers are removed from the bitstream, the VPS itself does not need to be changed. In other words, the VPS can contain information that describes layers, sub-layers, layer sets, output layer sets (OLSs), operation points, and output operation points that are not present in the bitstream. However, on the other hand, it is required that each sub-bitstream corresponding to an operation point or output operation point corresponding to a layer set and a target highest value of TemporalId, OpTid, less than or equal to 6 for which the layer set is specified by the VPS shall be a conforming bitstream. When the base layer is allowed to be not included in a layer set, such a sub-bitstream may contain zero layers in it (i.e. there are no VCL NAL units). Requiring such an empty sub-bitstream to be a conforming bitstream seems to not make sense.

Since some layers for which information is included in the VPS may be absent in the bitstream and not indicated in the VPS anyway, there does not seem to be good reason to indicate in the VPS that a base layer is not available (i.e. neither in the bitstream nor provided by external means). Actually, vps_base_layer_available_flag is only used to specify a requirement for the external means that provides the value of TargetOlsIdx. However, it seems out of scope of a video coding specification to specify such requirements for external means, and there are many other similar requirements for external means that would be similarly needed and that are not specified in HEVC, SHVC, and MV-HEVC anyway. One may argue that the flag can be used by legacy HEVC decoders to ignore bitstreams with this flag equal to 0. However, according to the HEVC spec, legacy HEVC decoders would just ignore the value of this flag, not the bitstream.

In bitstream conformance tests, each operation point corresponding to a layer set specified in the VPS in HEVC is tested, and each output operation point corresponding to an output layer set specified in the VPS in SHVC/MV-HEVC is tested. However, when not all sub-layers or all layers for an operation point (in HEVC) or an output operation point (in SHVC/MV-HEVC) are available, the bitstream conformance test should not be performed.

When a VPS or information therein is used for session negotiation, e.g. to agree on an (output) operation point, knowing which of the (output) operation points are actually fully available would be very useful. Such information would also be useful to a bitstream conformance checker to test only those (output) operation points that are fully available.

# Proposals

To address the issues discussed above, the following are proposed:

- Only those sub-bitstreams that correspond to fully available (output) operation points are required to be conforming bitstreams. An (output) operation point is said to be fully available if VCL NAL units are available (either present in the bitstream or provided by external means) for each layer included in the layer set and the highest value of TemporalId of all VCL NAL units in the bitstream is equal to or greater than the corresponding OpTid. One exception is for (output) operation points for which the layer set is the 0-th layer set when the base layer is provided by external means – in this case such (output) operation points are considered not fully available.

The following spec text for the definition of fully available (output) operation points is proposed:

When the OpLayerIdList of an operation point includes a value of nuh_layer_id greater than 0, the operation point is fully available if VCL NAL units are available (either present in the bitstream or provided by external means) for each layer included in the layer set and the highest value of TemporalId of all VCL NAL units in the bitstream is equal to or greater than the OpTid of the operation point, and not fully available otherwise.

When the OpLayerIdList of an operation point includes the value of nuh_layer_id equal to 0 only, the operation point is fully available if VCL NAL units are present in the bitstream for the layer with nuh_layer_id equal to 0 and the highest value of TemporalId of all VCL NAL units in the bitstream is equal to or greater than the OpTid of the operation point, and not fully available otherwise.

When an operation point is fully available, all output operation points associated with the operation point are also fully available.

Note that in the above definitions, "useless" layers included in an OLS that are neither output layers nor needed directly or indirectly for decoding the output layers are also required to be available for the corresponding (output) operation points to be fully available.

- Remove the vps_base_layer_available_flag, and add an SEI message to indicate which (output) operation points are fully available. From this SEI message, then whether the base layer is available can be concluded too. The vps_base_layer_available_flag is also included in the SEI message. The SEI message can be either provided in the bitstream or through external means, similarly as for parameter sets and the HRD-related SEI messages. It is suggested to discuss whether to use the information included in the SEI message in the specification of bitstream conformance tests to test only those (output) operation points that are fully available. The syntax and semantics of the SEI message are provided in Section 3.

# Fully available operation points SEI message

| fully_available_ops( payloadSize ) { | Descriptor |
| --- | --- |
| fao_sei_active_vps_id | u(4) |
| base_layer_available_flag | u(1) |
| for( i = base_layer_available_flag ? 0 : 1; i < NumLayerSets; i++ ) |  |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) |  |
| fao_sei_op_fully_available_flag[ i ][ j ] | u(1) |
| } |  |

fao_sei_active_vps_id identifies the active VPS of the CVS containing the fully available operation points SEI message. The value of fao_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

base_layer_available_flag equal to 0 indicates that the base layer is not available, i.e. neither present in the bitstream (when vps_base_layer_internal_flag is equal to 1) nor provided by external means (when vps_base_layer_internal_flag is equal to 0). base_layer_available_flag equal to 1 indicates that the base layer is available, either present in the bitstream (when vps_base_layer_internal_flag is equal to 1) or provided by external means (when vps_base_layer_internal_flag is equal to 0).

fao_sei_op_fully_available_flag[ i ][ j ] equal to 1 indicates that the operation point identified by OpLayerIdList containing all nuh_layer_id values of the i-th layer set and OpTid equal to i are fully available. fao_sei_op_fully_available_flag[ i ][ j ] equal to 0 indicates that the operation point identified by OpLayerIdList containing all nuh_layer_id values of the i-th layer set and OpTid equal to i are not fully available.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).