| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0051 |
| --- | --- |

| Title: | CE1: Simplification of Advanced Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

Proposed text is as follows. Deleted parts are marked as strikethrough and new added parts are highlighted in yellow (reduce worst case of number of additionally accessed blocks from 4 to 3) or green (uni-ARP for 4x4 chroma block).

- I.8.5.3.3.7	Bilinear sample interpolation and residual prediction process

- …

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

The location ( xP, yP )  is derived by:

  - xP = xCb + xBl		(Error: Reference source not found-1)

  - yP = yCb + yBl		(Error: Reference source not found-2)

The variable ivRefFlag is set equal to ( DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) = =  0 ), and the  variable availFlag is set equal to 0. The variable Z is set equal to 1 – X. When predFlagLZ is equal to 1, the variable ivRefZFlag is set equal to ( DiffPicOrderCnt( currPic, RefPicListZ[ refIdxLZ ] )  = =  0 ).

Depending on ivRefFlag and RpRefIdxLX, the following applies:

- If ivRefFlag is equal to 0 and RpRefIdxLX is not equal to −1, the variable availFlag is set equal to 1, the variable refIdxLX is set equal to RpRefIdxLX and the residual prediction motion vector scaling process as specified in subclause Error: Reference source not found is invoked with the prediction list utilization variable equal to X, the motion vector mvLX, and the RefPicListX[ refIdxLX ] and as inputs and modified mvLX as output.

- Otherwise, when ivRefFlag is equal to 1, the following applies:

  - If predFlagLZ is equal to 1 and ivRefZFlag is equal to 0, the following applies:

    - If RpRefIdxLZ is not equal to −1, the variable availFlag is set equal to 1, the residual prediction motion vector scaling process as specified in subclause Error: Reference source not found is invoked with the prediction list utilization variable equal to Z, the motion vector mvLZ, and the RefPicListZ[ refIdxLZ ] and as inputs and modified mvLZ as output.

    - When availFlag is equal to 1, motion vector mvT is set equal to mvLZ and prediction list utilization variable Y is set equal to Z.

  - Otherwise (predFlagLZ is equal to 0 or ivRefZFlag is equal to 1), the following applies:

    - Set variable W equal to ( predFlagLZ  &&  ivRefZFlag ) ? 0 : X

    - The derivation process for a motion vector from a reference block for residual prediction as specified in subclause Error: Reference source not found is invoked with ( xP, yP ), nPbW and nPbH, RefPicListXW[ refIdxLXW ], and mvLXW as inputs, and availFlag, motion vector mvT and prediction list utilization variable Y as outputs.

    - When availFlag is equal to 0 and RpRefIdxLXW is not equal to −1, availFlag is set equal to 1, mvT is set equal to (0, 0), Y is set equal to XW.

The motion vector mvCLX is set equal to mvLX.

The flag chromaArpFlag is set equal to 0 when X is equal to 1, both predFlagL0 and predFlagL1 are equal to 1 and both nPbW and nPbH are equal to 8 and 1 otherwise.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with currRefIdx as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause  with the luma locations ( xCb, yCb ), ( xBl, yBl ), , the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. and the flag chromaInterpFlag equal to 1 as inputs.

When availFlag is equal to 1 and iv_res_pred_weight_idx is not equal to 0, the following applies:

- Depending on ivRefFlag, the variables rpPic, rpRefPic, mvRp and curRefIdx are derived as specified in the following:

  - If ivRefFlag is equal to 0, the following applies:

    - Let rpPic be the picture with PicOrderCnt( rpPic ) equal to PicOrderCntVal and ViewIdx equal to RefViewIdx[ xP ][ yP ].

    - Let rpRefPic be the picture with PicOrderCnt( rpRefPic ) equal to RefPicListX[ RpRefIdxLX ] ) and ViewIdx equal to RefViewIdx[ xP ][ yP ],

    - The variable mvRp is set equal to MvDisp[ xP ][ yP ].

    - The variable curRefIdx is set equal to RpRefIdxLX.

  - Otherwise (ivRefFlag is equal to 1), the following applies:

    - Let rpPic be the picture RefPicListY[ RpRefIdxLY ].

    - Let rpRefPic be the picture with PicOrderCnt( rpRefPic ) equal to PicOrderCnt( rpPic ) and ViewIdx equal to RefViewIdx[ xP ][ yP ]

    - The variable mvRp is set equal to mvT.

    - The variable currRefIdx is set equal to RpRefIdxLY.

- The arrays rpSamplesLXL, rpSamplesLXCb, and rpSamplesLXCr  are derived as specified in the following:

  - Let the reference picture sample arrays rpPicLXL, rpPicLXCb, and rpPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture rpPic.

  - The arrays rpSamplesLXL, rpSamplesLXCb, and rpSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause  with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX equal to mvRp and mvCLX equal to mvRp, and the reference arrays with rpPicLXL, rpPicLXCb and rpPicLXCr and the flag chromaInterpFlag equal to chromaArpFlag as inputs.

- The arrays rpRefSamplesLXL, rpRefSamplesLXCb, and rpRefSamplesLXCr  are derived as specified in the following:

  - Let the reference picture sample arrays rpRefPicLXL, rpRefPicLXCb, and rpRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture rpRefPic.

  - The arrays rpRefSamplesLXL, rpRefSamplesLXCb, and rpRefSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause  with the luma locations ( xCb, yCb ), ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vector mvLX equal to ( mvLX + mvRp ) and the motion vector mvCLX equal to ( mvCLX + mvRp ), and the reference arrays with rpRefPicLXL, rpRefPicLXCb and rpRefPicLXCr and the flag chromaInterpFlag equal to chromaArpFlag as inputs.

- The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

- The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 predSamplesLXL[ x ][ y ] + 
	( ( rpSamplesLXL[ x ][ y ] − rpRefSamplesLXL[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-3)

- The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 predSamplesLXCb[ x ][ y ] + ( chromaArpFlag ?
	( ( rpSamplesLXCb[ x ][ y ] − rpRefSamplesLXCb[ x ][ y ] )  >>  shiftVal ) : 0 )	(Error: Reference source not found-4)

- The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 predSamplesLXCr[ x ][ y ] + ( chromaArpFlag ?
	( ( rpSamplesLXCr[ x ][ y ] − rpRefamplesLXCr[ x ][ y ] )  >>  shiftVal ) : 0 )	(Error: Reference source not found-5)

- I.8.5.3.3.7.1	Bilinear sample interpolation process

The specifications in subclause 8.5.3.3.3.1 apply with the following modifications:

- All invocations of the process specified in subclause 8.5.3.3.3.2 are replaced with invocations of the process specified in subclause Error: Reference source not found with chromaFlag equal to 0 as additional input.

- All invocations of the process specified in subclause 8.5.3.3.3.3 are replaced with invocations of the process specified in subclause Error: Reference source not found with chromaFlag equal to 1 as additional input.

[Note: not highlighted text is the editorial alternative of the deleted text in this sub-clause.]

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	a luma motion vector mvLX given in quarter-luma-sample units,

–	a chroma motion vector mvCLX given in eighth-chroma-sample units,

–	the selected reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCr.

–	a flag chromaInterpFlag specifying whether performing sample interpolation for chroma component,

Outputs of this process are:

–	an (nPbW)x(nPbH) array predSampleLXL of prediction luma sample values,

–	two (nPbW / 2)x(nPbH / 2) arrays predSampleLXCb, and predSampleLXCr of prediction chroma sample values.

The location ( xPb, yPb ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the given reference sample arrays is derived as follows:

xPb = xCb + xBl		(8-6)

yPb = yCb + yBl		(8-7)

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb, and refPicLXCr.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSampleLXL, the corresponding prediction luma sample value predSampleLXL[ xL, yL ] is derived as follows:

–	The variables xIntL, yIntL, xFracL, and yFracL are derived as follows:

xIntL = xPb + ( mvLX[ 0 ]  >>  2 ) + xL		(8-8)

yIntL = yPb + ( mvLX[ 1 ]  >>  2 ) + yL		(8-9)

xFracL = mvLX[ 0 ] & 3		(8-10)

yFracL = mvLX[ 1 ] & 3		(8-11)

–	The prediction luma sample value predSampleLXL[ xL, yL ] is derived by invoking the process specified in subclause I.8.5.3.3.7.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), and refPicLXL as inputs.

When chromaInterpFlag is equal to 0, the whole decoding process of this subclause terminates.

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXCb and refPicLXCr.

For each chroma sample location ( xC = 0..nPbW / 2 − 1, yC = 0..nPbH / 2 − 1 ) inside the prediction chroma sample arrays predSampleLXCb and predSampleLXCr, the corresponding prediction chroma sample values predSampleLXCb[ xC, yC ] and predSampleLXCr[ xC, yC ] are derived as follows:

–	The variables xIntC, yIntC, xFracC, and yFracC are derived as follows:

xIntC = ( xPb / 2 ) + ( mvCLX[ 0 ]  >>  3 ) + xC		(8-12)

yIntC = ( yPb / 2 ) + ( mvCLX[ 1 ]  >>  3 ) + yC		(8-13)

xFracC = mvLX[ 0 ] & 7		(8-14)

yFracC = mvLX[ 1 ] & 7		(8-15)

–	The prediction sample value predSampleLXCb[ xC, yC ] is derived by invoking the process specified in subclause I.8.5.3.3.7.2 with ( xIntC, yIntC ), ( xFracC, yFracC ), and refPicLXCb as inputs.

–	The prediction sample value predSampleLXCr[ xC, yC ] is derived by invoking the process specified in subclause I.8.5.3.3.7.2 with ( xIntC, yIntC ), ( xFracC, yFracC ), and refPicLXCr as inputs.