| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0052 |
| --- | --- |

| Title: | CE1 related: Alignment of disparity vector in inter-view motion and residual predictions for ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, two different disparity vectors are used to identify reference block (in reference view) in inter-view motion prediction and advanced residual prediction for a current block. Therefore, motion information and pixel samples in the two processes are misaligned because they are from two different reference blocks. To remove the misalignment, this contribution proposes to identify one single reference block when both advanced residual prediction applies for ARP and 3D-HEVC merge candidates rely on disparity vector for the current PU, with one single disparity vector. It is reported that there is no coding loss (0.1% coding gain for dependent views) compared to HTM under the common test conditions. The proposed method avoids DoNBDV and therefore depth access when ARP is enabled.

# Introduction

In 3D-HEVC, inter-view motion prediction and residual prediction [1] are enabled for non-base texture views.

Inter-view motion prediction is illustrated in Figure 1. One reference block of the current PU (Predict Unit) is identified by a disparity vector (denoted by DV), and then motion vectors of sub-blocks within the reference block are copied to corresponding sub-blocks of the current PU. The disparity vector DV is derived in DoNBDV (Depth oriented Neighboring Block based Disparity Vector) process.

Figure 1. Inter-view motion prediction

The prediction structure for temporal residual prediction is illustrated in Figure 2. One reference block (denoted by Base) in reference view is identified by a disparity vector (denoted by derivedDV) and accessed, and the temporal reference block of Base (denoted by BaseTRef) is also accessed. Difference between Base and BaseTRef is used to predict the temporal residual of current block, i.e., the difference between current block (denoted by Curr) and its temporal reference block (denoted by CurrTRef). The disparity vector derivedDV is derived in NBDV (Neighboring Block based Disparity vector) process.

Figure 2. Temporal residual prediction

Because two different disparity vectors are used in inter-view motion prediction and temporal residual prediction, two different reference blocks are identified for current block. This may cause following problems:

- Two different corresponding objects are assumed in the two processes thus may lead to coding performance loss.

- Accessing motion information and pixel samples from different reference blocks may lead to misaligned memory access.

- Unnecessary memory access (without coding performance benefit) for DoNBDV in case of ARP.

- Proposal

To address above problems, when both inter-view motion prediction and temporal residual prediction apply for current PU, one single disparity vector is used to identify one single reference block for both processes. Specifically, when ARP (Advanced Residual Prediction) is enabled for current PU, disparity vector generated in NBDV process is used for the following processes:

- Inter-view motion prediction.

- Shifted inter-view motion prediction.

- Disparity vector merging candidate.

- Shifted disparity vector merging candidate.

- ARP

In this way, DoNBDV and therefore depth access can be avoided when ARP is enabled.

The worst case of ARP in terms of memory bandwidth requirement is analyzed based on the template provided by AHG 10 [2]. Note that it is considered that the worst case is when a CU is 8x8 and coded with ARP. Meanwhile, bandwidth required by DoNBDV is also considered in ARP. In DoNBDV, four corner pixels of the corresponding depth block (of current block) in reference depth view are needed to derive disparity vector. Two methods are considered for obtaining these four corner pixels in the bandwidth analysis:

- Fetch four corner pixels of the depth block.

- Fetch the entire depth block.

Table 1 provides the bandwidth and the coding performance of ARP, proposed method and combination of this proposal with JCT3V-I0051 [3].

Table 1. Analysis of the worst case of ARP (Anchor is HEVC single view coding)

| Coding Tools | Complexity (for different memory pattern) |  |  |  | Coding performance |
| --- | --- | --- | --- | --- | --- |
|  | 1x1 | 4x2 | 4x4 | 8x2 |  |
| ARP (Fetching four corner pixels of the depth block) | 122% | 138% | 133% | 164% | CTC |
| ARP (Fetching the whole depth block) | 132% | 148% | 139% | 172% |  |
| Proposed | 122% | 135% | 128% | 158% | -0.03% |
| Proposed + JCT3V-I0051 | 94% | 103% | 96% | 119% | 0.06% |
| Proposed + JCT3V-I0051 method 1 | 101% | 113% | 106% | 132% | -0.01% |

As shown in the table, proposed method can reduce the memory bandwidth of ARP by up to 14% without any coding performance loss. When combined this proposal with JCT3V-I0051, up to 53% memory bandwidth can be reduced for ARP.

# Experimental results

The proposed method is integrated into current 3D-HTM 11.0 software [4] and simulations are performed based on the common test conditions (CTC) [5].

The coding performance in CTC (anchor is HTM 11.0) is provided in Table 1. As reported in Table 1, there is no coding performance loss or even -0.1% bitrate reduction in dependent views.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 1: Coding results for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.19% | -0.04% | -0.04% | -0.05% |  |  | -0.06% | 92.9% | 94.5% | 94.3% |
| Kendo | 0.00% | 0.07% | 0.01% | -0.04% |  |  | 0.01% | 93.3% | 94.5% | 95.8% |
| Newspapercc | -0.12% | -0.08% | -0.04% | -0.01% |  |  | -0.04% | 98.8% | 99.2% | 98.9% |
| GhostTownFly | -0.10% | -0.17% | -0.02% | -0.02% |  |  | -0.06% | 90.0% | 92.4% | 92.2% |
| PoznanHall2 | -0.21% | -0.32% | -0.10% | -0.08% |  |  | -0.06% | 102.6% | 98.6% | 104.6% |
| PoznanStreet | 0.14% | -0.02% | 0.02% | 0.02% |  |  | 0.03% | 106.2% | 105.4% | 108.3% |
| UndoDancer | 0.02% | -0.02% | 0.01% | 0.01% |  |  | -0.01% | 98.3% | 96.8% | 99.3% |
| Shark | -0.15% | -0.17% | -0.03% | -0.04% |  |  | -0.03% | 97.4% | 96.5% | 97.2% |
| 1024x768 | -0.10% | -0.02% | -0.02% | -0.03% |  |  | -0.03% | 95.0% | 96.1% | 96.3% |
| 1920x1088 | -0.06% | -0.14% | -0.03% | -0.02% |  |  | -0.03% | 98.9% | 97.9% | 100.3% |
| average | -0.08% | -0.09% | -0.02% |  |  | -0.03% | -0.03% | 97.4% | 97.2% | 98.8% |

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1001, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- T. Ikai, “AHG10: Complexity assessment of motion compensation process,” JCT3C-G0208, 7th Meeting: San José, US, 11–17 Jan. 2014.

- H. Liu and Y. Chen, “CE1: Simplification of advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-I0051, 9th Meeting: Sapporo, JP, 3–9 July 2014.

- 3DV-HTM version 11.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).