| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0052 |
| --- | --- |

| Title: | CE1 related: Alignment of disparity vector in inter-view motion and residual predictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

The proposed text is as follows. Deleted parts are marked as strikethrough and new added parts are highlighted in yellow.

- I.8.5.5	Derivation process for disparity vectors

…

When dvAvailFlag is equal to 0, refViewIdx is set equal to DefaultRefViewIdx, and mvDisp is set equal to ( 0, 0 ).

Depending on depth_refinement_flag[ nuh_layer_id ] and iv_res_pred_weight_idx[ xCb ][ yCb ], the following applies:

- If depth_refinement_flag[ nuh_layer_id ] is equal to 1 and iv_res_pred_weight_idx[ xCb ][ yCb ] is equal to 0, the following applies:

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, and the variable partIdc equal to 0 as inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The disparity vector mvRefinedDisp is set equal to ( disparitySamples[ 0 ][ 0 ], 0 ).

- Otherwise (depth_refinement_flag[ nuh_layer_id ] is equal to 0 or iv_res_pred_weight_idx[ xCb ][ yCb ] is not equal to 0), the disparity vector mvRefinedDisp is set equal to mvDisp.

…