| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0053 |
| --- | --- |

| Title: | Simplifications for 3D-HEVC merge candidate list generation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

As discussed in previous JCT-3V meetings, in derivation process of 3D-HEVC merge list, there are several aspects that may not be optimal. First, the insertion of HEVC merge candidates into 3D-HEVC merge candidate list depends on spatial position and availability of spatial merge candidates. Second, the VSP (View Synthesis Prediction) candidate may be generated in a way that requiring a status to be stored (a VSP variable/flag) for spatial neighboring so that the merging process takes this variable into consideration. Third, when deriving shifted disparity merging candidate, up to 5 spatial merge candidates may be checked to find the first available DMV (Disparity Motion Vector).

This contribution provides solutions that can be considered by 3D-HEVC. First, when inserting the additional merge candidates, the dependency on spatial positions of the HEVC merge candidates is removed. Second, the VSP flag is not used to generate VSP candidates therefore storage and check of spatial neighboring blocks are not needed. Third, only allow check up to 2 HEVC merge candidates when searching the first available DMV for the first two HEVC merge candidates. It is reported that proposed method has negligible coding loss compared to HTM under the common test conditions and the condition when depth dependent texture coding is not enabled.

# Introduction

## Construction of 3D-HEVC merge list based on the HEVC base merge list

In 3D-HEVC, the final merge candidate list includes entries from the HEVC base merge list and the additional candidates derived through so-called inter-view prediction [1]. More specifically, the major processes are described as follows:

- Spatial and temporal merge candidates are inserted into the base merge candidate list, similar as in HEVC version 1.

- Virtual candidates (a.k.a., combined bi-predictive merging candidates) and zero candidates are inserted into the base merge candidate list when necessary, so that the total number of candidates is equal to MaxNumMergeCand, which is equal to the number of single-view HEVC candidates as signaled in slice header plus one (if inter-view prediction or MPI is enabled). After this, the HEVC base candidate list is formed.

- The 3D-HEVC merge list is generated by inserting the additional 3D-HEVC candidates as well as candidates in the HEVC base candidate into one list.

- The additional 3D-HEVC candidates include

  - Inter-view predicted motion candidate, denoted as IvMC

  - Inter-view disparity vector candidate, denoted as IvDC

  - View synthesis prediction candidate, denoted as VSP

  - Shifting candidate, denoted as IvShift, being equal to either shifted inter-view predicted motion candidate, denoted as IvMCShift, or shifted disparity merging candidate, denoted by IvDCShift.

## A1 or B1 interaction between 3D-HEVC merge list and the HEVC base merge list

Several additional merge candidates may need to be pruned with two candidates, in HEVC base merge list, A1 or B1 if available. The candidates are: IvMC and IvDC.

Since the pruning relies on specific spatial positions of the candidates of the HEVC base merge candidate list, it is always required even after the HEVC base merge candidate is formed, to decide from which spatial position the current candidate is coming from. More specifically, the availability information of all spatial candidates also needs to be kept.

Such an interaction is not preferable, especially from the hardware design simplicity perspective.

Although the computational complexity of the merge candidate list construction is not high especially in this aspect, it probably requires unnecessary complication due to the abovementioned interaction.

## VSP candidate

The VSP candidate is generated by utilizing NBDV derived disparity vector and if a merge index refers to a VSP candidate, depth information can be accessed to assign disparity motion vectors for the current PU.

However, currently, a spatial merging candidate is converted to a VSP candidate when its VSP flag is true. To support this conversion, VSP flag of neighboring blocks has to be stored and considered in the merge list construction process.

Removing of such a flag has been proposed multiple times, but with some coding loss.

## Shifting candidate

A shifting candidate (IvShift) can be inserted into the merge candidate list, it is either a shifted inter-view predicted motion candidate (IvMCShift), of if IvMC is unavailable, a shifted inter-view disparity vector candidate, denoted as IvDCShift. It is worthwhile to mention that although two candidates IvMCShift and IvDCShift are involved here, only one final candidate is possibly inserted into the 3D-HEVC merge list.

Derivation process of IvDCShift is as follows [2]:

- Identify the first available disparity motion vector corresponding to the RefPicList0 from all existing candidates in the current 3D-HEVC merge list, which is the one after IvMC and IvDC are inserted (but these two are excluded). .

- If the DMV is available, its horizontal component is shifted by four to be used to form a IvDCShift candidate.

- Otherwise, the motion vector of IvDCShift is set to be DV shifted by four.

In the worst case scenario, motion vector of all candidates in the HEVC base merge list may be checked to identify the first available DMV, i.e., up to 5 motion vectors may be checked, which is considered as relatively complex.

## Evaluation of the merge candidates

The merge candidates are designed so that they can improve the coding efficiency with much smaller complexity increase (e.g., compared to per-pixel operations). The coding performance of the merge candidates may be evaluated under common test condition as well as the cases when there is no access of depth during texture coding (multiview compatible mode), i.e., when DoNBDV, BVSP and DBBP are turned off.

- Proposal

To address the above problems, this contribution proposes to:

- Remove dependency on spatial position when inserting HEVC candidates.

When pruning is needed between an additional merge candidate and the HEVC base merge candidate, instead of comparing with A1 or B1, comparison with the 0-th or 1-st candidate applies. .

- Remove VSP flag storage and check of spatial neighboring blocks.

Only insert one VSP candidate into 3D-HEVC merge list at a fixed position, and disallow converting HEVC spatial merge candidates to VSP candidates. Meanwhile, the VSP candidate is put prior to (instead of posterior to) IvDC candidate.

- Check at most 2 HEVC merge candidates when searching the first available DMV for the shifted disparity merging candidate, denoted by IvDCShift.

Proposed insertion order of candidates and pruning process is shown in Table 1. Here, bi, i = 0 to MaxNumMergeCand – 1, refers to the ith entry of candidate list based on the 2D merge list.

Table 1. Proposed insertion order and pruning process of merge candidates in 3D-HEVC

| Insertion order | Pruning with |
| --- | --- |
| IvMC |  |
| b0 | IvMC |
| b1 | IvMC |
| VSP |  |
| IvDC | b0, b1 |
| b2 |  |
| IvMCShift | IvMC |
| IvDCShift |  |
| b3, b4, b5 |  |

With proposed simplification, computation complexity of shifted inter-view merge candidate is reduced by more than 75%, as illustrated in Table 2.

Table 2. Complexity analysis of shifted disparity merging candidate

| Method | Addition | Comparison |
| --- | --- | --- |
| 3D-HEVC | 6 | 42 |
| Proposed | 3 | 10 |

# Experimental results

The proposed method is integrated into current 3D-HTM 11.0 software [3] and simulations are performed based on the common test conditions (CTC) [4]. Meanwhile, proposed method is also evaluated in multiview compatible mode, i.e., when DoNBDV, BVSP and DBBP are turned off (and the anchor is 3D-HTM 11.0 with DoNBDV, BVSP and DBBP off).

Following cases are tested:

- Case #1: Proposal #1 and proposal #2 (simplification of the merge candidate list construction and VSP candidate).

- Case #2: Simplification of IvDCShift.

  - Check at most 2 HEVC merge candidates when searching the first available DMV.

- Case #3: Case #1 + case #2.

The coding performance of the above test cases under CTC (anchor is HTM 11.0) as well as multiview compatible mode are provided in Table 3 - 8. As can be seen, there is negligible coding performance loss in all three cases.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 3. Coding results of case #1 for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.03% | 0.01% | -0.01% |  |  | 0.01% | 104.0% | 103.6% | 103.1% |
| Kendo | -0.10% | 0.16% | 0.00% | -0.04% |  |  | 0.00% | 98.3% | 99.5% | 99.6% |
| Newspapercc | -0.07% | -0.04% | -0.02% | 0.03% |  |  | 0.05% | 95.0% | 95.3% | 93.4% |
| GhostTownFly | 0.14% | 0.16% | 0.03% | 0.01% |  |  | 0.03% | 85.8% | 97.0% | 89.5% |
| PoznanHall2 | 0.17% | -0.21% | -0.03% | -0.02% |  |  | -0.01% | 105.1% | 102.0% | 105.5% |
| PoznanStreet | 0.25% | -0.06% | 0.02% | 0.02% |  |  | 0.03% | 93.5% | 93.5% | 93.1% |
| UndoDancer | 0.23% | 0.25% | 0.06% | 0.06% |  |  | 0.07% | 92.9% | 90.4% | 92.6% |
| Shark | 0.03% | -0.09% | -0.03% | -0.03% |  |  | 0.01% | 102.8% | 103.3% | 103.6% |
| 1024x768 | -0.06% | 0.05% | 0.00% | 0.00% |  |  | 0.02% | 99.1% | 99.5% | 98.7% |
| 1920x1088 | 0.16% | 0.01% | 0.01% | 0.01% |  |  | 0.03% | 96.0% | 97.2% | 96.9% |
| average | 0.08% | 0.03% | 0.00% |  |  | 0.00% | 0.02% | 97.2% | 98.1% | 97.6% |

Table 4. Coding results of case #1 for 3-view case (Disable BVSP, DoNBDV, DBBP)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.16% | -0.13% | 0.02% | 0.03% |  |  | 0.02% | 99.7% | 102.1% | 99.7% |
| Kendo | 0.07% | -0.12% | -0.01% | -0.01% |  |  | 0.03% | 111.5% | 108.9% | 110.9% |
| Newspapercc | 0.20% | -0.09% | 0.03% | 0.05% |  |  | 0.03% | 99.4% | 100.7% | 99.5% |
| GhostTownFly | 0.05% | 0.07% | 0.01% | 0.01% |  |  | -0.05% | 103.8% | 105.5% | 104.6% |
| PoznanHall2 | 0.28% | 0.22% | 0.11% | 0.10% |  |  | 0.05% | 99.0% | 102.1% | 98.9% |
| PoznanStreet | 0.02% | -0.04% | 0.02% | 0.00% |  |  | -0.01% | 99.2% | 100.8% | 99.7% |
| UndoDancer | -0.01% | -0.11% | -0.02% | -0.01% |  |  | 0.01% | 100.3% | 100.8% | 103.3% |
| Shark | -0.04% | 0.07% | 0.00% | 0.01% |  |  | 0.01% | 99.5% | 101.0% | 99.8% |
| 1024x768 | 0.14% | -0.11% | 0.01% | 0.02% |  |  | 0.02% | 103.5% | 103.9% | 103.4% |
| 1920x1088 | 0.06% | 0.04% | 0.02% | 0.02% |  |  | 0.01% | 100.4% | 102.0% | 101.3% |
| average | 0.09% | -0.02% | 0.02% |  |  | 0.02% | 0.01% | 101.5% | 102.7% | 102.1% |

Table 5. Coding results of case #2 for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.06% | 0.14% | 0.02% | 0.02% |  |  | -0.01% | 108.7% | 107.8% | 108.8% |
| Kendo | -0.02% | -0.06% | -0.03% | -0.06% |  |  | 0.04% | 96.1% | 99.5% | 96.2% |
| Newspapercc | 0.01% | 0.13% | 0.03% | 0.05% |  |  | 0.02% | 97.8% | 98.2% | 97.1% |
| GhostTownFly | -0.05% | -0.01% | 0.01% | 0.01% |  |  | -0.01% | 96.4% | 97.6% | 98.1% |
| PoznanHall2 | 0.01% | -0.19% | -0.05% | -0.05% |  |  | -0.04% | 110.3% | 103.1% | 109.3% |
| PoznanStreet | 0.07% | -0.01% | 0.00% | 0.00% |  |  | 0.03% | 104.5% | 102.4% | 103.6% |
| UndoDancer | 0.07% | 0.04% | 0.01% | 0.02% |  |  | 0.05% | 96.5% | 90.8% | 96.3% |
| Shark | -0.04% | -0.01% | -0.01% | 0.00% |  |  | 0.01% | 96.6% | 102.0% | 96.8% |
| 1024x768 | -0.03% | 0.07% | 0.00% | 0.00% |  |  | 0.01% | 100.8% | 101.8% | 100.7% |
| 1920x1088 | 0.01% | -0.04% | -0.01% | 0.00% |  |  | 0.01% | 100.9% | 99.2% | 100.8% |
| average | 0.00% | 0.01% | 0.00% |  |  | 0.00% | 0.01% | 100.9% | 100.2% | 100.8% |

Table 6. Coding results of case #2 for 3-view case (Disable BVSP, DoNBDV, DBBP)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.06% | -0.06% | 0.01% | 0.01% |  |  | 0.04% | 92.4% | 94.6% | 93.4% |
| Kendo | 0.18% | -0.09% | 0.00% | 0.00% |  |  | 0.04% | 98.6% | 100.2% | 97.7% |
| Newspapercc | 0.13% | -0.06% | 0.01% | 0.02% |  |  | 0.02% | 100.6% | 101.0% | 101.2% |
| GhostTownFly | 0.13% | 0.01% | 0.01% | 0.01% |  |  | 0.02% | 97.1% | 98.8% | 97.9% |
| PoznanHall2 | -0.02% | -0.04% | -0.02% | -0.02% |  |  | 0.02% | 92.3% | 96.9% | 93.1% |
| PoznanStreet | 0.01% | 0.09% | 0.00% | 0.00% |  |  | -0.02% | 95.7% | 93.7% | 96.3% |
| UndoDancer | 0.02% | -0.07% | -0.01% | 0.00% |  |  | 0.08% | 96.6% | 95.4% | 97.8% |
| Shark | 0.06% | -0.14% | -0.01% | -0.01% |  |  | 0.00% | 111.6% | 109.0% | 108.8% |
| 1024x768 | 0.12% | -0.07% | 0.01% | 0.01% |  |  | 0.03% | 97.2% | 98.6% | 97.4% |
| 1920x1088 | 0.04% | -0.03% | 0.00% | -0.01% |  |  | 0.02% | 98.7% | 98.7% | 98.8% |
| average | 0.07% | -0.05% | 0.00% |  |  | 0.00% | 0.02% | 98.1% | 98.7% | 98.3% |

Table 7. Coding results of case #3 for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.07% | 0.06% | 0.00% | 0.00% |  |  | 0.01% | 90.3% | 91.4% | 90.8% |
| Kendo | -0.20% | 0.23% | 0.00% | -0.03% |  |  | 0.02% | 100.4% | 99.8% | 100.2% |
| Newspapercc | -0.11% | -0.05% | -0.03% | 0.02% |  |  | 0.02% | 89.8% | 90.9% | 90.0% |
| GhostTownFly | 0.19% | 0.15% | 0.03% | 0.01% |  |  | 0.01% | 92.9% | 92.5% | 93.4% |
| PoznanHall2 | -0.03% | -0.19% | -0.05% | -0.02% |  |  | -0.01% | 108.9% | 103.9% | 110.7% |
| PoznanStreet | 0.29% | 0.01% | 0.04% | 0.04% |  |  | 0.07% | 101.0% | 100.9% | 101.5% |
| UndoDancer | 0.27% | 0.25% | 0.06% | 0.06% |  |  | 0.08% | 96.8% | 93.3% | 98.0% |
| Shark | 0.04% | -0.05% | -0.03% | -0.03% |  |  | 0.01% | 88.8% | 92.2% | 88.6% |
| 1024x768 | -0.12% | 0.08% | -0.01% | 0.00% |  |  | 0.02% | 93.5% | 94.1% | 93.7% |
| 1920x1088 | 0.15% | 0.03% | 0.01% | 0.01% |  |  | 0.03% | 97.7% | 96.6% | 98.4% |
| average | 0.05% | 0.05% | 0.00% |  |  | 0.01% | 0.03% | 96.1% | 95.6% | 96.6% |

Table 8. Coding results of case #3 for 3-view case (Disable BVSP, DoNBDV, DBBP)

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.12% | -0.04% | 0.04% | 0.04% |  |  | 0.01% | 91.8% | 92.7% | 93.0% |
| Kendo | 0.06% | -0.17% | -0.01% | 0.00% |  |  | -0.02% | 102.7% | 101.0% | 100.5% |
| Newspapercc | 0.12% | -0.07% | 0.01% | 0.03% |  |  | 0.01% | 92.5% | 95.9% | 92.8% |
| GhostTownFly | 0.04% | 0.08% | 0.01% | 0.00% |  |  | -0.04% | 95.8% | 97.5% | 96.2% |
| PoznanHall2 | 0.24% | 0.17% | 0.09% | 0.07% |  |  | 0.10% | 107.7% | 108.7% | 105.1% |
| PoznanStreet | 0.07% | 0.02% | 0.03% | 0.00% |  |  | -0.01% | 100.2% | 103.4% | 101.3% |
| UndoDancer | 0.01% | -0.05% | 0.00% | 0.01% |  |  | 0.06% | 99.7% | 96.2% | 100.2% |
| Shark | 0.07% | 0.00% | 0.00% | 0.01% |  |  | 0.02% | 106.7% | 102.5% | 105.9% |
| 1024x768 | 0.10% | -0.09% | 0.01% | 0.02% |  |  | 0.00% | 95.7% | 96.5% | 95.4% |
| 1920x1088 | 0.08% | 0.04% | 0.02% | 0.02% |  |  | 0.03% | 102.0% | 101.7% | 101.7% |
| average | 0.09% | -0.01% | 0.02% |  |  | 0.02% | 0.02% | 99.6% | 99.7% | 99.4% |

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1001, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- Y. Chen, G. Tech, K. Wegner and S. Yea, “Test Model 8 of 3D-HEVC and MV-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1003, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- 3DV-HTM version 11.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).