| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0053 |
| --- | --- |

| Title: | Simplifications for 3D-HEVC merge candidate list generation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

Proposed text is as follows. Deleted parts are marked as strikethrough and new added parts are highlighted in yellow. Meanwhile, some bug fix and editorial improvements of 3D-HEVC text is highlighted in purple.

- I.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when MergeFlag[ xPb ][ yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the flag ivpMvFlag, specifying, whether the current PU is coded using inter-view motion prediction,

- the flag vspModeFlag, specifying, whether the current PU is coded using view synthesis prediction,

- the flag subPbMotionFlag, specifying, whether the motion data of the current PU has sub prediction block size motion accuracy,

- the flag dispDerivedDepthFlag, specifying, whether the current PU uses disparity derived depth,

- the variable dispDerivedDepthVal (when dispDerivedDepthFlag is equal to 1).

[Ed. (GT): In particular two things need to be check in this process: 1.) Are the limits on candidates in the list correct ( e.g. MaxNumMergeCand vs.  5 + NumExtraMergeCand  ) 2.) Is ( xOrigP, yOrigP ) and ( xPb, yPb ) used correctly in all places? ]

The function differentMotion( N, M ) is specified as follows:

- If one of the following conditions is true, differentMotion( N, M ) is equal to 1:

  - predFlagLXN  !=  predFlagLXM (with X being replaced by 0 and 1),

  - mvLXN  !=  mvLXM (with X being replaced by 0 and 1),

  - refIdxLXN  !=  refIdxLXM (with X being replaced by 0 and 1),

- Otherwise, differentMotion( N, M ) is equal to 0.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for the base merge candidate list as specified in subclause  is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - For N being replaced by 0 to MaxNumMergeCand - 1, inclusive A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagbN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagbN is set equal to 0.

  - Depending on iv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively).

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], DispAvailabilityIdc[ xPb ][ yPb ], and , dbbp_flag[ xPb ][ yPb ] the following applies:

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, or dbbp_flag[ xPb ][ yPb ] is equal to 1, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, and dbbp_flag[ xPb ][ yPb ] is equal to 0), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xPb, yPb ) and the variables nPbW and nPbH as inputs, and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on mpi_flag[ nuh_layer_id ], the following applies:

    - If mpi_flag[ nuh_layer_id ] is equal to 0, the variables availableFlagT and availableFlagD are set equal to 0.

    - Otherwise (mpi_flag[ nuh_layer_id ] is equal to 1), the following applies:

      - The derivation process for inter layer predicted sub prediction block motion vector candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the variable refViewIdx being equal to −1, and the variable mvDisp being equal to ( 0, 0 ) as inputs, and the outputs are the prediction utilization flag predFlagLXT, the motion vector mvLXT and the reference indices refIdxLXT (with X being 0 or 1, respectively).

      - The flag availableFlagT is set equal to ( predFlagL0T  | |  predFlagL1T ).

      - The derivation process for the disparity derived merging candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the outputs are the flag availableFlagD, the prediction utilization flag predFlagLXD, the reference index refIdxLXD, the motion vector mvLXD (with X being 0 or 1, respectively), and the variable dispDerivedDepthVal.

  - The variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, DepthFlag is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in baseMergeCandList extMergeCandList, baseMergeCandList extMergeCandList, and i as inputs, and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

  - The merging candidate list, extMergeCandList, is constructed as follows:

baseCanRemoval[ 0 ] = 1
			baseCanRemoval[ 1 ] = 1
			i = 0
			if( availableFlagT )
				extMergeCandList[ i++ ] = T
			if( availableFlagD )
				extMergeCandList[ i++ ] = D
			if( availableFlagIvMC  &&  ( !availableFlagT  | |  differentMotion( T, IvMC ) ) )
				extMergeCandList[ i++ ] = IvMC
			N = DepthFlag ? T : IvMC
			if( availableFlagA1availableFlagb0  &&  ( !availableFlagN  | |  differentMotion( N, b0A1 ) ) )
				extMergeCandList[ i++ ] = b0A1
			else
				baseCanRemoval[ 0 ] = 0
			if( ( availableFlagb0  &&  availableFlagN  &&  !differentMotion( N, b0 ) )  | | 
			availableFlagB1( availableFlagb1  &&  ( !availableFlagN  | |  differentMotion( N, b1B1 ) ) ) )
				extMergeCandList[ i++ ] = b1B1
			else
				baseCanRemoval[ 1 ] = 0
			if( availableFlagB0 )
				extMergeCandList[ i++ ] = B0	(Error: Reference source not found-1)
			if( availableFlagIvDC  &&  ( !availableFlagA1  | |  differentMotion( A1, IvDC ) )  &&  
				( !availableFlagB1  | |  differentMotion( B1, IvDC ) )  &&  ( i < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC
			if( availableFlagVSP  &&  !ic_flag  &&  iv_res_pred_weight_idx  = =  0  &&  
									i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = VSP
			if( availableFlagIvDC  &&  ( ! baseCanRemoval[ 0 ]  | |  differentMotion( b0, IvDC ) )  &&  
				( ! baseCanRemoval[ 1 ]  | |  differentMotion( b1, IvDC ) ) )
				extMergeCandList[ i++ ] = IvDC
			if( availableFlagA0availableFlagb2  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = b2A0
			if( availableFlagB2  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = B2
			if( availableFlagIvMCShift )
				IvShift = IvMCShift
			else if ( availableFlagIvDCShift )
				IvShift = IvDCShift
			if( ( availableFlagIvMCShift  &&  i < ( 5 + NumExtraMergeCand )  &&
					( !availableFlagIvMC  | |  differentMotion( IvMC, IvMCShift ) ) )  | |  
					availableFlagIvDCShift )
				extMergeCandList[ i++ ] = IvMCShift
			for( N = 3; i < MaxNumMergeCand ; N++ )
				extMergeCandList[ i++ ] = bN

  - The variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, DepthFlag is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in extMergeCandList, extMergeCandList, and i as inputs, and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

  - The merging candidate list, extMergeCandList, is constructed as follows:

if( availableFlagIvDCShift )
				extMergeCandList[ i++ ] = IvDCShift
			j = 0
			while( i < MaxNumMergeCand ) {	(Error: Reference source not found-2)
				N = baseMergeCandList[ j++ ]
				if( N  !=  A1  &&  N  !=  B1  &&  N  !=  B0  &&  N  !=  A0  &&  N  !=  B2 )
					extMergeCandList[ i++ ] = N
			}

  - The variable N is derived as specified in the following:

    - If ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies:

        - N = baseMergeCandList[ MergeIdx[ xOrigP ][ yOrigP ] ]	(Error: Reference source not found-3)

    - Otherwise, ( ( nOrigPbW + nOrigPbH ) is not equal to 12 ), the following applies:

        - N = extMergeCandList[ MergeIdx[ xOrigP ][ yOrigP ] ]	(Error: Reference source not found-4)

  - The derivation process for a view synthesis prediction flag as specified in subclause  is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nPbW and nPbH, the merge candidate indicator N as the inputs, and the output is the mergeCandIsVspFlag.

  - The variable vspModeFlag is derived as specified in the following:

      - vspModeFlag = mergeCandIsVspFlag( N  = =  VSP )  &&  !ic_flag  &&  
	( iv_res_pred_weight_idx  = = 0 )  &&  availableFlagVSP	(Error: Reference source not found-5)

  - The variable subPbMotionFlag is derived as specified in the following:

      - subPbMotionFlag = ( ( ( N  = =  IvMC )  &&  ( PartMode  = =  PART_2Nx2N ) )
		  | |  vspModeFlag )  && !dbbp_flag	(Error: Reference source not found-6)

      - [Note (FJ): This needs to be aligned in the software, as VSP is now also implemented as a special case of subPbMotionPrediction and DBBP does not support subPbMotionPrediction.]

  - The following assignments are made with X being replaced by 0 or 1:

      - mvLX = subPbMotionFlag ? 0 : mvLXN		(Error: Reference source not found-7)

      - refIdxLX = subPbMotionFlag ? −1 : refIdxLXN	(Error: Reference source not found-8)

      - predFlagLX = subPbMotionFlag ? 0 : predFlagLXN	(Error: Reference source not found-9)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies:

      - refIdxL1 = −1		(Error: Reference source not found-10)

      - predFlagL1 = 0		(Error: Reference source not found-11)

  - The disparity availability flag ivpMvFlag is derived as follows:

      - ivpMvFlag = !DepthFlag  &&  ( ( N  =  =  IvMC )  | |  ( N  = =  IvMCShift ) )	(Error: Reference source not found-12)

  - The variable dispDerivedDepthFlag is derived as follows:

      - dispDerivedDepthFlag = ( N  = =  D )		(Error: Reference source not found-13)

- I.8.5.3.2.15	Derivation process for the shifted disparity merging candidate

This process is not invoked when DepthFlag is equal to 1.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

- the availability flags availableFlagN,

- the reference indices refIdxL0N and refIdxL1N,

- the prediction list utilization flags predFlagL0N and predFlagL1N,

- the motion vectors mvL0N and mvL1N,

- a merging candidate list mergeCandList,

- the variable numMergeCand specifying the number of merge candidates in list mergeCandList.

Outputs of this process are:

- the flag availableFlagIvDCShift, specifying whether shifted disparity merging candidate is available,

- the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift,

- the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift,

- the motion vectors mvL0IvDCShift and mvL1IvDCShift.

- The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to min( 2, MaxNumMerge - 1 )CandnumMergeCand - 1, inclusive, the following applies:

  - The variable N is set equal to mergeCandList[ i ].

  - The derivation process for a view synthesis prediction flag as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the merge candidate indicator N as inputs, and the output is the mergeCandIsVspFlag.

  - When availableFlagIvDCShift is equal to 0 and availableFlagN is equal to 1, the candidate N is not equal to IvMC or IvDC, and mergeCandIsVspFlag is not equal to 0, predFlagL0N is equal to 1 and ViewIdx( RefPicList0[ refIdxL0N ] ) is not equal to ViewIdx, the following applies:

    - availableFlagIvDCShift is set equal to 1

    - predFlagLXIvDCShift is set equal to predFlagLXN, ( with X being replaced by 0 and 1 )

    - refIdxLXIvDCShift is set equal to refIdxLXN, ( with X being replaced by 0 and 1 )

    - mvL0IvDCShift[ 0 ] is set equal to mvL0N[ 0 ] + 4

    - mvL0IvDCShift[ 1 ] is set equal to ( view_synthesis_pred_flag[ nuh_layer_id ] ? 0 : mvL0N[ 1 ] )

    - mvL1IvDCShift = mvL1N

When availableFlagIvDCShift is equal to 0 and availableFlagIvDC is equal to 1, availableFlagIvDCShift is set to 1 and the following applies for X being 0 to 1, inclusive:

- predFlagLXIvDCShift is set equal to predFlagLXIvDC,

- refIdxLXIvDCShift is set equal to refIdxLXIvDC,

- mvLXIvDCShift[ 0 ] is set equal to mvL0IvDC[ 0 ] + 4

- mvLXIvDCShift[ 1 ] is set equal to mvL0IvDC[ 1 ]

- I.8.5.3.2.17	Derivation process for a view synthesis prediction flag

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

- a merge candidate indicator N, specifying the merge candidate.

Outputs of this process are:

- a variable mergeCandIsVspFlag specifying, whether the merge candidate is a view synthesis prediction  merge candidate.

The variable mergeCandIsVspFlag is derived as specified in the following:

- If N is equal to VSP, mergeCandIsVspFlag is set equal to 1,

- Otherwise, if N is equal to A1, B1, B0, A0, or B2, the following applies:

  - The luma position ( xN, yN ) is specified in Table Error: Reference source not found-1 depending on N.

  - If one of the following conditions is true, the variable mergeCandIsVspFlag is set equal to VspModeFlag[ xN ][ yN ].

    - N is equal to A1 or A0

    - N is equal to B0, B1, or B2 and ( yN  >>  Log2CtbSizeY ) is equal to ( yCb >> Log2CtbSizeY )

  - Otherwise, ,mergeCandIsVspFlag is set equal to 0.

- Otherwise, mergeCandIsVspFlag is set equal to 0.

Table Error: Reference source not found-1 – Specification of xN and yN depending on N

| N | A1 | B1 | B0 | A0 | B2 |
| --- | --- | --- | --- | --- | --- |
| xN | xPb − 1 | xPb + nPbW − 1 | xPb + nPbW | xPb − 1 | xPb − 1 |
| yN | yPb + nPbH − 1 | yPb − 1 | yPb − 1 | yPb + nPbH | yPb − 1 |

- I.8.5.3.2.18	Derivation process for the base merge candidate list

The specifications in subclause 8.5.3.2.1 apply, with the following modifications:

- Steps 9 and 10 are removed.

- "When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates" is replaced by "When slice_type is equal to B and numMergeCand is less than 5, the derivation process for combined bi-predictive merging candidates"

- "temporal luma motion vector prediction in subclause 8.5.3.2.7 is invoked" is replaced by "temporal luma motion vector prediction in subclause Error: Reference source not found is invoked"

- The outputs of the process are replaced by:

  - a modified luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

  - two variables nPbW and nPbH specifying the modified width and the height of the luma prediction block,

  - a modified variable partIdx specifying the modified index of the current prediction unit within the current coding unit.

  - an original luma location ( xOrigP, yOrigP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

  - two variables nOrigPbW and nOrigPbH specifying the original width and the height of the luma prediction block,

  - the merge candidate list, mergeCandList, with entries being identified by bN and N being equal to 0 through MaxNumMergeCand-1, inclusive,

  - the luma motion vectors mvL0N and mvL1N, with N being replaced by all entries of mergeCandList

  - the reference indices refIdxL0N and refIdxL1N, with N being replaced by all entries of mergeCandList

the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of mergeCandList